Part 1  General Description

  - Project Overview:

    - Prepare and paint truck bay floor, pit rails and brake test metal guards with two (2) coats of paint.

  - Site Location:

    - The Kenosha Safety & Weight Enforcement Facility is located on I-94 westbound approximately 0.5 mile north of the Illinois border in Kenosha County, Wisconsin.

  - Quality Assurance

    - Contractor shall follow manufacturer’s directions and specifications for painting preparation, application and applicable industry acceptable standards and methods.

  - Warranty

    - Provide manufacturer’s warranty for specified individual products.

    - Contractor will provide any necessary paint repairs due to product or installation error for a six month period after painting is complete at no cost to owner.

Part 2  Products

  - Acceptable Manufacturer(s)

    - Aquapoxy Paint or approved equal quality manufacturer.

  - Products

      - Aquapoxy II, TS-920 paint in color 416 light grey or like product and color and quality.

      - Aquapoxy II, TS-920 paint in color Safety SC Yellow or like product and color and quality.

      - Aquapoxy TS-630 Pro-Tex slip resistant additive or like product and color and quality.

Part 3 Installation

  - Method of Installation

    - Prepare inspection bay floors, pit rails and brake testing guard plates as directed by manufacturer instructions to receive two coats of paint. Patch as necessary any cracks with epoxy or cement as is applicable.

    - Mix paint and slip resistant additive per manufacturer directions.

    - Paint current grey floor areas grey and current yellow lines and brake test metal guards yellow (two coats).

    - Instruct building owner project advisor (RFW) on dry time and allowable use timeframes.

    - Leave one gallon of each color paint with the project advisor.

  - Cleaning of Work Areas

    - Properly dispose of all waste materials.

    - Leave all work areas broom clean.

    - Provide operating manuals and all warranty information to project advisor.