WS 1073 EROSION CONTROL IMPLEMENTATION PLAN (ECIP) WORKSHEET      (Rev. 6/12)

Following TRANS 401 of Wisconsin Administrative Code, an ECIP for a project shall be provided to the appropriate WisDOT region office of construction and to the appropriate WDNR liaison as identified in the plan at least 14 days prior to the pre-construction conference, or at a time otherwise agreed upon by WisDOT, WDNR and the prime contractor.  The ECIP shall be prepared in a detailed, written and pictorial format that identifies the schedule, timing, and methodology for the contractor's implementation of the project’s erosion control plan. See the ECIP Worksheet Instructions in the Appendix for additional information regarding ECIP contents.

Project ID: 				  Highway: 			  County:

Name of Road/Project:

Type of Work:

Prime Contractor:

Address:

Contact Person: 					  Phone:

DOT Project Manager: 				  Project Leader:

# A. The following shall complement the WisDOT project erosion control plan.

1.	Principal contact of the contractor responsible for installation, maintenance, and removal of erosion control and storm water management measures at the project sites.

|  | Name: |  | Phone: |  |
| --- | --- | --- | --- | --- |
|  | Firm: |  |  |  |
|  | Address: |  |  |  |

2.	A description of the intended timetable and sequence of major land disturbing activities at the project sites.

3.	A description of erosion control and storm water management measures to be utilized and a schedule for implementing them, including staging construction and maintenance to limit disturbed areas subject to erosion; timing and use of erosion control mobilizations; method for winter shut-down; and the removal of temporary measures.

- For each structure on the project identify:

  - How any Special Provisions relating to bridge removal will be met.

  - The structure removal capture system to be used.

  - Dewatering methods and locations.

  - Protection around abutments and pier(s).

  - Location and protection of stockpile(s).

  - How water will be handled (i.e. diversion channel, pumping), include detailed plan.

  - Location of staging areas.

  - Any changes needed to the 404 permit.

5.	A description of any additions, amendments, deletions or modifications to the projects erosion control plan or any of the contract documents which pertain to erosion control and stormwater management for the project sites.

B.  Erosion Control Implementation Plan Requirements - Selected Sites

Project ID: 				  Highway: 			  County:

Name of Road/Project:

Type of Work:

Prime Contractor:

Address:

Contact Person: 					  Phone:

DOT Project Manager: 				  Project Leader:

The ECIP shall also include, at a minimum, a narrative and pictorial description for each of the selected sites, if any, and attendant erosion control and storm water management measures for the selected sites.  If the combined area of the project site and all selected sites disturbs 1 or more acres as determined by WisDOT the following information is required for each selected site.

If a selected site is used prior to WisDOT approval, it is not covered under the Cooperative Agreement between DOT and DNR and all applicable permits need to be obtained before the selected site can be used.

| 1. | Selected Site Name: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Address: |  |  |  |  |  |  |  |  |  |
|  | City/Village/Town: |  |  |  |  | County: |  |  |  |  |
|  | Township Range Section 1/4 Sect. 1/4-1/4 Sect. |  |  |  |  |  |  |  |  |  |

Include a location map, i.e. a plat map.

| 2. | Principal contact of the contractor or other person responsible for installation, maintenance, and removal of erosion control and storm water management measures at the selected site. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Name: |  |  |  |  |  | Phone: |  |  |
|  | Firm: |  |  |  |  |  |  |  |  |
|  | Address: |  |  |  |  |  |  |  |  |

| 3. | Is this a commercial pit? Yes / No |  |  | If no, continue to #4 |  |
| --- | --- | --- | --- | --- | --- |
|  | If yes, Name of the pit: |  |  |  |  |
|  | Contact for the pit: |  |  | Phone: |  |
|  | Include cover sheet of NR 216, NR 135 or COMM permit displaying the permit number, CUP number and expiration date. |  |  |  |  |
|  | Will the waste or borrow be in the permitted area? Yes / No If yes then end of Part B for this selected site. If no then complete remainder of Part B. |  |  |  |  |

- Have applicable permits been obtained?  Yes / No

5.	Is the selected site on tribal land?	Yes / No

- Has the Archaeological Review (Form DT1919) been sent to BEES?           Yes / No        What was the Bureau recommendation?	              Have not received response yet	                     ______ Survey Recommended                  High Potential                 OK to Proceed

- Construction activity dates at the selected site:   Start: _________  Complete:__________

- A narrative description of the selected site as it exists before construction, the nature of the activities to be performed at the site including approximate quantity of waste/borrow material, and land use anticipated after restoration to the site.

- A description of the intended sequence of major land disturbing activities at the selected site.

- Estimated total area of selected site: ___________Total disturbed area:_____________

- Immediate receiving waters:  									 (Attach FEMA Floodplain maps)

- Runoff coefficients at the selected site. (Attach the Runoff Coefficient Table)       	         Supply the following estimates: Site slope before construction: 		 After:

- Site map(s) including: (See Trans 401.08(2)(b)(11) for details).

  - Boundaries of the site and areas of soil disturbance.

  - Existing topography and drainage patterns, roads and surface waters.

  - Drainage patterns and approximate slopes anticipated after major grading activities.

  - Location of major structural and non-structural erosion control and stormwater management practices.

  - Location of areas where stabilization will be employed, including but not limited to vegetation, following construction or maintenance activities.

  - Area and extent of wetland acreage on the site, whether disturbed or not.

  - Locations where storm water is discharged to a surface water or wetland.

  - Locaton of any internal haul roads.

(Recommend using USGS maps, Orthophotos, SCS Soils maps, or equivalent.)

- A description of appropriate erosion control and storm water management measures that will be employed at the selected site to prevent sediments and pollutants from reaching waters of the state, including wetlands.  The plan shall clearly describe the appropriate best management practice for each major activity identified and the timing during the construction process that the measures will be implemented.  The description of best management practices shall include:

- Description of permanent or temporary erosion control and storm water management measures. Plans shall ensure the preservation of existing vegetation where practical.

- Description of structural practices to divert runoff away from exposed soils, to store flows or to otherwise limit runoff and the discharge of pollutants from the site.

- Management of overland flow at the site.

- Trapping of sediment in channelized flow.

- Staging construction to limit bare areas subject to erosion.

- Protection of downslope drainage inlets where they occur.

- Minimization of tracking at the site.

- Clean up of off-site sediment deposits.

- Proper disposal of building and waste material at the site.

- Stabilization of drainage ways.

- Installation of permanent stabilization practices as soon as possible after final grading.

- Minimization of dust to the maximum extent practical.

- Stabilization of the disturbed portions of the site.

16.	A description of the procedures to maintain vegetation, best management practices and other protective measures, in good and effective operating condition.  If the selected site will remain open for more than 2 weeks without construction activities (i.e. over-winter), how will the site be stabilized and how often will it be inspected?

# If permanent infiltration devices are employed, complete:

17.	Existing data describing the surface soil, subsoils, and depth to groundwater at the selected site. (Refer to Soil Conservation Service’s County Soil Survey Book or equivalent where available.)

# C. Amendments

# The contractor shall follow the procedure outlined in Trans 401.08(3) for all amendments.

The ECIP shall be amended when there is a change in design, construction, operation or maintenance at a project or selected site that has the reasonable potential for a discharge to waters of the state and that has not been addressed in the ECIP; or when the best management practices required by the plan fail to reduce adverse impacts to waters of the state caused by a discharge.

Amendments are subject to the written approval of the Department of Transportation after consultation with the DNR.

Please label all attachments with the corresponding Section number (i.e., Attachment B10)

# Appendix - Erosion Control Implementation Plan (ECIP) Worksheet Instructions

The prime contractor implementing the erosion control plan for the project shall develop the Erosion Control Implementation Plan (ECIP).  The prime contractor shall also use the ECIP to develop and implement an erosion control plan for selected sites, if any.  The prime contractor is referred to ch. Trans 401, Wis. Administrative Code for a detailed account of the items required in the ECIP.  The ECIP may not be implemented prior to its written approval by the Department of Transportation in consultation with the Department of Natural Resources.

The ECIP for a project shall be provided to the appropriate WisDOT region office of construction and to the appropriate WDNR liaison as identified in the plan; delivered either personally or by registered or certified mail, return receipt requested, by the prime contractor at least 14 days prior to the pre-construction conference, or at a time otherwise agreed upon by WisDOT, WDNR and the prime contractor.  The ECIP shall be prepared in a detailed, written and pictorial format that identifies the schedule, timing and methodology for the contractor's implementation of:

- The project's erosion control plan.

- The erosion control plan for selected sites. "Selected sites," means any borrow site or material disposal (waste) site used exclusively for projects administered by WisDOT.

The following is a description of the requirements that are needed for an Erosion Control Implementation Plan (ECIP) under Trans 401.  The associated numbers coincide with the ECIP worksheet.  The ECIP is not intended to restate contract requirements relating to environmental issues.  The ECIP shall contain information as to how the contract erosion control requirements will be implemented by the prime contractor.

Section A is required for any DOT administered project that contains an erosion control bid item.  However, the detail of an ECIP depends on the project type.  DOT administered projects that do not contain bid items for erosion control do not require the submittal of an ECIP, unless specified otherwise by the DOT.  For example, long line stripping, etc.…

A1.	Identify the contractor’s representative in charge of installing, maintaining and removing the erosion control devices, i.e. erosion control subcontractor.  Include phone numbers that will directly contact this representative at any time of the day or night, not the office number.

A2. 	The contractor’s progress schedule with all land disturbing and erosion control activities, including erosion control mobilizations.

A3. 	The contractor must explain how they will implement the erosion control plan into their construction stages and operations.  Prepare a narrative that describes how the erosion control practices fit into the project and show when erosion control mobilizations are to occur.  Use drawings to illustrate staging as well as proposed changes.  Each erosion control item must be shown and labeled.  Show when specific erosion control practices will be placed or removed for each operation or stage of the project.  Indicate when temporary measures will be removed.  Describe any additional measures not included in the erosion control plan due to late season work.  If any portion of the project will remain open for an extended period of time (i.e. over-winter) indicate how the area will be stabilized and how often and by whom it will be inspected.

A4. 	For each structure on the project identify the following

- Refer to any and all Special Provisions that relate to each structure and how they will be met.

- What method will be used to prevent material from falling?

- Identify what method will be used for dewatering, the maximum pumping rate for this method, where will it be located and if vegetation is required to at the outlet.

- Method to create a barrier between the abutments or piers and any open water.

- Where will any stockpiles be located and how will they be protected from entering a waterway or wetland.

- Method to divert or handle live water while working on the structure.  Include a detailed plan.

- Where will the staging area be located and how will it be separated from any live water or wetlands.

- If there is an existing 404 permit is it adequate or will contractor methods require an amendment?  If there is not a 404 permit do the contractor methods require one?

A5.	Any changes to the erosion control plan are identified in this section.

Section B of the worksheet is for selected sites (borrow and material disposal sites) only.  Complete Section B for each selected site.  Selected sites that do not involve processing of materials and are used exclusively in DOT projects shall be addressed in the ECIP.  The ECIP for selected sites shall include, at a minimum, a narrative and pictorial description of each of the selected sites (plan and cross section views as appropriate), the erosion control measures used at each site, and a schedule for implementing them. Selected sites that involve material processing must be in compliance with NR 216 and other laws for use in a DOT project, but do not require the submittal of a full ECIP. For these sites, the ECIP should document the compliance of the processing sites with NR 216 and other applicable laws.

B1. 	Name and location of selected site.  Attach a plat map or other location map.

B2. 	Identify the contractor’s representative in charge of installing, maintaining, and removing the erosion control devices.  Include a phone number that will directly contact this representative, not the office number.

B3. 	If this pit is permitted by another state agency or governing body the complete ECIP is not required, but proof that it is permitted is required.  If the work done is on the pit property, but not in the permitted area, a complete ECIP Section B submittal is required.

B4.	Local zoning ordinances must still be met.

B5.	State Statute 84.06(12) Borrow Sites exempts contractors from getting local zoning permits for borrow sites and makes the landowner responsible for noise abatement, landscaping and restoration, there is a sunset date of July 1, 2011.  Material disposal (waste) sites must still follow all local zoning rules and regulation and get permits where applicable.

B6.	Lands owned or held by a recognized Tribe will need to go through the tribe for the approval process.

B7.	An Archaeological Review must be sent to BEES for review for each selected site.

B8. 	Start and completion dates of construction activities on selected site.

B9.	Describe the selected site before, during and after construction.  Identify the existing site conditions and use (i.e. farmed field, lawn).  Identify if it is a borrow or material disposal (waste) site including how material will be removed or deposited and approximate quantity of material.  Discuss the post construction use of this site.

B10. 	Timetable for selected sites similar to the progress schedule for the project site.

B11. 	Area of each individual selected site.  Area of disturbance.

B12. 	List immediate receiving waters that are directly affected by runoff from the selected sites.  If there is an immediate receiving water, special care must be taken to protect this area.  A check to see if this area is a wetland is recommended.

B13. 	Attach the DOT’s Runoff Coefficient Table, and note the before and after construction slopes.

B14. 	Map showing where site is located.  Please see Trans 401.08(2)(b)(11) for details.  Identify the topography, drainage patterns, anticipated slopes and areas of disturbance, location and timing of structural controls, non-structural controls, areas where stabilization will be employed, and areas that will be vegetated on the selected site.

- Use plan or location map to outline both the property and the area of disturbance.

- Use USGS Topographical map or similar to show existing conditions. http://nationalmap.gov/gio/viewonline.html

- Use same map to identify areas of soil disturbance.

- Use same map to identify post construction topography at the site.

- Identify where the BMPs will be used.

- Where will the site be stabilized, i.e. seed or sod.

- Identify any impacted, affected or nearby wetlands.  If there are any wetlands that may be affected, then a qualified Wetland Delineator must establish the boundaries. http://www.dnr.state.wi.us/wetlands/mapping.html

- Locate where stormwater will be discharged from the site during and after construction and any protection needed at the outfall.

- Locate any existing or created haul roads within the site boundaries that will be used.

B15.	Where and how best management practices will be used.

- Which BMPs are going to be used and where.

- Topsoil berms to divert water away from exposed surfaces.

- How will the overland flows be contained, diverted and prevent sediment from leaving the site.

- Ditch checks, sumps, sediment traps, etc.

- Identify if the way the site is staged will prevent excessive runoff.

- Protect all nearby inlets that may likely be affected by the site.

- Type of tracking pad if needed.

- Method and frequency of cleaning any sediment that left the site. (i.e. sweeping at end of each day if tracking pad is not 100% successful at keeping sediment on site.)

- Disposal methods for all non-sediment on site.

- Permanent stabilization of any ditches, channels or depressions where water will likely flow post-construction.

- Schedule permanent stabilization as soon as practical to prevent possible future erosion.

- Identify any dust control practices that will be used.

- Vegetation or signed agreement with property owner if it will be farmed or for other use immediately after the site is closed.

B16. 	Describe the maintenance procedures that will be used on the selected site.

B17. 	Soil information is required when permanent infiltration devices will be used on the selected site.  The information will usually come from Soil Conservation Service County Soil Survey (SCS) book. http://websoilsurvey.nrcs.usda.gov/app/

Section C.  Please refer to Trans 401.08(3) for all amendments.  DNR should be notified by DOT of any planned amendments to the plan.

ECIP’s may be done in stages, if approved by the DOT (for example, all selected sites are not known at the time of ECIP submittal).  The ECIP for the initial project should indicate when the other stages would be submitted to the DOT.