# High Strength Steel Anchor Rod Installation Tensioning Record

Wisconsin Department of Transportation

# DT2321     4/2008

Purpose

Submit this form to assure the proper installation of:

- Each sign bridge or sign support installed under standard spec 641.

- Each type 9, 10, 12, and 13 structure installed under standard spec 657.

- Each high mast lighting structure installed under standard spec 660.

Tensioning of anchor bolts is vital to the safety of the traveling public and may severely reduce the life of the structure if not performed properly.

Procedure

Install High Strength Steel Anchor Rods (Grade 55) according to standard spec 641.3.1.2 which says:

(1)	Lubricate anchor bolt threads and nuts with bees wax or other high-wax lubricant. Set leveling nuts to the required elevation before installing the structure. Adjust top nuts and leveling nuts to align and plumb the structure. Ensure that all nuts are snug tight with no gaps. Tighten each top nut 1/3 turn past snug for bolts 1 1/2 inch (38 mm) or smaller in diameter and 1/6 turn for larger diameter bolts conforming to the tightening sequence specified on department form DT2321. If required, install jamb nuts wrench tight.

(2)	Complete department form DT2321 for each structure. Indicate the parties responsible for the installation and submit the form to the engineer for inclusion in the permanent project record.

# Tightening Sequence

6 & 8 Rod Installation Patterns

| Checklist |  |  | Comments |
| --- | --- | --- | --- |
| Structure Number: | Yes | No |  |
| Anchor rods, nuts and washers received according to plans? |  |  |  |
| Anchor rods, nuts and washers received in good condition? |  |  |  |
| The nuts could be freely moved “by hand” through the whole range of use prior to installation? |  |  |  |
| The threads were properly lubricated prior to installation? |  |  |  |
| The leveling nuts and top nuts were snug prior to installation rotation? |  |  |  |
| The nuts had full thread engagement? |  |  |  |
| The applied turn after snug met the minimum standard as listed above? |  |  |  |
| The second top nut “jam nut” was installed wrench tight? Note: Jam nuts are only required for high mast light poles, and cantilevered sign structures or if plan requires them. |  |  |  |
| List any items that were performed outside this procedure and the name of the engineer who approved the changes: |  |  |  |
| Additional comments: |  |  |  |

Responsible party, contractor or representative.      Name:     					Date: