Roadway Lighting Safety Guidelines

Final Report

By

Lang Yu

Research Assistant

Zhixia Li, Ph.D.

Assistant Researcher

Andrea Bill

Traffic Safety Engineering Program Manager

David A. Noyce, Ph.D., P.E.

Professor and Director

Wisconsin Traffic Operations and Safety (TOPS) Laboratory

University of Wisconsin-Madison

Department of Civil and Environmental Engineering

July 2015

DISCLAIMER

This research was funded by the Wisconsin Department of Transportation. The contents of this report reflect the views of the authors who are responsible for the facts and accuracy of the data presented herein. The contents do not necessarily reflect the official views or policies of the Wisconsin Department of Transportation, Federal Highway Administration or the University of Wisconsin.

This document is disseminated under the sponsorship of the Department of Transportation in the interest of information exchange. The agencies listed above assume no liability for its contents or use thereof. This report does not constitute a standard, specification, or regulation, and its contents are not intended for construction, bidding, or permit purposes.

The name of any products or manufacturers listed herein does not imply an endorsement of those products or manufacturers. Trade and manufacturers’ names appear in this report only because they are considered essential to the object of the document.

Technical Report Documentation Page

| 1. Report No. | 2. Government Accession No. |  |  | 3. Recipient's Catalog No. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4. Title and Subtitle Roadway Lighting Safety Guidelines |  |  |  | 5. Report Date July 2015 |  |  |
|  |  |  |  | 6. Performing Organization Code |  |  |
| 7. Author(s) Lang Yu, Zhixia (Richard) Li, Andrea Bill, and David A. Noyce |  |  |  | 8. Performing Organization Report No. |  |  |
| 9. Performing Organization Name and Address Traffic Operations and Safety (TOPS) Laboratory 1241 Engineering Hall 1415 Engineering Dr. Madison, WI, 53706 |  |  |  | 10. Work Unit No. (TRAIS) |  |  |
|  |  |  |  | 11. Contract or Grant No. |  |  |
| 12. Sponsoring Agency Name and Address Wisconsin Department of Transportation 4802 Sheboygan Ave, Madison, WI, 53707 |  |  |  | 13. Type of Report and Period Covered |  |  |
|  |  |  |  | 14. Sponsoring Agency Code |  |  |
| 15. Supplementary Notes |  |  |  |  |  |  |
| 16. Abstract Lighting is a proven countermeasure to address roadways safety during nighttime. However, no guidelines on lighting were developed based on a quantitative safety analysis of lighting benefits. Therefore, this research conducted a comprehensive analysis and modeling on the impact of lighting on nighttime crash frequency on Wisconsin STN freeway and interchanges. Various roadway geometry and operational data, including lighting and 5-year crash data were collected. The freeway network was divided into three categories (freeway segments, interchange segments, interchange ramps) and smaller segments. A crash analysis was conducted to qualitatively understand the safety benefits of lighting. Crash rates and injury severity distributions were compared for nighttime lighted and unlighted crashes. Crash characteristics were also compared using statistical methods. Besides, a series of safety performance functions were developed to predict nighttime crashes under lighted and unlighted conditions. Based on model results, responsive and proactive tools were developed to facilitate the decision making of lighting installation. A benefit cost analysis was also completed to determine the highest b/c ratio scenarios. Guidelines on freeway and interchange lighting were proposed according to the developed models and analysis results. |  |  |  |  |  |  |
| 17. Key Word Roadway lighting, Nighttime crashes, SPF |  |  | 18. Distribution Statement Distribution Unlimited |  |  |  |
| 19. Security Classif. (of this report) Unclassified |  | 20. Security Classif. (of this page) Unclassified |  |  | 21. No. of Pages 166 | 22. Price |

Form DOT F 1700.7 (8-72)	Reproduction of completed page authorized

# Executive Summary

Historical crash data showed that the absence of lighting during nighttime resulted in a disproportionally large number of traffic crashes and fatalities. Many sources have identified lighting as a proven safety countermeasure to address roadway crashes during nighttime. Both FHWA and AASHTO provide lighting guidelines. Several states also have their own lighting manual. However, most were modified based on AASHTO guidelines. The biggest issue is that most states, including Wisconsin, have not looked holistically at the safety of a roadway with regards to lighting. Also, little previous researches have completed quantitative assessment of the benefits of lighting from a traffic safety standpoint. Therefore, most current lighting guidelines are not based on historical crash data. In this context, this research aims at developing lighting guidelines for Wisconsin considering safety benefits of lighting on freeways through comprehensive crash analysis and modeling.

The scope of this research includes all freeways and interchanges within the STN network. The entire network was divided into three roadway types, namely freeway segments, interchange segments and interchange ramps due to their distinctive characteristics. Lighting pole location data was collected based on STN lighting data and manual verification through Google Street View. Five years of crash data were collected and classified by daytime and nighttime crashes. Nighttime crashes were further divided into nighttime lighted and nighttime unlighted crashes. In addition, AADT and various roadway geometric and traffic related variables were also collected.

A comprehensive safety analysis was carried out to qualitatively understand the safety benefits of lighting. Crash rates were compared between lighted and unlighted segments. In addition, crash injury severity distribution was compared between lighted and unlighted conditions. Furthermore, various crash characteristics under lighted and unlighted conditions were compared. These characteristics include crash types, month of year, hour of day, special geometric, weather, and roadway conditions, as well as driver contributing factors. Key findings from the qualitative crash analysis and comparisons include:

- For freeway segments, unlighted segments have significantly higher crash rates than lighted segments.

- For ramps, the overall comparison between lighted and unlighted ramps shows a statistically significant difference. However, the difference between two-lane and three-lane ramps is not statistically significant.

- Nighttime unlighted crashes had significantly more fatal and injury crashes than nighttime lighted crashes.

- Daytime low-visibility crashes, which usually occur under foggy, rainy or snowy conditions, share many common characteristics with nighttime unlighted crashes.

To assist quantitative understanding and prediction of nighttime crashes under both lighted and unlighted conditions, safety performance functions (SPFs) were developed. A total of 16 SPFs were developed for different segment types, number of lanes and lighting conditions. A negative binomial generalized linear model was used for developing the SPFs. An SPF models crash frequency as a function of segment length, AADT, and a series of crash modification factors (CMFs). CMFs include roadway geometry variables such as shoulder width, median width, presence of median barrier, presence of horizontal curves, area type, and presence of ramp meter, etc. Comparing base models for lighted and unlighted SPFs showed that:

- Lighting reduces nighttime crash frequencies for all segment types, though with different magnitude.

- Interchange segments have higher nighttime crash frequencies than non-interchange freeway segments.

- Six-lane freeway segments and interchange segments have lower nighttime crash frequencies than four-lane freeway segments and interchange segments.

- The reduction of nighttime crash frequency by lighting generally increases as AADT increases.

- Entry ramps have higher nighttime crash frequencies than exit ramps under either unlighted or lighted conditions.

Using the developed SPFs, three tools have been developed to assist engineers to quantitatively understand the lighting benefits when making decisions on lighting installations and other geometric safety treatments. The tools can be categorized as two types, namely responsive tools and proactive tools. Responsive tools were developed to identify locations where lighting is needed and the benefit that can be obtained, including a set of unlighted segments crash risk and lighting benefit maps. Proactive tools were developed for use in the planning phase to assist decision making of installing lighting and other safety treatments, including a crash reduction look-up table and an Excel spreadsheet tool. The Excel spreadsheet tool is able to compare the predicted nighttime crashes with and without lighting installed for all three segment types.

A benefit cost analysis was also conducted to determine the high benefit-cost scenarios. Benefit/cost ratios were computed based on 10-year lighting benefits with lighting installation and 10-year operation cost. Key findings are summarized as follows:

- All of the B/C ratios for freeway and interchange segments are greater than 1.0.

- Most low and some medium AADT ramps scenarios have a B/C ratio of less than 1.0.

Lighting guidelines were developed based on the safety benefits of lighting under different scenarios for different segment types. Scenarios of the top 20th percentile of lighting benefits are recommended for lighting installation. General conditions under which lighting installation is recommended are:

- Freeway segments: In most cases, segments with more than 90,000 AADT would be recommended for lighting. In some cases, segments with lower AADT combined with certain roadway geometry were also recommended for lighting.

- Interchange segments: In most cases, segments with more than 80,000 AADT would be recommended for lighting. In some cases, segments with lower AADT combined with certain roadway geometry were also recommended for lighting.

- Interchange Ramps: In most cases, segments with more than 15,000 AADT would generally be recommended for lighting. In some cases, certain ramp type, ramp metering and geometry factor combinations will lower the AADT threshold for lighting installation.

Considering that there are still a number of locations that meet the AADT criteria as described in Sections 9.1 through 9.3, while funds available for lighting installation is limited,  a six-step method for prioritizing locations for lighting installation is provided.

# Table of Contents

Executive Summary	iii

Table of Contents	vi

List of Tables	xi

List of Figures	xiii

1.	Introduction	1

2.	Project Goals and Objectives	2

3.	Literature Review	3

3.1	Lighting Design and Warrant Guidelines	3

3.1.1	National Level Lighting Guidelines	3

3.1.2	State Level Lighting Guidelines	3

3.2	Separation of Daytime/Nighttime	6

3.3	Lighting Safety Analysis	7

3.3.1	Before-and-After & Comparison Analysis	7

3.3.2	Performance Measure	7

3.3.3	Lighting Safety Characteristics Analysis	9

3.3.4	Crash Modification Factors	10

3.4	New Lighting Technologies	11

3.5	Roadway Lighting Economics	12

3.6	SPFs Development Method	12

3.7	Freeway/Interchange Safety Analysis	13

3.7.1	Interchange Types and Components	13

3.7.2	Freeway/Interchange Lighting	16

3.7.3	Freeway/Interchange Safety Performance Functions	16

3.8	Literature Summary	17

4.	Data Collection	18

4.1	Lighting Data	21

4.2	Crash Data	23

4.3	Traffic Volume Data	27

4.4	Roadway Geometry Data	27

4.4.1	Number of Lanes	28

4.4.2	Lane Width	28

4.4.3	Left Shoulder Width and Right Shoulder Width	29

4.4.4	Median Width	29

4.4.5	Median Type	29

4.4.6	Median Barrier Type	30

4.4.7	Weaving	30

4.4.8	Horizontal Curve	30

4.4.9	Shoulder Rumble Strip	31

4.4.10	Interchange Type	31

4.4.11	Ramp Type	31

4.5	Other Data	32

4.5.1	Area Type	32

4.5.2	Presence of Ramp Meter	32

4.5.3	IRI	33

4.6	Data Summary	33

5	Comprehensive Safety Analysis	35

5.1	Analysis of Nighttime Crash Rates	35

5.1.1	Nighttime Crash Rate Computation	36

5.1.2	Removing outliers	36

5.1.3	Summary of Nighttime Crash Rate	37

5.1.4	Nighttime Crash Rate Comparison	38

5.2	Injury Severity Distribution Comparison	41

5.3	Crash Characteristics Comparisons	44

5.3.1	Month Distribution	45

5.3.2	Hour Distribution	47

5.3.3	Crash Type	48

5.3.4	Vehicle Damage Type	49

5.3.5	Median Barrier	50

5.3.6	Horizontal Curve	51

5.3.7	Vertical Curve	52

5.3.8	Roadway Conditions	53

5.3.9	Speed Limit	54

5.3.10	Weather Conditions	55

5.3.11	Driver Age	57

5.3.12	Driver Gender	57

5.3.13	Driver Contributing Factors	58

5.4	Summary	59

6	Development of Nighttime Safety Performance Functions	61

6.1	List of SPFs	61

6.2	SPF Development and Identified Significant Factors	63

6.3	Developed SPFs (Base SPF with CMFs)	65

6.3.1	SPF for Lighted Four-Lane Freeway Segments	66

6.3.2	SPF for Lighted Six-Lane Freeway Segments	66

6.3.3	SPF for Unlighted Four-Lane Freeway Segments	66

6.3.4	SPF for Unlighted Six-Lane Freeway Segments	67

6.3.5	SPF for Lighted Four-Lane Interchange Segments	67

6.3.6	SPF for Lighted Six-Lane Interchange Segments	68

6.3.7	SPF for Unlighted Four-Lane Interchange Segments	68

6.3.8	SPF for Unlighted Six-Lane Interchange Segments	68

6.3.9	SPF for Lighted One-Lane Entry Ramps	69

6.3.10	SPF for Lighted One-Lane Exit Ramps	69

6.3.11	SPF for Unlighted One-Lane Entry Ramps	69

6.3.12	SPF for Unlighted One-Lane Exit Ramps	70

6.3.13	SPF for Lighted Two-Lane Entry Ramps	70

6.3.14	SPF for Lighted Two-Lane Exit Ramps	71

6.3.15	SPF for Unlighted Two-Lane Entry Ramps	71

6.3.16	SPF for Unlighted Two-Lane Exit Ramps	71

6.4	Comparisons between Base SPF Models	72

6.4.1	Freeway Segments	72

6.4.2	Interchange Segments	72

6.4.3	One-Lane Interchange Ramps	73

6.4.4	Two-Lane Interchange Ramps	73

6.5	Sensitivity Analysis for Continuous CMFs	74

6.5.1	CMFs for Lighted Four-Lane Freeway Segments	74

6.5.2	CMFs for Lighted Six-Lane Freeway Segments	75

6.5.3	CMFs for Unlighted Four-Lane Freeway Segments	75

6.5.4	CMFs for Unlighted Six-Lane Freeway Segments	77

6.5.5	CMFs for Lighted Four-Lane Interchange Segments	78

6.5.6	CMFs for Lighted Six-Lane Interchange Segments	78

6.5.7	CMFs for Unlighted Four-Lane Interchange Segments	79

6.5.8	CMFs for Unlighted Six-Lane Interchange Segments	80

6.5.9	CMFs for Lighted One-Lane Entry Ramps	81

6.5.10	CMFs for Lighted One-Lane Exit Ramps	81

6.5.11	CMFs for Unlighted One-Lane Entry Ramps	82

6.5.12	CMFs for Unlighted One-Lane Exit Ramps	83

6.5.13	CMFs for Lighted Two-Lane Entry Ramps	83

6.5.14	CMFs for Lighted Two-Lane Exit Ramps	84

6.5.15	CMFs for Unlighted Two-Lane Entry Ramps	85

6.5.16	CMFs for Unlighted Two-Lane Exit Ramps	85

6.6	Injury Severity Distribution Model	86

7	Lighting Safety Analysis Tools	88

7.1	Proactive Tool: Nighttime Crash and Crash Reduction Look-Up Table	88

7.2	Proactive Tool: Excel Spreadsheet Tool	94

7.3	Responsive Tool: Unlighted Crash Risk and Lighting Benefit Maps	97

8	Benefit Cost Analysis	105

8.1	Crash Costs and Lighting Benefits	105

8.2	Lighting Costs	106

8.3	Benefit/Cost Ratios	108

9	Lighting Safety Guidelines	109

9.1	Freeway Segments	109

9.1.1	Four-Lane Freeway Segments	109

9.1.2	Six-Lane Freeway Segments	111

9.2	Interchange Segments	113

9.2.1	Four-Lane Interchange Segments	113

9.2.2	Six-Lane Interchange Segments	114

9.3	Interchange Ramps	116

9.3.1	One-Lane Entry Ramps	116

9.3.2	One-Lane Exit Ramps	118

9.3.3	Two-Lane Entry Ramps	120

9.3.4	Two-Lane Exit Ramps	121

9.4	Steps to Prioritize Locations for Lighting Installation	123

10	Uniqueness, Benefit and Limitations	124

10.1	Uniqueness	124

10.2	Benefit	124

10.3	Limitations	124

11	Conclusions	126

References	128

Appendix A Glossary	131

Appendix B Data Summary by County	134

Appendix C Top 100 Unlighted Segments Concerning Crash Risk	138

Appendix D Top 100 Unlighted Segments Concerning Lighting Benefit	152

# List of Tables

Table 1 Comparison of Different Lighting Types (32)	11

Table 2 Segment Data Descriptive Statistics	21

Table 3 List of Added Lighting Pole Locations	22

Table 4 Number of Lighted and Unlighted Segments by Type	22

Table 5 Table of Sunrise/Sunset Time in Different Regions of Wisconsin (Central Standard Time)	24

Table 6 Nighttime Crash Data Summary by Region	27

Table 7 AADT Descriptive Statistics	27

Table 8 Number of Lanes for Different Segment Types	28

Table 9 Lane Width Descriptive Statistics	28

Table 10 Left Shoulder Width Descriptive Statistics	29

Table 11 Right Shoulder Width Descriptive Statistics	29

Table 12 Median Width Descriptive Statistics	29

Table 13 Median Type Distribution	30

Table 14 Median Barrier Type Distribution	30

Table 15 Weaving Segment Data Summary	30

Table 16 Curve Data Summary	31

Table 17 Shoulder Rumble Strip Data Summary	31

Table 18 Area Type for Different Segment Types	32

Table 19 Ramp Meter Data Summary	32

Table 20 IRI Descriptive Statistics	33

Table 21 Summary of Data for Freeway and Interchange Segments	33

Table 22 Summary of Data for Interchange Ramps	34

Table 23 Crash Rate Outlier Threshold (Crash/100 Million VMT)	37

Table 24 Crash Rate Data Descriptive Statistics	37

Table 25 Weighted T-Test Result for Mean Crash Rates	41

Table 26 Injury Severity Distribution for Different Crash Categories	42

Table 27 Injury Severity Distribution for Midwest States (11)	42

Table 28 Proportion Test Result with Respect to Fatal + Incapacitating Injury Crashes (K+A)	43

Table 29 Proportion Test Result with Respect to Fatal Crashes (K)	43

Table 30  Proportion Test Result with Respect to Fatal + Injury Crashes (K+A+B+C)	43

Table 31 Variables for Characteristics Comparisons	45

Table 32 Month Distribution for Different Crash Categories	46

Table 33 Proportion Test Results with Respect to Winter Months	46

Table 34 Hour Distribution for Different Crash Categories	47

Table 35 Crash Type Distribution	48

Table 36 Proportion Test Results with Respect to Single-Vehicle Crashes	49

Table 37 Vehicle Damage Type Distribution	49

Table 38 Median Barrier Distribution	50

Table 39 Proportion Test Result for Roadways with Median Barrier	51

Table 40 Horizontal Curve Summary	51

Table 41  Proportion Test Result for Horizontal Curves	52

Table 42 Vertical Curve Summary	52

Table 43 Proportion Test Result for Vertical Curves	53

Table 44 Roadway Conditions Distribution	53

Table 45  Proportion Test Result for Adverse Roadway Conditions	54

Table 46 Speed Limit Distribution	55

Table 47 Weather Conditions Distribution	56

Table 48  Proportion Test Result for Non-clear/cloudy Weather Conditions	56

Table 49 Driver Gender Distribution	58

Table 50 Proportion Test Result for Driver Gender	58

Table 51 Driver Contributing Factors	59

Table 52 List of SPFs Developed	61

Table 53 Potential Independent Variables Considered in SPFs for Interchange Ramps	61

Table 54 Potential Independent Variables Considered in SPFs for Freeway Segments	62

Table 55 Potential Independent Variables Considered in SPFs for Interchange Segments	62

Table 56 Significant Factors in the Final SPFs for Freeway Segments	64

Table 57 Significant Factors in the Final SPFs for Interchange Segments	64

Table 58 Significant Factors in the Final SPFs for Interchange Ramps	65

Table 59 Injury Severity Distribution for Freeway Segments	87

Table 60 Injury Severity Distribution for Interchange Segments	87

Table 61 Injury Severity Distribution for Interchange Ramps	87

Table 62 Sweet Spots Ranges for Geometric Features	88

Table 63 Worst Case Nighttime Crash Frequency (Freeway & Interchange Segment)	89

Table 64 Worst Case Nighttime Crash Frequency (Interchange Ramp)	89

Table 65 Reduction of Nighttime Crashes by Treatments (4-Lane Freeway Segment)	90

Table 66 Reduction of Nighttime Crashes by Treatments (6-Lane Freeway Segment)	90

Table 67 Reduction of Nighttime Crashes by Treatments (4-Lane Interchange Segment)	91

Table 68 Reduction of Nighttime Crashes by Treatments (6-Lane Interchange Segment)	91

Table 69 Reduction of Nighttime Crashes by Treatments (1-Lane Entry Ramp)	92

Table 70 Reduction of Nighttime Crashes by Treatments (1-Lane Exit Ramp)	92

Table 71 Reduction of Nighttime Crashes by Treatments (2-Lane Entry Ramp)	92

Table 72 Reduction of Nighttime Crashes by Treatments (2-Lane Exit Ramp)	93

Table 73 National Safety Council Crash Unit Costs	105

Table 74 Ten-Year Lighting Benefits for Freeway and Interchange Segments	106

Table 75 Ten-Year Lighting Benefits for Interchange Ramps	106

Table 76 Ten-Year B/C Ratio for Freeway and Interchange Segments	108

Table 77 Ten-Year B/C Ratio for Interchange Ramps	108

Table 78 Lighting Benefit for Four-Lane Freeway Segments	111

Table 79 Lighting Benefit for Six-Lane Freeway Segments	112

Table 80 Lighting Benefit for Four-Lane Urban Interchange Segments	113

Table 81 Lighting Benefit for Four-Lane Rural Interchange Segments	114

Table 82 Lighting Benefit for Six-Lane Urban Interchange Segments	115

Table 83 Lighting Benefit for Six-Lane Rural Interchange Segments	116

Table 84 Lighting Benefit for 1-Lane Entry Urban Interchange Ramps	117

Table 85 Lighting Benefit for 1-Lane Entry Rural Interchange Ramps	118

Table 86 Lighting Benefit for 1-Lane Exit Urban Interchange Ramps	119

Table 87 Lighting Benefit for 1-Lane Exit Rural Interchange Ramps	120

Table 88 Lighting Benefit for 2-Lane Entry Interchange Ramps	121

Table 89 Lighting Benefit for 2-Lane Exit Interchange Ramps	122

# List of Figures

Figure 1 Allocation of Daytime and Nighttime Hours by Month for St. Cloud, MN (22)	6

Figure 2 Typical Interchange Types (41)	14

Figure 3 Weaving Types (42)	15

Figure 4 Typical Ramp Types (43)	15

Figure 5 Components of Freeway and Interchange	18

Figure 6 Map of Freeways and Interchanges in Wisconsin	19

Figure 7 Freeway and Lighting Maps for	20

Figure 8 Example of U.S. Naval Observatory Data (Central Standard Time)	24

Figure 9 Light Pole Spacing and Illumination Range	25

Figure 10 Crash Report Review Process	25

Figure 11 Nighttime Crashes Classification	26

Figure 12 Interchange Type Distribution	31

Figure 13 Ramp Type Distribution	32

Figure 14 Crash Rate Boxplots for Each Segment Type	37

Figure 15 Freeway Segments Crash Rate Comparison (Outlier Removed)	39

Figure 16 Interchange Segments Crash Rate Comparison (Outlier Removed)	39

Figure 17 Interchange Ramps Crash Rate Comparison (Outlier Removed)	40

Figure 18 Injury Severity Distribution for Different Crash Groups	42

Figure 19 Month Distribution for Different Crash Categories	46

Figure 20 Hour Distribution for Different Crash Categories	47

Figure 21 Crash Type Distribution	48

Figure 22 Vehicle Damage Type Distribution	49

Figure 23 Median Barrier Distribution	50

Figure 24 Horizontal Curve Summary	51

Figure 25 Vertical Curve Summary	52

Figure 26 Roadway Conditions Distribution	54

Figure 27 Speed Limit Distribution	55

Figure 28 Weather Conditions Distribution	56

Figure 29 Driver Age Distribution	57

Figure 30 Driver Gender Distribution	58

Figure 31 Driver Contributing Factors	59

Figure 32 Base Model Comparisons for Freeway Segments	72

Figure 33 Base Model Comparisons for Interchange Segments	73

Figure 34 Base Model Comparisons for One-Lane Interchange Ramps	73

Figure 35 Base Model Comparisons for Two-Lane Interchange Ramps	74

Figure 36 CMFs for Lighted Four-Lane Freeway Segments	75

Figure 37 CMFs for Lighted Six-Lane Freeway Segments	75

Figure 38 CMFs for Unlighted Four-Lane Freeway Segments	76

Figure 39 CMFs for Unlighted Six-Lane Freeway Segments	77

Figure 40 CMFs for Lighted Four-Lane Interchange Segments	78

Figure 41 CMFs for Lighted Six-Lane Interchange Segments	79

Figure 42 CMFs for Unlighted Four-Lane Interchange Segments	80

Figure 43 CMFs for Unlighted Six-Lane Interchange Segments	81

Figure 44 CMFs for Lighted One-Lane Entry Ramps	81

Figure 45 Lane Width CMFs for Lighted One-Lane Exit Ramps	82

Figure 46 CMFs for Unlighted One-Lane Entry Ramps	83

Figure 47 CMFs for Unlighted One-Lane Exit Ramps	83

Figure 48 CMFs for Lighted Two-Lane Entry Ramps	84

Figure 49 CMFs for Lighted Two-Lane Exit Ramps	84

Figure 50 CMFs for Unlighted Two-Lane Entry Ramps	85

Figure 51 CMFs for Unlighted Two-Lane Exit Ramps	86

Figure 52 Illustration of the Proactive Tool – Main Page	94

Figure 53 Illustration of the Proactive Tool – Analysis Worksheet	96

Figure 54 Illustration of the Proactive Tool – Empirical Bayes Analysis	97

Figure 55 Nighttime Crash Risk Map for Unlighted Freeway Segments	99

Figure 56 Nighttime Crash Risk Map for Unlighted Interchange Segments	100

Figure 57 Nighttime Crash Risk Map for Unlighted Interchange Ramps	101

Figure 58 Lighting Benefit Map for Unlighted Freeway Segments	102

Figure 59 Lighting Benefit Map for Unlighted Interchange Segments	103

Figure 60 Lighting Benefit Map for Unlighted Interchange Ramps	104

Figure 61 Freeway Median Light Poles with Twin Luminaires (Image Source: Wikipedia)	107

# Introduction

Although the number of fatal crashes that occurred in daylight is about the same as those in darkness, only 25 percent of vehicle-miles traveled occur at night (1). As a result, the nighttime fatality rate is three times the daytime rate. According to Fatality Analysis Reporting System 2012 data, approximately 50 percent more fatalities occurred on dark unlighted roadways than on lighted ones (2). A National Cooperative Highway Research Program (NCHRP) report states that darkness (or the absence of lighting) resulted in a disproportionately large number (in relation to exposure) of crashes and fatalities (3). In this context, there is a need to address nighttime traffic safety issues. Many sources have identified lighting as a proven safety countermeasure to address roadway crashes during nighttime (1). Published studies using different methodologies tended to converge on an average of 20 to 30 percent reduction in nighttime crash risk by installing lighting (3). A Minnesota report indicated that the installation of street lights at rural intersections reduced the nighttime crash frequency by 26 to 40 percent, the nighttime crash rate by 25 to 40 percent, the nighttime single vehicle crash frequency by 29 to 53 percent, the nighttime multiple vehicle crash frequency by 63 percent, and the nighttime crash severity by 26 percent (4). In addition, a Federal Highway Administration (FHWA) report indicates that intersection illumination has the greatest benefit to cost ratio of all intersection safety improvements (5).

Although lighting proved to be a high benefit/cost countermeasure, it is not feasible to light every mile of the highway due to financial and environmental implications. The American Association of State Highway and Transportation Officials (AASHTO) Roadway Lighting Design Manual has lighting warrants for continuous freeway, complete interchange and partial interchange lighting for freeways (6). However, the warrants do not represent a requirement for lighting. They are only an indication of situations where lighting should be considered. While there are some states that have this policy, most have modified the AASHTO warrants.

The biggest issue is that most of the states have not looked holistically at the safety and operations of a roadway with regards to lighting. And in most cases the lighting guidelines currently in use are not based on historical crash data. Therefore, it is necessary to develop guidelines for determining where lighting should be provided in Wisconsin based on historical crash data. These guidelines will objectively look at safety to determine lighting installation. Freeways and interchanges are two of the most important components of a national highway system due to providing greater mobility compared with other roadways. However, they are also sources of a high number of crashes, with more than 5,000 fatal crashes occurring in 2012 (2). Therefore, this report put particular focus on analyzing and quantifying the safety benefit of providing lighting on freeways and interchanges.

# Project Goals and Objectives

The goal of this project is to develop lighting guidelines for Wisconsin by considering safety benefits due to lighting installations on freeways.

Specific objectives of this project are listed as follows:

- Collect lighting and other relevant data on statewide freeways.

- Conduct quantitative nighttime crash analysis with respect to roadway lighting.

- Develop nighttime safety performance functions (SPFs) for the following three roadway categories based on historical crash data.

  - Freeway Segments

  - Interchange Segments

  - Interchange Ramps

- Develop tools based on the SPFs to facilitate quantifying the safety benefits of lighting and assist with decision making for lighting installation. Two types of tools will be developed:

  - Proactive Tool: in the planning phase to assist decision making of installing lighting and other safety treatments.

  - Reactive Tool: identify locations where lighting is needed and the benefit that can be obtained

- Develop lighting guidelines based on the quantified highest safety benefits by providing lighting.

# Literature Review

This section presents a comprehensive review of the state-of-the-practice literature related to lighting warrants/guidelines and safety analysis. Topics covered include lighting guidelines, lighting safety benefit analysis, lighting technology and economics, and interchange safety analysis.

## Lighting Design and Warrant Guidelines

### National Level Lighting Guidelines

Current national level lighting guidelines include the American Association of State Highway and Transportation Official (AASHTO) Lighting Design Guide and the FHWA Lighting Handbook. The AASHTO Guide provides a general overview of lighting systems from the point of view of the transportation departments and recommends minimum levels of quality. The guide contains lighting warrants for continuous freeway, complete interchange, and partial interchange lighting for freeways based on average daily traffic volume, geometry of interchanges, proximity of developments and night/day crash rate (6).

However, warrants for other functional classes of highways are not specified in this document. For rural highways, lighting of spot locations in rural areas should be considered whenever the driver is required to pass through a section of road with complex geometry or raised channelization. The lighting design treatment is typically similar to that for freeway ramp terminals (6). Also, the warrants do not represent a requirement for lighting, but are only an indication of situations where lighting should be considered. Most states follow this guideline to create their own lighting requirements.

The FHWA Lighting Handbook provides information on policy and guidance, basic terms and concepts, warranting criteria, lighting impacts, application considerations and other systems and issues which are closely related highway lighting (1). Apart from freeways and interchanges as mentioned in AASHTO book, it also presents a warranting method for other types of roadways, which is based on factors grouped into geometric, operational, environmental, and crash factors. For each factor a numeric rating (R) from 1 to 5 corresponding to the defined criterion is assigned. Each criterion is assigned a weight (W) to indicate its relative importance. The rating value (R) is multiplied by the weight (W) to obtain a point-score (R x W) for each criterion characteristic, indicating its relative significance. The overall point-score for all items indicates the need for lighting, as well as the relative risk on that road compared to other roadways (1).

### State Level Lighting Guidelines

Most states follow the AASHTO guideline to create their own lighting requirements as an individual document or as part of a document for the functional class listed. The following paragraphs list the relevant documents from different states.

California

California addressed lighting issues in their Traffic Manual. It generally states to install roadway lighting if a disproportionate number of crashes occur at night (7).

Colorado

The Colorado Lighting Design Guide mostly adopts AASHTO standards for freeways and interchanges. However, for city street lighting, the following warrant conditions are stated:

- The respective governmental agencies concur that lighting will contribute to the efficiency, safety and comfort of motorists and pedestrians;

- Streets where the ratio of nighttime to daytime accidents is high;

- Locations where severe or unusual weather or atmospheric conditions exist; and

- Locations where the local governmental agency is willing to pay an appreciable percentage of or wholly finance the lighting installation. (8).

Delaware

The Delaware Lighting Design Guidelines indicated the locations that lighting should or may be considered, but did not provide any details or justifications (9).

Illinois

Illinois Bureau of Design and Environment Manual states that facilities with raised medians, major urban arterials, or pedestrian sidewalks are locations where lighting is needed. Locations with a high night-to-day crash ratio, high conflict location and intersection with a high crash rate should also be considered for lighting (10).

Minnesota

Minnesota Department of Transportation Roadway Lighting Design Manual and the Minnesota Traffic Engineering Handbook (11) indicated modified warrants on the basis of AASHTO guidelines for continuous roadways. For other roadways, it requires lighting be considered when the following conditions exist at intersections:

- Volume: The traffic signal warrant volumes for the minimum vehicular volume warrant, the interruption of continuous traffic warrant, or the minimum pedestrian volume warrant are satisfied for any single hour during conditions other than daylight, excluding the time period between 6:00 a.m. and 6:00 p.m.

- Crashes: There are three or more crashes per year occurring during conditions other than daylight. Currently, thresholds for ratios of night to day crash rates are being developed for non-freeway facilities. Check the Traffic Engineering Manual for updates.

- Intersecting Roadway: The intersecting roadway is lighted.

- Ambient Light: Illumination in areas adjacent to the intersection adversely affects the drivers' vision.

- Channelization: The intersection is channelized and the 85th percentile approach speed exceeds 40 miles per hour. A continuous median is not considered as channelization for the purpose of this warrant.

- School Crossing: Scheduled events occurring at least once per week during the school year make it necessary for 100 or more pedestrians to cross at the school crossing during any single hour in conditions other than daylight, or a traffic engineering study indicates a need for lighting.

- Signalization: The intersection is signalized.

- Flashing Beacons: The intersection has a flashing beacon. (12)

New Hampshire

New Hampshire Traffic Design Manual used both AASHTO guidelines for freeways and Minnesota requirements for other roadways (13).

New Jersey

NJDOT Design Manual indicates that for non-controlled access facilities, a specially designed form is used to determine whether lighting is warranted. The form considers geometric, operational, environmental and accident factors (14).

Oregon

Oregon Lighting and Design Policy stated that for non-freeways lighting may be warranted if a crash analysis indicates that at least thirty-percent of crashes occur at night and the total crash rate for the section is above the statewide average for similar roadway character (15). Also, a spot location may be considered for lighting when at least thirty percent of crashes occur at night or the ratio of the nighttime crash rate to the day crash rate is 1.5 or higher. Crash rates for spot locations should be calculated on a per million entering vehicle basis. Engineering judgment and other factors should be considered (15).

Texas

Texas Illumination Manual also used spot lighting safety lighting. Spot lighting usually consists of one to five units intended to illuminate a nighttime hazard, such as sections with complex geometry or raised channelization (16).

Washington

Washington Department of Transportation Design Manual indicated conditions for additional illumination at diminished level of service, high nighttime collision frequency and nighttime pedestrian accident locations (17).

Wisconsin

Wisconsin Facilities Development Manual indicated that there are several cases where safety concerns have been evaluated and lighting is always installed, including:

- Signalized intersections;

- Roundabouts; and

- Milwaukee area freeways.

For other roadway types, lighting is typically not installed unless it can be proven that the lack of illumination is the cause of the accidents/confusion at the site and the installation of lighting is the only remedy (18). In addition, the document stated that “the Bureau of Traffic Operations - Electrical Engineering and Electronic Unit must approve all proposed continuous lighting system installations on state truck highways. If there is a possibility that the project may include the installation of new continuous lighting, the designer shall work with the Region traffic section to submit DT1198 the "Roadway Lighting Approval Request". The designer should begin the lighting permitting process as soon as practical. These documents should be submitted and approved before any commitments are made concerning the installation of highway lighting.” (18)

## Separation of Daytime/Nighttime

To analyze the safety benefits of roadway lighting, nighttime crashes and crashes that were possibly caused by the lack of roadway lighting must be accurately identified. A Washington Department of Transportation report indicated that the time of year should be considered when determining whether a collision occurred at night. A collision occurring at 5:00 P.M. in July would be a daytime collision, but a collision occurring at the same time in December would be during hours of darkness (17).

Most research used lighting related options in crash reports to identify nighttime crashes. A Florida research report used crashes where lighting conditions were reported as “Dark (No Street lighting)” to identify nighttime unlit crashes (19). A Kentucky research report also used “darkness-roadway lighting off” as an indication of nighttime unlighted crashes. Crashes were separated into three groups: crashes occurring during daylight hours, crashes during nighttime hours, and crashes during nighttime hours with no roadway lighting (20).

When considering periods of dawn and dusk, a Minnesota Department of Transportation reported that, because of the difficulty of distinguishing dusk and dawn from daytime or nighttime, this part of the crash data was omitted from the study and it only included those clearly marked with daytime and nighttime (21).

A study completed by Iowa State University used U.S. Naval Observatory data to identify sunset and twilight time and used to determine when daytime and nighttime hours by month occurred for different research locations (22). Figure 1 shows an example of allocation of daytime/nighttime hours for St. Cloud, MN based on U.S. Naval Observatory data. This method will be more accurate than using the options from crash reports.

Figure 1 Allocation of Daytime and Nighttime Hours by Month for St. Cloud, MN (22)

## Lighting Safety Analysis

### Before-and-After & Comparison Analysis

A substantial amount of research was completed to compare the safety benefits of roadway lighting before and after its installation. Isebrands et al. found a 31 percent lower night-to-day crash ratio with lighting at rural isolated intersections (23). Preston et al. conducted before and after studies and showed a reduction in night-to-day crash ratios of 22 to 40 percent (24). A meta-analysis of 37 research studies indicated a 30 percent reduction in nighttime intersection crashes (25). The Highway Safety Manual (HSM) expected a 38 percent reduction in nighttime fatal and injury intersection crashes after lighting (26).

A study based on Dutch crash data showed that injury accidents are reduced by 50 percent after lighting. The effect of road lighting is significantly smaller during adverse weather and road surface conditions than during fine conditions. The effects on pedestrian, bicycle and moped accidents are significantly larger than the effects on automobile and motorcycle accidents (27).

Bruneau et al. compared the safety benefits of continuous freeway lighting and interchange-only lighting. Special motorway maps with section referencing were used to collect the study parameters: lighting type, length of lighted areas, and the number and location of interchanges. They found that continuous lighting reduces the overall accident rate by 33 percent in comparison with interchange lighting alone and by 49 percent compared to no lighting conditions. Furthermore, a breakdown by categories of average daily traffic for these comparisons reveals that accident reductions are still valid regardless of traffic flow. Finally, the authors recommended that continuous lighting be added to sections of motorways with ADT of over 40,000 (28).

A Minnesota research report indicated that nighttime crashes were significantly reduced at rural at-grade intersections when lighting was installed. Lighting resulted in a 25 to 50 percent reduction in the nighttime crash rate and a 20 to 30 percent reduction in the nighttime crash to total crash ratio (21).

A study completed by Iowa State University evaluated the effectiveness of rural street lighting in reducing nighttime crashes at isolated rural intersections. Two methods were used to analyze the intersections crash data for Minnesota (22). A comparative study concluded that unlighted intersections had a ratio of night to total crashes 27 percent higher than lighted intersections. A before-and-after analysis showed a 27 percent reduction in nighttime crash frequency.

### Performance Measure

A number of performance indices were used in different researches to evaluate daytime and nighttime crashes. A list of performance measures is shown below.

Night-to-Day Crash Ratios (3)

N/D= NDw-NDwoNDwo 						(1)

Where,

N/D	=night-to-day crash ratio;

N 	=number of nighttime crashes, aggregated over a defined time period and roadway type;

D 	=number of daytime crashes, aggregated over a defined time period and roadway type;

w	= subscript to denote the presence of fixed roadway lighting on the respective roadway type; and

wo 	= subscript to denote the absence of fixed roadway lighting on the respective roadway type.

Modified Crash Ratios (19)

Lee et al. developed two parameters through a modification of night-to-day crash ratios by considering traffic volume and vehicle-mile-traveled.

NCI1=NNightVNight/NDayVDay						(2)

Where,

NCI1	=Nighttime crash Index 1;

NNight 	=number of nighttime crashes;

NDay	=number of daytime crashes;

VNight	=percentage of nighttime traffic volume; and

VDay	=percentage of daytime traffic volume.

NCI2=NNight/AvgNNight					(3)

Where,

NCI2		=Nighttime crash Index 2;

NNight		=Number of nighttime crashes per mile per 10,000 vehicles; and

AvgNNight	=Average number of nighttime crashes per mile per 10,000 vehicles.

Odds Ratio (27)

Odds Ratio= NNightLit/NDayLitNNightUnlit/NDayUnlit					(4)

Where,

NNightLit	=Number of nighttime crashes on lit roads;
 	NDayLit	=Number of daytime crashes on lit roads;

NNightUnlit	=Number of nighttime crashes on unlit roads; and

NDayUnlit	= Number of daytime crashes on unlit roads.

Relative Crash Rate Reduction (28)

The relative accident rate reduction, expressed as a percentage, is obtained by relating the smaller ratio to the larger one, shown in the following equation:

AR=1-R1R2*100 						(5)

Where,

AR 	= accident reduction;

R1	= ratio1 (smaller ratio) of (nighttime crashes/100 million vkmt) to (day crashes/100 million vkmt); and

R2 	= ratio2 (larger ratio) of (nighttime crashes/100 million vkmt) to (day crashes/100 million vkmt).

### Lighting Safety Characteristics Analysis

A Kentucky report stated that a large number of locations identified as having a nighttime crash rate were rural locations where the nighttime crashes can be addressed with improved delineation (20). They found out an overrepresentation of nighttime crashes compared to nighttime ADT. The higher percentage of crashes during darkness compared with the percentage of volume during this time period shows the higher crash rate at night. It was also concluded that nighttime crashes were reduced by approximately 45 percent after the addition of roadway lighting at intersections (20). Key comparison results of different characteristics of daylight crashes and dark-unlit crashes are shown as follows.

- Crashes occurring during darkness were more severe during daylight;

- A higher percentage of crashes during darkness occurred on weekends;

- A much higher percentage of crashes during darkness involved only one vehicle;

- Snow and ice conditions were present in a higher percentage of nighttime crashes;

- The percentage occurring on a curve was higher during darkness;

- The percentage varied as the number of hours of daylight changed with substantially more during daylight between June and August and much more during darkness between December and February.

- The percentage of crashes at an intersection was much higher during daylight conditions. Rear end collisions were more common during daylight. The types of crashes occurring more often during darkness were fixed object, run off roadway, animal, and shoulder/parked vehicle crashes.

- Crashes during daylight had a much higher percentage involving another vehicle while those during darkness more often involved a fixed object or animal.

- Factors listed substantially more often during darkness were animal action, alcohol, drugs, speed, and fell asleep. Factors listed substantially more often during daylight were failure to yield and inattention.

An NCHRP report investigated lighting and different crash types. It is assumed that the issues involving pedestrian and intersection crashes at night are primarily associated with visibility; hence these are scenarios in which lighting might have the greatest effect in regards to the reduction of nighttime crashes; run-off road crashes, on the other hand, are likely due in large part to factors (e.g., fatigue, intoxication) in which lighting is not expected to have a direct influence (29). The report stated that the positive safety benefits of roadway lighting appears to grow larger with increased roadway geometric complexity and population density and has the largest benefit at locations with pedestrian activity. The effect is smallest at locations where there are the fewest opportunities for pedestrian conflict and for conflicting traffic patterns, such as midblock locations (29).

Lee et al. constructed a linear relationship between average illumination levels and number of crashes. This research found out that in general, nighttime crashes at various illumination levels are proportionally distributed among the inventory of illumination levels for roadways (19).

Donnell et al. developed a framework for analyzing the benefits of roadway lighting. Several key issues were explored including the availability of relevant crash, lighting, and roadway inventory data; relevant data element structures; proposed analysis taxonomies to assess lighting-safety effects within and across different intersection classifications; specification and estimation of models to estimate expected crash frequencies during day and night; techniques to interpret model parameters, including variable elasticity; and tests of model transferability across states. A sample framework execution using Minnesota intersection data is provided. Results indicate a much lower overall safety benefit from lighting than published studies, but are consistent with estimates included in Highway Safety Manual research (30).

Gibbons et al. developed a model called the Dynamic Driver Visual Model (DDVM). Conceptually, a DDVM is a system of rules, statistics, and expectations that can be used to define how a driver collects visual information from the environment. Several information sources were investigated, and several dependent variables were identified. Researchers collected data on how information from signage, objects, lighting, pavement markings, and other vehicles moderates a driver’s visual search of the roadway environment. Logistic regression analyses were performed on the collected data to identify common characteristics to be implemented in the DDVM. These variables included age, lighting, vehicle headlamps, several different objects, glance time, target luminance, and contrast information (31). The following conclusions were reached:

- Moving objects, especially those in the lit condition, were detected sooner than those in the unlit conditions. Lighting that makes pedestrian and bicyclist movement visible provides a large attentional draw for a driver.

- Larger objects were detected sooner than smaller objects. However, these results can be modified by the type of material (e.g., signage).

- Age effects influenced detection distance of objects at the various cutoff distances.

- Target luminance influenced detection of target objects, especially at longer distances.

### Crash Modification Factors

The crash modification factor is defined as follows in the HSM, with the absence of lighting as a base condition.

CMFLighting=1.0-1.0-0.72*pinr-0.83*ppnr*pnr			(6)

Where,

pinr = proportion of total nighttime crashes for unlighted roadway segments that involve a fatality or injury;

ppnr = proportion of total nighttime crashes for unlighted roadway segments that involve property damage only;

pnr = proportion of total crashes for unlighted roadway segments that occur at night.

## New Lighting Technologies

After several decades of relatively slow and gradual change, light source technologies for roadway lighting are evolving rapidly. Many new options for roadway lighting are available, and there is more information about how light interacts with the human visual system (32).

The High Pressure Sodium (HPS) lamps are currently the most widely used. HPS lamps produce are popular because of their relatively low initial cost, efficiency, long useful lives, and ability to maintain relatively high light output throughout their lives. In the past decade or so, several alternatives to HPS have emerged:

Metal halide (MH) lamps: Newer MH lamps with ceramic arc tubes and new methods of starting have much increased efficiency, life and lumen maintenance.

Fluorescent and induction lamps: more recent fluorescent lamp types known as induction lamps, which use radio frequencies to stimulate the material in the lamp to produce light, are becoming more widespread.

Light-emitting dioxide (LED) sources: LED lighting systems potentially have very long rated lives–perhaps double that of HPS systems--and can exhibit good lumen maintenance when fixtures are designed with proper heat management. Well-designed LED outdoor luminaires can provide the required surface illumination using less energy and with improved uniformity, compared to HID sources (13). The following table illustrates the comparison of the lighting technologies mentioned above.

Table 1 Comparison of Different Lighting Types (32)

|  | High Pressure Sodium (HPS) | Metal Halide (MH) | Fluorescent Induction | Light Emitting Diode (LED) |
| --- | --- | --- | --- | --- |
| Efficacy (lumens/watt) | 80-120 | 60-110 | 60-90 | 70-120 |
| Power (watts) | 35-400 | 70-400 | 55-200 | 55-330 |
| Operating Life (hours) | 24,000-30,000 | 10,000-20,000 | 60,000 | 30,000-100,000 |
| Correlated Color Temp. (kelvins) | 2100 | 2800-4200 | 2700-6500 | 3000-8000 |

The City of Trinidad in southeast Colorado implemented a unique lighting design that involved embedding LED lighting into the concrete barriers of the overpass. This design limited the amount of light spilling from the overpass by keeping the light concentrated to the roadway (33). The results of this research indicate the lighting design is an alternative; however, additional considerations to the beam angle, beam width, beam height, and spacing may improve the implementation of the system (33).

Adaptive lighting has recently been proposed by FHWA and guidelines are provided in a research report. Through the development of a relationship between crashes and lighting, criteria have been chosen that affect the selection of the lighting level on a roadway. A lighting level selection methodology has been developed with these parameters. These parameters can also be used to actively determine a lighting level, meaning that the lighting system could be changed to adapt to the current needs of the driver and the roadway. The implementation of these systems requires a lighting control system and dimmable luminaires (34).

## Roadway Lighting Economics

Benefit cost analyses are common for evaluating traffic safety countermeasures. Many previous studies investigated the benefits and costs of highway illumination. The tangible capital and operational costs are easy to assess, but the intangible or intractable costs such as medical/ambulance services, productivity loss (including time delayed by traffic flow), mental health support, police/fire services, property loss and quality of life are largely unknown. The benefits are also difficult to quantify because the deferred safety and security costs constitute the benefits (29). A 1996 FHWA report indicated that illumination had a benefit/cost ratio of 26.8 and is ranked highest of all highway safety improvements (5).

Janoff and McCunney developed a complete methodology for conducting a benefit/cost analysis for roadway lighting systems (35). The formulas use input from several variables including crash rate, area type, population density, and visibility-related quantities along with crash and system costs. A more in-depth approach was created earlier at the Texas Transportation Institute where researchers combined various lighting design criteria and roadway geometrics in their analysis technique (36). The analysis was created to calculate costs on a per mile basis and included estimated costs associated with vehicle-illumination unit crashes.

An economic before-and-after analysis included in a Minnesota report shows a benefit/cost ratio of 15. The report concluded that illumination of rural intersections is a cost-effective nighttime crash countermeasure and safety improvement system superior to other, statistically unproven systems in use (12).

Multiple economic analyses were completed by Box for comparing illumination levels and lighting systems to crash costs as part of his various research projects. Box concluded in his study that the cost savings of improved lighting due to crash reductions was $253,000 (37).

## SPFs Development Method

Safety Performance Functions (SPFs) are statistical models used to estimate the mean crash frequency for a specific roadway type based on traffic volume, segment length and other influencing factors (38). In previous researches, different regression models and methods have been proposed or applied to develop safety performance functions.

The conventional linear regression model is the simplest model. Through the use of different transformation techniques, linear regression can flexibly model a variety of linear and non-linear relationships. However, the major disadvantage of conventional linear regression models with respect to crash modeling is that the number of crashes is a discrete, non-negative, small-value random variable. The use of linear regression models is not commonly utilized because crash data typically do not satisfy the assumptions of such models, normal error structure and constant error variance (39).

Generalized linear regression models (GLMs) are the most popular statistical models used in crash modeling. GLMs include a list of different models: Poisson models, negative binomial models, zero-inflated models and other models. Poisson models assume the error structure follows a Poison distribution. Negative binomial models assume the error structure follows a negative binomial distribution. Compared to conventional linear models, Poisson models and negative binomial models can better fit the crash data. Negative binomial models can also account for the problem of over-dispersion of crash data (40).

## Freeway/Interchange Safety Analysis

Background information on freeways/interchanges and relevant safety and lighting analysis literature are presented in this section.

### Interchange Types and Components

An interchange is defined as a “system of interconnecting roadways in conjunction with one or more grade separations that provides for the movement of traffic between two or more roadways or highways on different levels (26).” According to the AASHTO A Policy on Geometric Design of Highways and Streets (41), typical interchange types include the following (illustrations in Figure 2),

- Trumpet,

- Three-leg directional,

- One quadrant,

- Diamond,

- Single point urban interchange,

- Partial cloverleaf,

- Full cloverleaf, and

- All-directional four leg.

Figure 2 Typical Interchange Types (41)

According to the Highway Capacity Manual, freeways are constituted by three types of segments:

- Merge and diverge segments (ramps),

- Weaving segments, and

- Basic freeway segments.

Weaving segments can be further divided into one-sided weaving segments and two-sided weaving segments. The following figure illustrates examples of one-sided and two-sided weaving segments (42). Ramps also have different types of configurations. Typical ramp types are illustrated in Figure 4.

Figure 3 Weaving Types (42)

Figure 4 Typical Ramp Types (43)

### Freeway/Interchange Lighting

The AASHTO Roadway Lighting Design Guide defines warrants for Continuous Freeway Lighting, Complete Interchange Lighting and Partial Interchange Lighting based on warrant conditions including:

- Traffic volumes,

- Spacing of freeway interchanges,

- Lighting in adjacent areas, and

- Night-to-day crash ratio (6).

FHWA Roadway Lighting Handbook suggested that complete interchange lighting is the preferred practice, but where a complete interchange lighting system is not feasible to implement, a partial interchange lighting system comprised of two high-mast installations per ramp is recommended, with one fixture located on the inner ramp curve near the gore, and one fixture located on the outer curve of the ramp, midway through the controlling curvature (1).

### Freeway/Interchange Safety Performance Functions

The 1st edition of the Highway Safety Manual (HSM) includes specific prediction procedures for rural two-lane highways, rural multilane highways, and urban and suburban arterials. The following factors are listed in the HSM when constructing safety performance functions for roadway segments (HSM, 2010):

- Length of segment (miles)

- Number of lanes

- AADT (vehicles per day)

- Lane width (feet)

- Shoulder width (feet)

- Median width (feet)

- Speed limit (mph)

- Shoulder type (paved/gravel/composite/turf)

- Presence or absence of horizontal curve (curve/tangent)

  - Curve length

  - Curve radius

  - Presence or absence of spiral curve

  - Superelevation rate

- Grade (percent)

- Driveway density (driveway per mile)

- Presence or absence of roadway segment lighting

- Presence or absence of automated speed enforcement

- Heavy vehicle percentage (percent)

As a matter of fact, the HSM does not include a safety prediction methodology for freeways and interchanges (26). It only provided crash modification factors for converting at grade intersections to interchanges, interchange with crossroad above freeway and modify speed change lane design. To address this need, NCHRP Project 17-45 focused on the safety performance methodology and analysis tool for freeways and interchange. The report developed a prioritized list of factors for SPF’s and CMF’s on freeway segment and ramps. This was undertaken because of “the large number of candidate SPFs and CMFs that were identified and the realization that sufficient project resources would not be available to adequately address each potential influence through statistical analysis” (43). The prioritization process focused on consideration of four criteria.

- One criterion used represents the “importance” of the SPF or CMF.

- A second criterion used is the level of effort required to obtain the raw data needed to define the key (or only) input variables for a given SPF or CMF.

- A third criterion addresses the case where an SPF or CMF is associated with multiple input variables.

- A fourth criterion used is the relative need of the SPFs or CMFs for each of the freeway or interchange components.

The 17-45 Report incorporated data from three different states, and included detailed crash, traffic, geometry and other information. Based on the data, a series for SPFs and CMFs were developed for freeway segments, interchange ramps and ramp terminals. Each type was then separated according to factors such as area type, the number of lanes to predict multi-vehicle crashes, single-vehicle crashes, injury crashes and property damage-only crashes.

## Literature Summary

Lighting guidelines exist at both national and state levels. FHWA and AASHTO both have guidelines for lighting installation. Several states also have their own guidelines, but most are modified from on the AASHTO document. None of the states have developed lighting guidelines based on quantitative safety analysis results. Thus, lighting safety analysis needs to be conducted for developing state lighting guidelines.

Previous research revealed that lighting is an effective method to address traffic safety during nighttime. A 20 to 30 percent reduction in crashes was found in most previous studies. Numerous safety indices were developed to evaluate the safety benefits of lighting. Lighting was also shown to be the most cost-effective way to reduce crashes at rural intersections. In order to clearly differentiate nighttime crashes, a new method using USNO sunrise/sunset time was used, which yielded more accurate results when compared to options in crash reports. As new lighting types emerge, its benefit/cost compared to traditional lighting types must be assessed.

Freeways and interchanges are key components of the national highway system. The HSM provides few analysis methods and tools for freeways and interchanges. As a supplement, NCHRP 17-45 developed SPFs for freeways and interchanges. However, no lighting SPFs can be found. Thus, lighting SPFs needs to be developed in order to better assess the safety benefits of lighting based on Wisconsin data.

# Data Collection

Data collection was conducted for all roadways within the study scope (i.e., freeway segments, freeway interchange segments, and freeway interchange ramps), and consisted of the following elements that are introduced in each subsection:

- Lighting data;

- Crash data;

- Traffic volume;

- Geometric data; and

- Other data.

The scope of this project includes all freeways on the state trunk network (STN) and interchanges on freeways. The definitions of terminology can be found in Appendix A: Glossary. Expressway refers to limited access roadways with mostly grade separated interchanges and only a few access points such as driveways, and does not include signalized intersections. Therefore vehicle traffic on expressways 500 ft. away from at grade intersections is also considered uninterrupted flow and is also considered as a freeway. Note that intersecting arterial roadways and interchange ramp terminals are not included within the study scope.

Figure 5 Components of Freeway and Interchange

Wisconsin Freeway Map

Figure 6 Map of Freeways and Interchanges in Wisconsin

Figure 7 Freeway and Lighting Maps for

(a) Madison Area; (b) Milwaukee Area; (c) Green Bay-Appleton Area; (d) Northwest Region

The identification of freeways generally follows the designated freeway and expressway map according to Wisconsin Department of Transportation (WisDOT) (Revised Dec 31th, 2013) (44), with an addition of a few fully accessed controlled freeway segments that are not listed in the WisDOT map. The entire roadway network included in this project is illustrated by Figure 6. A detailed map of specific areas is shown in Figure 7.

Freeways were divided into smaller segments with a length of no more than 1.6 miles to ensure consistency of cross sections according to the evaluation of homogeneity of freeway segments using satellite maps.  These segments were classified into freeway segments and interchange ramps, while freeway segments were further categorized into interchange segments and non-interchange segments. Figure 5 shows key components of an interchange, where the interchange segment is illustrated.

Table 2 summarizes all segments identified with information of the total number of segments or average length for each type of segments. There are in total 1,229 non-interchange freeway segments, 519 interchange segments and 2,150 interchange ramps. The average length for non-interchange segments is approximately 1.2 miles, while the average length is 0.5 miles for interchange segments and 0.25 miles for interchange ramps.

Table 2 Segment Data Descriptive Statistics

| Type | Description | Counts/Miles |
| --- | --- | --- |
| Number of Segments | Number of Freeway Segments | 1,229 |
|  | Number of Interchanges | 536 |
|  | Number of Interchange Segments | 519 |
|  | Number of Interchange Ramps | 2,150 |
| Total Segment Length | Total Length of Freeway Segments (mi) | 1,465.2 |
|  | Total Length of Interchange Segments (mi) | 248.4 |
|  | Total Length of Freeways (mi) | 1,713.6 |
| Freeway Segment Length | Maximum Length of Freeway Segments (mi) | 1.832 |
|  | Minimum Length of Freeway Segments (mi) | 0.175 |
|  | Average Length of Freeway Segments (mi) | 1.192 |
| Interchange Segment Length | Maximum Length of Interchange Segments (mi) | 1.210 |
|  | Minimum Length of Interchange Segments (mi) | 0.110 |
|  | Average Length of Interchange Segments (mi) | 0.479 |
| Interchange Ramp Length | Maximum Length of Interchange Ramps (mi) | 1.396 |
|  | Minimum Length of Interchange Ramps (mi) | 0.063 |
|  | Average Length of Interchange Ramps (mi) | 0.258 |

Table B-1 in Appendix B lists the total length of freeway segments, number of interchanges, number of freeway segments and number of interchange ramps for each county in Wisconsin. The table is arranged in descending order with regards to freeway length. Dane County has the longest mileage of freeways, while Milwaukee County has the largest number of interchanges and interchange ramps. Among the 72 counties, 26 counties do not have any freeway or interchanges.

## Lighting Data

Lighting data were retrieved from State Light Pole database maintained by WisDOT. Lighting data were presented in the form of a lighting pole GIS shapefile that contains the locations of lighting poles on the State Trunk Network (STN). Only location information is included in the shapefile. No lighting type information is available.

Lighting data from STN data layer proved to be incomplete because several freeway locations where lighting exists did not have any lighting poles in the shapefile. As a solution, the project team manually checked all roadways included in the scope of work to identify missing lighting pole locations using Google Street View. As a result, lighting pole locations that were missing were eventually added to the STN lighting pole shapefile. Table 3 shows a list of manually added lighting pole locations.

Table 3 List of Added Lighting Pole Locations

| Route Type | Route Number | City/County | Location Detail |
| --- | --- | --- | --- |
| Interstate | I 43 | Waukesha | CTH O interchange |
| Interstate | I 43 | Green Bay | 1.63 mile segment |
| Interstate | I 94 | Milwaukee | 3.97 mile segment |
| Interstate | I 94 | Waukesha | CTH T interchange |
| Interstate | I 94 | Kenosha | ST 165 interchange |
| Interstate | I 794 | Milwaukee | 0.62 mile segment |
| Interstate | I 894 | Milwaukee | 1.12 mile segment |
| US Highway | US 41 | Winnebago | W Bell St., STH 76, Witzel Ave, W 9th Ave interchanges |
| US Highway | US 41 | Green Bay | CTH AAA, CTH G, CTH F interchanges |
| US Highway | US 41 | Waukesha | CTH Q interchange |
| US Highway | US 53 | Eau Claire | ST 29 interchange |
| State Highway | STH 23 | Sheboygan | 0.34 mile segment, N Taylor Dr. interchange |
| State Highway | STH 30 | Madison | 0.59 mile segment, US 151,US 51 interchanges |
| State Highway | STH 172 | Green Bay | 1.15 mile segment |

Table 4 summarizes lighting pole counts on different segment types. Only 7 percent of freeway segments, 17 percent of interchange segments, and 22 percent of ramps are lighted.  In terms of mileage, only 97.2 miles out of the 1,714 miles of freeway segments are lighted, which represents a mere 5.6 percent.

Table 4 Number of Lighted and Unlighted Segments by Type

| Segment Type | Segment Type | Lighting Type | Count | Percentage |
| --- | --- | --- | --- | --- |
| Freeway Segment | Statewide | Lighted | 92 | 7.4% |
|  |  | Unlighted | 1137 | 92.6% |
|  | Madison Area (Dane County) | Lighted | 9 | 9.5% |
|  |  | Unlighted | 86 | 90.5% |
|  | Milwaukee Area (Milwaukee and Waukesha County) | Lighted | 102 | 72.3% |
|  |  | Unlighted | 39 | 27.7% |
| Interchange Segment | Statewide | Lighted | 93 | 17.3% |
|  |  | Unlighted | 426 | 82.7% |
|  | Madison Area (Dane County) | Lighted | 6 | 11.1% |
|  |  | Unlighted | 48 | 88.9% |
|  | Milwaukee Area (Milwaukee and Waukesha County) | Lighted | 95 | 76.6% |
|  |  | Unlighted | 29 | 23.4% |
| Interchange Ramp | Statewide | Lighted | 474 | 22.0% |
|  |  | Unlighted | 1676 | 78.0% |
|  | Madison Area (Dane County) | Lighted | 58 | 26.0% |
|  |  | Unlighted | 165 | 74.0% |
|  | Milwaukee Area (Milwaukee and Waukesha County) | Lighted | 421 | 80.8% |
|  |  | Unlighted | 120 | 19.2% |

## Crash Data

Five years of crash data from 2008 to 2012 were collected and processed. Because the yearly number of nighttime crashes is small, especially for interchange ramps, five years of crashes were accumulated to minimize the proportion of segments with zero crashes. A large proportion of zero crash segments can negatively impact the accuracy of safety performance function (SPF) modeling and other statistical analysis (43).

Initial crash data were collected from the WisTransPortal crash database and included approximately 263,475 crashes for the entire STN network. WisTransPortal contains a relatively complete database of Wisconsin Traffic Accident Extract data from 1994 in Wisconsin (45). All the crash data was then mapped in ArcGIS environment. Freeway and interchanges crashes were filtered through spatial selections in ArcGIS, which resulted in 80,926 crashes.

Each crash was then classified as a daytime or nighttime crash. The cut-off times for daytime and nighttime were based on the United States Naval Observatory (USNO) Sunrise/Sunset time data rather than the “lighting conditions” field in the crash reports to ensure accuracy. The USNO data contains detailed sunrise and sunset time for each day of each year for over 22,000 locations in the U.S. (46). An example of the USNO Sunrise/Sunset data is shown in Figure 8.

Because different regions have similar sunrise and sunset time, the research team decided to use one set of sunrise and sunset time for each of the WisDOT defined regions. A major city in each region was chosen to represent the entire region. The following list shows the selected cities in each region:

- NW Region: Eau Claire

- NC Region: Wausau

- NE Region: Green Bay

- SW Region: Madison

- SE Region: Milwaukee

The research team also observed that a year difference in sunrise and sunset time at the same location was small enough to be ignored (i.e., one or two minutes). Therefore, data from 2010 was used to represent the five years data. Sunrise and sunset times in each month were then averaged. Table 5 summarizes the final sunrise/sunset time table used in this research. Using this time table, 32,180 crashes were identified as nighttime crashes.

Figure 8 Example of U.S. Naval Observatory Data (Central Standard Time)

Table 5 Table of Sunrise/Sunset Time in Different Regions of Wisconsin (Central Standard Time)

| Month | NW |  | NC |  | NE |  | SW |  | SE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sunrise | Sunset | Sunrise | Sunset | Sunrise | Sunset | Sunrise | Sunset | Sunrise | Sunset |
| Jan. | 07:39 | 16:53 | 07:32 | 16:47 | 07:25 | 16:38 | 07:25 | 16:51 | 07:18 | 16:47 |
| Feb. | 07:04 | 17:37 | 06:55 | 17:30 | 06:52 | 17:21 | 06:52 | 17:31 | 06:44 | 17:28 |
| Mar. | 06:16 | 18:14 | 06:06 | 18:08 | 06:04 | 17:58 | 06:08 | 18:06 | 06:04 | 18:03 |
| Apr. | 05:19 | 18:53 | 05:10 | 18:47 | 05:08 | 18:37 | 05:12 | 18:43 | 05:04 | 18:39 |
| May. | 04:36 | 19:30 | 04:27 | 19:24 | 04:25 | 19:13 | 04:32 | 19:16 | 04:25 | 19:13 |
| Jun. | 04:19 | 19:54 | 04:11 | 19:48 | 04:07 | 19:39 | 04:18 | 19:39 | 04:12 | 19:34 |
| Jul. | 04:36 | 19:48 | 04:29 | 19:40 | 04:21 | 19:34 | 04:34 | 19:33 | 04:30 | 19:26 |
| Aug. | 05:10 | 19:10 | 05:03 | 19:03 | 04:55 | 18:57 | 05:05 | 18:59 | 05:02 | 18:48 |
| Sep. | 05:47 | 18:14 | 05:40 | 18:05 | 05:31 | 18:03 | 05:39 | 18:07 | 05:36 | 17:55 |
| Oct. | 06:24 | 17:19 | 06:17 | 17:09 | 06:07 | 17:08 | 06:14 | 17:12 | 06:10 | 17:03 |
| Nov. | 07:05 | 16:37 | 06:79 | 16:28 | 06:48 | 16:25 | 06:53 | 16:32 | 06:49 | 16:25 |
| Dec. | 07:38 | 16:26 | 07:32 | 16:18 | 07:22 | 16:13 | 07:23 | 16:24 | 07:19 | 16:19 |

The next step was assigning nighttime crashes to each segment, and determining whether the crash occurred on a lighted or unlighted segment. According to the Highway Illumination Manual, effective light pole spacing ranges between 220 ft. and 270 ft. for different lighting pole types and placement (16). For this research, 120 ft. was used as the threshold for maximum illuminated range (half of 240 ft. light pole spacing), as shown in Figure 9. Nighttime crashes that occurred outside of maximum illuminated range were considered under unlighted conditions.

Figure 9 Light Pole Spacing and Illumination Range

Figure 10 Crash Report Review Process

To ensure data accuracy, a manual review of crash reports was performed to ensure the location accuracy of crash data. The crash report review is conducted for crashes at ramps or segment boundaries. The process of crash report review is illustrated in Figure 10.

In addition to nighttime crashes, daytime crashes that occurred under low-visibility conditions, such as adverse weather conditions like rain, snow or fog were also collected. Daytime low-visibility crashes may share similar characteristics with nighttime crashes. Daytime low-visibility crashes were collected based on the following criteria followed by a manual review.

- “Lighting Condition” marked as “Dark”; or

- “Low visibility” Flag on; or

- “Weather Conditions” marked as fog.

As a result, a total of 828 crashes were identified as daytime low visibility crashes. A summary of crash classification result is illustrated in Figure 11. A summary of nighttime and daytime low-visibility crashes for each region is presented in Table 6. A detailed list of crash numbers by county is summarized in Table B-2 in Appendix B.

Figure 11 Nighttime Crashes Classification

Table 6 Nighttime Crash Data Summary by Region

| Region | NW | NC | NE | SW | SE | State Total |
| --- | --- | --- | --- | --- | --- | --- |
| Total Length of Freeway (mi) | 360.46 | 309.20 | 323.87 | 448.63 | 271.81 | 1714 |
| Total Number of Crashes | 8,628 | 8,460 | 14,836 | 18,089 | 30,913 | 80,926 |
| Total Number of Nighttime Crashes | 4,136 | 3,959 | 6,079 | 7,280 | 10,726 | 32,180 |
| Nighttime / Total | 47.9% | 46.7% | 40.9% | 40.2% | 34.7% | 39.7% |
| Number of Nighttime Crashes on Freeway Segments | 3342 | 2782 | 3169 | 4849 | 4716 | 18858 |
| Number of Nighttime Crashes on Interchange Segments | 615 | 818 | 2,035 | 1,907 | 4,401 | 9,776 |
| Number of Nighttime Crashes on Interchange Ramps | 231 | 234 | 729 | 728 | 1,468 | 3,390 |
| Number of Daytime Low Visibility Crashes | 102 | 85 | 172 | 256 | 207 | 828 |

## Traffic Volume Data

Traffic volume as represented by the Annual Average Daily Traffic (AADT) was collected as a key variable in safety analysis and SPF modeling. AADT data was sourced from WisDOT Meta-Manager database. The five-year average AADT was used to represent the traffic volume within the entire study period. Note that the AADT data represents the combined volume of both directions.

Table 7 summarizes the statistics of the AADT data collected. The average AADT for freeway segments is about 31,000 veh/day. Interchange segments have a larger AADT of approximately 43,300 veh/day. Interchange ramps have an average AADT of approximately 3,800 veh/day.

Table 7 AADT Descriptive Statistics

| Segment Type | Descriptive Statistics (veh/day) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 4,500 | 159,000 | 31,070 | 28,892 |
| Interchange Segments | 4,600 | 157,000 | 43,351 | 35,865 |
| Interchange Ramps | 80 | 32,300 | 3,803 | 3,695 |

## Roadway Geometry Data

Roadway geometry data was collected in order to be further included as potential variables in SPF modeling. Geometric data that were collected are listed as follows.

- Number of lanes;

- Lane width;

- Left and right shoulder width;

- Median width and type;

- Curve proportion and radius;

- Median barrier presence and type;

- Weaving presence and type;

- Ramp and interchange type.

Note that various GIS shapefiles within the Wisconsin STN database were used to collect the geometric data.

### Number of Lanes

The number of lanes was collected as the total number of lanes for both travel directions. As summarized in Table 8, the majority of freeway segments and interchange segments are 4-lane segments. Eight-lane segments are relatively rare and are all lighted and within the Milwaukee area. One-lane ramps are prevalent, accounting for 81 percent of all interchange ramps.

Table 8 Number of Lanes for Different Segment Types

| Segment Type | Number of Lanes | Count of Segments | Percentage |
| --- | --- | --- | --- |
| Freeway Segment | 4 | 1054 | 85.8% |
|  | 6 | 165 | 13.4% |
|  | 8 | 10 | 0.8% |
| Interchange Segment | 4 | 378 | 72.8% |
|  | 6 | 135 | 26.0% |
|  | 8 | 6 | 1.2% |
| Interchange Ramp | 1 | 1745 | 81.2% |
|  | 2 | 368 | 17.1% |
|  | 3 | 37 | 1.7% |

### Lane Width

The AASHTO “Green Book” specifies design criteria for various roadway geometric elements.  A 12-ft lane width is specified for freeways as a desirable value. Ramp lane width ranges from 12 ft to 30 ft as referred in the Green Book (41). In this project, lane width data was collected from the Meta-Manager database. Manual review via Google aerial maps was performed verify and correct lane width values that look “unreasonable”. Table 9 summarizes the lane width data for different segment types.

Table 9 Lane Width Descriptive Statistics

| Segment Type | Descriptive Statistics (ft) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 11.0 | 20.0 | 12.03 | 0.33 |
| Interchange Segments | 12.0 | 20.0 | 12.07 | 0.56 |
| Interchange Ramps | 11.0 | 30.0 | 14.71 | 3.03 |

### Left Shoulder Width and Right Shoulder Width

According the “Green Book”, shoulder width ranges from 4 to 12 ft for freeway segments and 1 to 10 ft for interchange ramps. Shoulder width data was collected separately for left shoulder width and right shoulder width. A number of values were manually reviewed to validate and improve data quality. Descriptive statistics for the left shoulder width and the right shoulder width are summarized in Tables 10 and 11, respectively.

Table 10 Left Shoulder Width Descriptive Statistics

| Segment Type | Descriptive Statistics (ft) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 4 | 13 | 7.24 | 1.93 |
| Interchange Segments | 4 | 14 | 7.35 | 2.08 |
| Interchange Ramps | 2 | 17 | 6.21 | 2.31 |

Table 11 Right Shoulder Width Descriptive Statistics

| Segment Type | Descriptive Statistics (ft) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 4 | 15 | 10.22 | 1.76 |
| Interchange Segments | 6 | 16 | 10.31 | 1.55 |
| Interchange Ramps | 2 | 16 | 8.06 | 2.01 |

### Median Width

The “Green Book” specifies a minimum median width of 10 ft for urban freeways and 36 ft for rural freeways. Table 12 summarizes the median width descriptive statistics for different segment types.

Table 12 Median Width Descriptive Statistics

| Segment Type | Descriptive Statistics (ft) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 10 | 290 | 59.91 | 26.67 |
| Interchange Segments | 10 | 600 | 58.59 | 46.49 |

### Median Type

Median type was divided into four categories: grass being the most common, paved, wood and other. The median type distribution for different segment types is summarized in Table 13.

Table 13 Median Type Distribution

| Median Type | Number of Freeway Segments |  | Number of Interchange Segments |  |
| --- | --- | --- | --- | --- |
| Grass | 1011 | 82.2% | 398 | 76.7% |
| Paved | 124 | 10.1% | 111 | 21.4% |
| Wood | 77 | 6.3% | 6 | 1.2% |
| Other | 11 | 1.7% | 4 | 0.7% |

### Median Barrier Type

Both the existence and type of median barrier was collected. Only 15 percent of freeway segments and 27 percent of interchange segments have median barriers. Concrete barriers were the most common type of median barrier, while cable barriers and guardrails were also observed. A summary of median barrier type data is shown in Table 14.

Table 14 Median Barrier Type Distribution

| Median Type | Number of Freeway Segments |  | Number of Interchange Segments |  |
| --- | --- | --- | --- | --- |
| Cable | 32 | 2.6% | 16 | 3.1% |
| Concrete | 143 | 11.6% | 120 | 23.1% |
| Guardrail | 5 | 0.4% | 4 | 0.8% |
| No Barrier | 1049 | 85.3% | 379 | 73.0% |

### Weaving

The occurrence of a weaving segment may have a negative impact on safety. A segment is considered a weaving segment when there is less than 2000 ft between adjacent interchanges. Only 6 percent of all segments are identified as weaving segments, among which more than a half exists in the Madison and Milwaukee areas, as summarized in Table 15.

Table 15 Weaving Segment Data Summary

| Type | Count | Percentage |
| --- | --- | --- |
| Non-Weaving Segments | 1165 | 94.1% |
| Weaving Segments | 73 | 5.9% |
| Madison Area Weaving Segments | 15 | 20.5% |
| Milwaukee Area Weaving Segments | 28 | 38.4% |
| Other Area Weaving Segments | 30 | 41.1% |

### Horizontal Curve

Both curve occurrence and curve radius data was collected. More than two thirds of freeway segments and 40 percent of interchange segments contained horizontal curves. A summary of curve data collected is shown in Table 16.

Table 16 Curve Data Summary

| Type | Count | Percentage |
| --- | --- | --- |
| Freeway Segments with curve | 797 | 64.9% |
| Freeway Segments without curve | 432 | 35.1% |
| Interchange Segments with curve | 223 | 42.3% |
| Interchange Segments without curve | 296 | 57.7% |

### Shoulder Rumble Strip

Rumble strips are applied along the direction of travel following an edge line or centerline to alert drivers when they drift from their lane. In this project, shoulder rumble strip data was collected. Table 17 summarizes the shoulder rumble strip data for different segment types.

Table 17 Shoulder Rumble Strip Data Summary

| Type | Count | Percentage |
| --- | --- | --- |
| Freeway Segments with Shoulder Rumble Strip | 945 | 76.3% |
| Freeway Segments without Shoulder Rumble Strip | 293 | 23.7% |
| Interchange Segments with Shoulder Rumble Strip | 317 | 60.3% |
| Interchange Segments without Shoulder Rumble Strip | 209 | 39.7% |

### Interchange Type

Interchange type data was collected in this project. The distribution of interchange types is shown in Figure 12. More than 60 percent of all interchanges are diamond interchanges. Partial cloverleaf and half-diamond interchanges are the second and third most common types. Examples of interchange types can be found in Appendix A: Glossary.

Figure 12 Interchange Type Distribution

### Ramp Type

Ramp type data was collected in this research. Distribution of ramp types is shown in Figure 13. More than 60 percent of all ramps are diamond ramps. Examples of ramp types can be found in Appendix A: Glossary.

Figure 13 Ramp Type Distribution

## Other Data

In addition to the variables summarized in Sections 4.1 through 4.4, additional relevant data was collected. The data includes area type, presence of ramp meter, and International Roughness Index (IRI).

### Area Type

Table 18 shows the area type for different segment types. Approximately 30 percent of freeway segments and more than 50 percent of interchange segments and interchange ramps are in urban area.

Table 18 Area Type for Different Segment Types

| Segment Type | Area Type | Count | Percentage |
| --- | --- | --- | --- |
| Freeway Segment | Urban | 362 | 29.5% |
|  | Rural | 867 | 70.5% |
| Interchange Segment | Urban | 280 | 53.9% |
|  | Rural | 239 | 46.1% |
| Interchange Ramp | Urban | 1190 | 55.3% |
|  | Rural | 960 | 44.7% |

### Presence of Ramp Meter

The presence of ramp meters on interchange ramps is summarized in Table 19. All metered ramps were identified as being located in the Madison and Milwaukee areas.

Table 19 Ramp Meter Data Summary

| Type | Count | Percentage |
| --- | --- | --- |
| Ramps with Meter | 126 | 5.9% |
| Ramps without Meter | 2024 | 94.1% |
| Madison Area Ramp Meter | 8 | 6.3% |
| Milwaukee Area Ramp Meter | 118 | 93.7% |
| Other Area Ramp Meter | 0 | 0.0% |

### IRI

The IRI is used as a surrogate measure for pavement friction values. As IRI decreases, pavement friction decreases and roadway surface becomes smoother. Table 20 shows the summary of the IRI data for different segment types.

Table 20 IRI Descriptive Statistics

| Segment Type | Descriptive Statistics |  |  |  |
| --- | --- | --- | --- | --- |
|  | Min | Max | Mean | Std. Dev. |
| Freeway Segments | 0.501 | 3.891 | 1.704 | 0.626 |
| Interchange Segments | 0.548 | 3.891 | 1.722 | 0.572 |

## Data Summary

Table 21 summarizes all data elements collected for freeway and interchange segments. Data collected generally conforms to those data elements used in the NCHRP 17-45 report. International Roughness Index, lighting and speed limit were additional data elements due to the need of this project.

Table 21 Summary of Data for Freeway and Interchange Segments

| NCHRP 17-45 Variables | Variables in this Project | Type | Range |
| --- | --- | --- | --- |
| AADT | AADT | Continuous | See Table 4 |
| Length | Length | Continuous | See Table 4 |
| Number of Lanes | Number of Lanes | Categorical | 4, 6 or 8 |
| Area Type | Area Type | Categorical | Urban/Rural |
| Lane Width | Lane Width | Continuous | In ft. |
| Outside Shoulder Width | Outside Shoulder Width | Continuous | In ft. |
| Inside Shoulder Width | Inside Shoulder Width | Continuous | In ft. |
| Median Width | Median Width | Continuous | In ft. |
| Weaving Type | Weaving Type | Categorical | Weaving, Non-weaving |
| Curve Proportion | Curve Proportion | Continuous | 0 to 1 |
| Curve Radius | Curve Radius | Continuous | In ft. |
| Rumble Strip | Rumble Strip | Categorical | Y/N |
| Shoulder Rumble Strips | Shoulder Rumble Strips | Categorical | Y/N |
| Median Barrier Proportion | Median Barrier Proportion | Continuous | 0 to 1 |
| Median Barrier Type | Median Barrier Type | Categorical | Concrete, Cable, Guardrail, No Barrier |
| Median Type | Median Type | Categorical | Grass, paved, wood, other |
|  | Lighting | Categorical | Y/N |
|  | International Roughness Index | Continuous |  |
|  | Speed Limit | Categorical | 45,55 or 65 mph |
| Horizontal Clearance |  |  |  |
| Grade |  |  |  |

Table 22 summarizes all data elements collected for interchange ramps.

Table 22 Summary of Data for Interchange Ramps

| NCHRP 17-45 Variables | Variables in this Project | Type | Range |
| --- | --- | --- | --- |
| AADT | AADT | Continuous | See Table 4 |
| Length | Length | Continuous | See Table 4 |
| Ramp Type | Ramp Type | Categorical | Diamond, ParClo Loop, Outer Connection etc. |
| Interchange Type | Interchange Type | Categorical | Diamond, Cloverleaf, Trumpet etc. |
| Area Type | Area Type | Categorical | Urban/Rural |
| Ramp Direction | Ramp Direction | Categorical | Entry/Exit |
| Number of Lanes | Number of Lanes | Categorical | 1 or 2 |
| Lane Width | Lane Width | Continuous | In ft. |
| Outside Shoulder Width | Outside Shoulder Width | Continuous | In ft. |
| Inside Shoulder Width | Inside Shoulder Width | Continuous | In ft. |
| Ramp meter | Ramp Meter | Categorical | Y/N |
|  | Lighting | Categorical | Y/N |
| Grade |  | Continuous |  |

# Comprehensive Safety Analysis

This section presents a comprehensive safety analysis to qualitatively understand the benefit of providing roadway lighting.  Three sets of analyses were performed. They are:

- Crash Rate Analysis: compares crash rates for different segment types under lighted/unlighted conditions.

- Injury Severity Analysis: compares injury severity distributions for different crash categories.

- Crash Characteristics Analysis: compares crash characteristics for different crash categories.

To prepare the analysis, all crashes were classified into four crash categories with different lighting features. The four categories are:

- Nighttime crashes on lighted roadways;

- Nighttime crashes on unlighted roadways;

- Daytime crashes under low visibility conditions; and

- Other daytime crashes.

Specifically, the four crash categories were classified using the following criteria:

  - Daytime/Nighttime crashes: According to U.S. Naval Observatory Sunrise/Sunset data, aggregated by month and region;

  - Lighted Crashes/Unlighted Crashes: Based on lighting data manual review and 240 ft. light pole spacing; and

  - Daytime Low visibility crashes:

    - “Lighting Condition” marked as “Dark”, or

    - “Low visibility” Flag on, or

    - “Weather Conditions” marked as fog

Three sets of analyses were designed to investigate the difference between the four crash categories, aimed at exploring factors that impact crash frequency and severity under lighted and unlighted conditions, and eventually identify general conditions under which lighting can make a difference in terms of reducing crash frequency and severity. Quantification of the difference is discussed in detail in Section 6 of the report.

## Analysis of Nighttime Crash Rates

The first analysis computed crash rates for each segment/ramp based on the number of nighttime crashes and segment/ramp information. This analysis consists of the following steps:

- Step 1: Compute crash rates in terms of nighttime crashes per 100 million vehicle-miles traveled;

- Step 2: Perform t-test to identify statistical difference between average crash rates for different crash categories; and

- Step 3: Test whether the crash rate on unlighted segments/ramps is significantly higher than on lighted segments/ramps.

### Nighttime Crash Rate Computation

The crash rate for each segment/ramp was computed using the following equation:

CR=N*100,000,000AADT*365*Y*L					(7)

Where,

CR	= Nighttime crash rate, expressed in terms of 100 million vehicle miles traveled;

N	= Total number of nighttime crashes within the study period;

AADT	= Average Annual Daily Traffic Volume, in veh/day;

Y 	= Number of years of crash data; and

L	= Length of roadway segment in miles.

The average nighttime crash rate was computed based on the weighted average of crash rate with respect to segment/ramp length as represented by the following equation:

AvgCR=CR1*L1+CR2*L2+…+CRN*LN				(8)

Where,

AvgCR		=Average Nighttime Crash Rate;

CRN		=Nighttime crash rate for the nth segment; and

LN		=Length of the nth segment.

### Removing outliers

A nighttime crash rate boxplot for each segment type is shown in Figure 14. Note that the crash data range is larger compared to its concentration range, which indicates that outliers need to be removed before statistical analysis. An outlier is an observation that lies at an abnormal distance from other values in a random sample from a population. Although detection of outliers have various standards, one of the most popular methods is defined by the following rules:

- Mild Outlier Lower Bound: Q1 – 1.5*IQR;

- Mild Outlier Upper Bound: Q3 + 1.5*IQR;

- Extreme Outlier Lower Bound: Q1 – 3*IQR; and

- Extreme Outlier Upper Bound: Q3 + 3*IQR.

Where,

Q1	= Lower quartile of the data (25 percentile);

Q3 	= Upper quartile of the data (75 percentile); and

IQR 	= Inter-quartile range, defined as Q3 – Q1.

Figure 14 Crash Rate Boxplots for Each Segment Type

In this analysis, the criteria to detect extreme outliers were used as a conservative approach. Table 23 shows the calculated outlier upper and lower bounds for each dataset. Since the lower bound of the data is negative, only the upper bound was used to identify outliers in the data. A total of 12 freeway segments, 11 interchange segments and 73 ramps were identified as outliers and removed from the analysis.

Table 23 Crash Rate Outlier Threshold (Crash/100 Million VMT)

| Roadway Type | Q1 | Q3 | IQR | Upper Bound | Total Number of Segments | Number of Segments Removed |
| --- | --- | --- | --- | --- | --- | --- |
| Freeway Segments | 17.05 | 39.01 | 21.96 | 104.89 | 1229 | 12 |
| Interchange Segments | 35.95 | 75.93 | 39.98 | 195.87 | 519 | 11 |
| Ramps | 0.00 | 144.20 | 144.20 | 576.80 | 2150 | 73 |

### Summary of Nighttime Crash Rate

Table 24 shows a summary of crash rate data, classified by the segment type and the number of lanes after outliers being removed. The 2006 national crash rate was 196 crashes per 100 Million VMT. Note that the national data contains both daytime and nighttime crashes and accounts for all roadway types.

Table 24 Crash Rate Data Descriptive Statistics

| Roadway Type | Number of Lanes | Lighting | Sample Size | Average Crash Rate | Min Crash Rate | Max Crash Rate | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Freeway Segments | 4 | Lighted | 23 | 18.91 | 3.7 | 80.6 | 17.6 |
|  |  | Unlighted | 1031 | 31.36 | 2.0 | 103.4 | 18.3 |
|  | 6 | Lighted | 59 | 16.97 | 4.8 | 86.1 | 20.9 |
|  |  | Unlighted | 106 | 26.68 | 4.0 | 41.9 | 8.1 |
|  | 8 | Lighted | 10 | 36.56 | 10.9 | 87.0 | 22.5 |
|  | Overall | Lighted | 92 | 25.03 | 3.7 | 87.0 | 20.6 |
|  |  | Unlighted | 1137 | 30.20 | 2.0 | 103.4 | 18.1 |
| Interchange Segments | 4 | Lighted | 24 | 47.20 | 17.2 | 173.3 | 35.3 |
|  |  | Unlighted | 354 | 61.14 | 3.3 | 174.8 | 33.4 |
|  | 6 | Lighted | 63 | 36.67 | 12.3 | 141.6 | 28.8 |
|  |  | Unlighted | 72 | 55.09 | 10.2 | 84.5 | 16.9 |
|  | 8 | Lighted | 6 | 65.95 | 21.8 | 121.8 | 32.3 |
|  | Overall | Lighted | 93 | 53.58 | 12.3 | 174.8 | 32.8 |
|  |  | Unlighted | 426 | 57.25 | 3.3 | 173.3 | 31.1 |
| Ramps | 1 | Lighted | 300 | 98.12 | 0 | 2473.2 | 237.6 |
|  |  | Unlighted | 1446 | 141.58 | 0 | 4179.6 | 257.4 |
|  | 2 | Lighted | 148 | 89.89 | 0 | 952.8 | 166.5 |
|  |  | Unlighted | 219 | 110.62 | 0 | 1829.8 | 180.4 |
|  | 3 | Lighted | 25 | 69.48 | 0 | 284.6 | 90.3 |
|  |  | Unlighted | 12 | 116.41 | 0 | 224.4 | 89.9 |
|  | Overall | Lighted | 473 | 96.59 | 0 | 2473.2 | 212.7 |
|  |  | Unlighted | 1677 | 129.91 | 0 | 4179.6 | 247.8 |

Note1: ALL 8-lane freeway segments and interchange segments are lighted.

Note2: Crash Rate in 100 Million VMT.

### Nighttime Crash Rate Comparison

The Kolmogorov–Smirnov test (K-S test) is used to test whether two empirical probability distributions were significantly different, in this research whether crash rates on lighted/unlighted segments have significant different distributions. The test statistics is presented by the following equation.

Dn,n'=sup|F1,nx-F2,n(x)|				(9)

Where,

F1,n	= Empirical distribution function for the first sample;

F2,n	= Empirical distribution function for the second sample; and

sup	= Supremum function.

D = 0.1785

p = .0110*

* Denotes statistically significant result.

Figure 15 Freeway Segments Crash Rate Comparison (Outlier Removed)

D = 0.0737

p = .8155

* Denotes statistically significant result.

Figure 16 Interchange Segments Crash Rate Comparison (Outlier Removed)

D = 0.2644

p = <.0001*

* Denotes statistically significant result.

Figure 17 Interchange Ramps Crash Rate Comparison (Outlier Removed)

Figures 15 through 17 illustrate the crash rate probability density distributions for each segment type and show the K-S test results for each pair. Results are summarized as follows:

- Nighttime crash rate distributions on lighted and unlighted freeway segments are significantly different (p-value <.05);

- Nighttime crash rate distributions on lighted and unlighted interchange ramps are significantly different (p-value <.05); and

- Nighttime crash rate distributions on lighted and unlighted interchange segments do not show significant difference (p-value =.855).

The two-sample t-test is used to determine whether two population means are equal; in this research, it determined whether the mean crash rates for lighted segments and unlighted segments are equal. Because the mean crash rate was computed as a weighted average, a weighted t-test was used. Weighted t-tests produce two-sample t-tests comparing weighted data one another. They were used to compare the statistical significance of the difference between two weighted average crash rates. The test statistics are presented by the following equation.

T=Y1-Y2s12N1+s22N2							(10)

Where,

Y1,Y2		= Sample mean for sample 1 and sample 2;

s1,s2		= Standard deviation for sample 1 and sample 2; and

N1,N2		= Sample size for sample 1 and sample 2.

Table 25 Weighted T-Test Result for Mean Crash Rates

| Roadway Type | Number of Lanes | t-value | p-value |
| --- | --- | --- | --- |
| Freeway Segments | 4 | 3.3791 | .0037 * |
|  | 6 | 3.1618 | .0029 * |
|  | Overall | 2.1427 | .0310 * |
| Interchange Segments | 4 | 1.5701 | .1414 |
|  | 6 | 2.7092 | .0109 * |
|  | Overall | 0.6892 | .4937 |
| Ramps | 1 | 2.6634 | .0095 * |
|  | 2 | 0.8818 | .3821 |
|  | 3 | 0.7246 | .4926 |
|  | Overall | 2.7791 | .0062* |

Note: Cut off value for p-value: 0.05

* Denotes Statistically Significant

The results of weighted t-tests are presented in Table 25. Results indicate that:

- For freeway segments, unlighted segments have significantly higher crash rates than lighted segments;

- For interchange segments, crash rate on unlighted segments is only significantly higher for 6-lane segments; and

- For ramps, the overall crash rates on lighted and unlighted ramps shows statistically significant differences. However, for 2-lane and 3-lane ramps, the difference in crash rates is not statistically significant under lighted and unlighted conditions.

## Injury Severity Distribution Comparison

The analysis of injury severity distribution provides investigation into whether providing lighting can significantly reduce crash severity. The procedure of this analysis consists of the following steps:

- Step 1: Compute and compare injury severity distribution among all crash categories;

- Step 2: Perform proportion test to compare the statistical significance of difference in proportions of severe injury crashes (K+A), fatal crashes (K) and injury crashes; and

- Step 3: Test if nighttime unlighted segments or ramps have significantly more severe crashes than lighted segments or ramps.

Table 26 summarizes the injury severity distribution for different crash categories.

Table 26 Injury Severity Distribution for Different Crash Categories

| Crash Category | PDO | C | B | A | Injury | K | K+A | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Daytime All | 35216 | 7725 | 4113 | 1216 | 13054 | 188 | 1404 | 48458 |
|  | 72.7% | 15.9% | 8.5% | 2.5% | 26.9% | 0.4% | 2.9% | 100% |
| Daytime Low Visibility | 564 | 123 | 91 | 36 | 250 | 8 | 44 | 822 |
|  | 68.6% | 15.0% | 11.1% | 4.4% | 30.4% | 1.0% | 5.4% | 100% |
| Nighttime Lighted | 5448 | 1310 | 557 | 140 | 2007 | 18 | 158 | 7473 |
|  | 72.9% | 17.5% | 7.5% | 1.9% | 26.9% | 0.2% | 2.1% | 100% |
| Nighttime Unlighted | 17512 | 5019 | 1680 | 495 | 7194 | 111 | 606 | 24817 |
|  | 70.6% | 20.2% | 6.8% | 2.0% | 29.0% | 0.4% | 2.4% | 100% |

Figure 18 Injury Severity Distribution for Different Crash Groups

As a comparison, lighted and unlighted rural intersection injury severity data in Minnesota is shown in the following table.

Table 27 Injury Severity Distribution for Midwest States (11)

| Crash Category | Fatal | Injury | Property Damage Only | Total |
| --- | --- | --- | --- | --- |
| Lighted Rural Intersections | 3 | 88 | 150 | 241 |
|  | 1.2% | 36.5% | 62.2% | 100% |
| Unlighted Rural Intersections | 36 | 740 | 1465 | 2241 |
|  | 1.6% | 33.0% | 65.4% | 100% |

A proportion test is used to determine whether the difference between two proportions is statistically significant. The test uses the following test statistics:

Pooled sample proportion: p=p1*n1+p2*n2n1+n2				(11)

Standard error: SE=p*1-p*1n1+1n2 			(12)

Test statistic: z=p1-p2SE				                        (13)

Table 28 Proportion Test Result with Respect to Fatal + Incapacitating Injury Crashes (K+A)

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Nighttime Lighted |
| Daytime Low Visibility | 17.1 (<.0001 *) | - | - |
| Nighttime Lighted | 14.6 (<.0001 *) | 32.7 (<.0001 *) | - |
| Nighttime Unlighted | 12.7 (.0004 *) | 27.3 (<.0001 *) | 2.67 (.1118 ) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

Table 29 Proportion Test Result with Respect to Fatal Crashes (K)

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Nighttime Lighted |
| Daytime Low Visibility | 5.6 (.0181 *) | - | - |
| Nighttime Lighted | 3.4 (.0641) | 10.5 (.0012 *) | - |
| Nighttime Unlighted | 1.3 (.2582) | 3.7 (.0546) | 5.6 (.0175 *) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

Table 30  Proportion Test Result with Respect to Fatal + Injury Crashes (K+A+B+C)

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Nighttime Lighted |
| Daytime Low Visibility | 6.5 (.0101 *) | - | - |
| Nighttime Lighted | 0.16 (.6891) | 6.6 (.0101 *) | - |
| Nighttime Unlighted | 36.1 (<.0001 *) | 1.4 (.2428) | 15.2 (<.0001 *) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

Tables 28 through 30 summarize the results for proportion tests with respect to K+A crashes, K crashes, and K+A+B+C crashes.

For the proportion of fatal + incapacitating injury crashes,

- Nighttime lighted and nighttime unlighted crashes do not show significant difference.

- All other crash pair comparisons show statistical significance at 0.05 level.

For the proportion of fatal crashes,

- Nighttime unlighted crashes have significantly more fatal crashes than nighttime lighted crashes.

- Comparison between daytime crashes and nighttime lighted crashes does not show significant difference.

- Comparison between daytime low visibility crashes and nighttime unlighted crashes does not show significant difference.

For the proportion of fatal + injury crashes,

- Nighttime unlighted crashes have significantly more fatal + injury crashes than nighttime lighted crashes.

- Daytime crashes and nighttime lighted crashes do not show significant difference.

- Daytime low-visibility crashes and nighttime unlighted crashes do not show significant difference.

## Crash Characteristics Comparisons

The objectives of crash characteristics comparisons are as follows:

- Identify unique characteristics for each crash category using daytime crashes as the benchmark condition;

- Identify factors that can lead to nighttime lighted and unlighted crashes.

The characteristics that were analyzed are summarized in Table 31.

Table 31 Variables for Characteristics Comparisons

| Type | Variable | Description |
| --- | --- | --- |
| Crash Characteristics | Month | Month of year |
|  | Crash type | Angle, Rear-end, etc. |
|  | Hour | Hour of day |
|  | Vehicle Damage | Minor, moderate, severe |
| Highway Characteristics | Location type | Urban/Rural |
|  | Traffic Way | Divided, undivided, w/ or w/o barrier |
|  | Roadway Geometry | Curve, hill |
|  | Speed Limit | In mph |
| Environment Characteristics | Roadway Conditions | Dry, snow, ice, wet |
|  | Weather Conditions | Clear, cloudy, snow, rain, fog |
| Driver Characteristics | Driver age |  |
|  | Driver gender | M/F |
|  | Driver contributing factors | - Too fast for conditions |
|  |  | - Failure to have control |
|  |  | - Inattentive driving |
|  |  | - Alcohol and drug |
|  |  | - Disregard traffic control |
|  |  | - Following too close |
|  |  | - Failure to yield |
|  |  | - Improper overtaking etc. |

A two proportion test was used for comparing characteristics among the four crash categories. The test statistics are represented by the following equations:

Pooled sample proportion: p=p1*n1+p2*n2n1+n2				(14)

Standard error: SE=p*1-p*1n1+1n2			(15)

Test statistic: z=p1-p2SE						(16)

### Month Distribution

This section analyzes the impact of different months on the safety performance of providing lighting. Table 32 and Figure 19 show the crash proportion distribution in different months of a year for the four crash categories. Month of year was further divided into three categories: winter months (November-March) and non-winter months according to the weather characteristics in Wisconsin. Proportion tests were used to compare the statistical difference in the proportion of winter month crashes among the four crash categories. Test results are summarized in Table 33. The following findings are identified:

- For nighttime crashes, the proportion of winter month crashes is significantly higher under lighted condition than unlighted condition.

- Nighttime unlighted and daytime low visibility crashes did not show statistically significant difference for the proportion of winter months crashes.

- For daytime crashes, the proportion of winter month crashes is significantly higher funder low visibility condition than other condition.

Table 32 Month Distribution for Different Crash Categories

| Month | Daytime All |  | Daytime Low Visibility |  | Nighttime Lighted |  | Nighttime Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Winter Months | 21300 | 43.9% | 531 | 64.6% | 5083 | 68.2% | 15268 | 61.8% |
| Non-winter Months | 27158 | 56.1% | 291 | 35.4% | 2375 | 31.8% | 9454 | 38.2% |
| Total | 48458 | 100.0% | 822 | 100.0% | 7458 | 100.0% | 24722 | 100.0% |

Figure 19 Month Distribution for Different Crash Categories

Table 33 Proportion Test Results with Respect to Winter Months

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 138 (<.0001 *) | - | - |
| Nighttime Lighted | 1517 (<.0001 *) | 4.12 (.0422 *) | - |
| Nighttime Unlighted | 2428 (<.0001 *) | 0.59 (.4397) | 60.7 (<.0001 *) |

Note1: Chi-square value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Hour Distribution

This section compares the hour distributions for the four crash categories. Table 34 and Figure 20 show the hour distributions. The following results were obtained from the analysis:

- Nighttime lighted and unlighted crashes demonstrate similar percentages during late nighttime (7PM – 4AM);

- The proportion of early morning crashes (4AM-7AM) is higher under unlighted condition than lighted condition; and

- The proportion of early nighttime crashes (4PM-7PM) is higher under lighted condition than unlighted condition.

Table 34 Hour Distribution for Different Crash Categories

| Hour of Day | Nighttime Lighted |  | Nighttime Unlighted |  |
| --- | --- | --- | --- | --- |
| 0 | 369 | 4.9% | 1256 | 5.1% |
| 1 | 353 | 4.7% | 1120 | 4.5% |
| 2 | 433 | 5.8% | 1133 | 4.6% |
| 3 | 276 | 3.7% | 1053 | 4.3% |
| 4 | 238 | 3.2% | 1352 | 5.5% |
| 5 | 345 | 4.6% | 1964 | 7.9% |
| 6 | 232 | 3.1% | 2224 | 9.0% |
| 7 | 6 | 0.1% | 765 | 3.1% |
| 16 | 185 | 2.5% | 575 | 2.3% |
| 17 | 1095 | 14.7% | 1787 | 7.2% |
| 18 | 859 | 11.5% | 1746 | 7.1% |
| 19 | 569 | 7.6% | 1715 | 6.9% |
| 20 | 577 | 7.7% | 1931 | 7.8% |
| 21 | 734 | 9.8% | 2677 | 10.8% |
| 22 | 659 | 8.8% | 1883 | 7.6% |
| 23 | 528 | 7.1% | 1538 | 6.2% |
| Total | 7458 | 100.0% | 24719 | 100.0% |

Figure 20 Hour Distribution for Different Crash Categories

### Crash Type

This section compares the proportion of single-vehicle crashes among the four crash categories. Table 35 and Figure 21 show the crash type proportions for the four crash categories. Two crash types were used, namely single vehicle crashes (run-off-road crashes) and multi-vehicle crashes (all other crash types). Proportion tests were used to compare the statistical difference in the proportion of single vehicle crashes. Results are summarized in Table 36. Key findings are:

- For nighttime crashes, the proportion of single vehicle crashes is significantly higher under unlighted condition than lighted condition.

- Daytime and daytime low visibility crashes did not show statistically significant difference for the proportion of single vehicle crashes.

- Nighttime lighted and daytime low visibility crashes did not show statistically significant difference for the proportion of single vehicle crashes.

- This result can be explained by due to the lack of lighting during nighttime; drivers cannot clearly distinguish the alignment and marking of the roadways, causing higher risk of running-off-road.

- This finding implies that lighting can potentially reduce run-off-road crashes.

Table 35 Crash Type Distribution

| Crash Type | Daytime All |  | Daytime Low Visibility |  | Nighttime Lighted |  | Nighttime Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Vehicle Crashes | 20731 | 42.9% | 340 | 41.6% | 3076 | 41.3% | 19743 | 80.0% |
| Multi Vehicle Crashes | 27629 | 57.1% | 478 | 58.4% | 4368 | 58.7% | 4936 | 20.0% |
| Total | 48360 | 100.0% | 818 | 100.0% | 7444 | 100.0% | 24679 | 100.0% |

Figure 21 Crash Type Distribution

Table 36 Proportion Test Results with Respect to Single-Vehicle Crashes

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 0.51 (.4769) | - | - |
| Night Lighted | 6.24 (.0125 *) | 0.01 (0.9231) | - |
| Night Unlighted | 9116 (<.0001 *) | 697 (<.001 *) | 731 (<.001 *) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Vehicle Damage Type

This section compares the proportion of different vehicle damage types among the four categories. Vehicle damage types are classified into five categories, including none, very minor, minor, moderate, severe and very severe in the crash report. Table 37 and Figure 22 show the vehicle damage level distribution of the four crash categories. It can be observed that the vehicle damage levels are closely identical for each of the four crash categories.

Table 37 Vehicle Damage Type Distribution

| Vehicle Damage | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| None | 2012 | 4.4% | 26 | 3.3% | 184 | 2.6% | 412 | 2.0% |
| Some Damage | 32104 | 70.0% | 503 | 63.9% | 4442 | 64.0% | 13906 | 66.2% |
| Severe Damage | 11709 | 25.6% | 258 | 32.7% | 2321 | 33.5% | 6693 | 31.9% |
| Total | 45825 | 100.0% | 787 | 100.0% | 6947 | 100.0% | 21011 | 100.0% |

Figure 22 Vehicle Damage Type Distribution

### Median Barrier

This section compares the proportion of crashes on roadways with median barrier among the four crash categories. Table 38 and Figure 23 show the number of crashes on roadways with barriers and without barriers for the four crash categories. Daytime all crashes can be regarded as a baseline conditions in this analysis, with approximately half with median barriers and half without barriers.

The results for the two proportion test are listed in Table 39.

- Results showed that all pairs of crash categories have significantly different proportions of crashes that occurred on roadways with median barriers.

- Nighttime lighted crashes have the largest percentage of crashes on roadways with median barriers. This may be due to the fact that most lighted freeways are divided with median barriers.

Table 38 Median Barrier Distribution

| Median Barrier | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Barrier | 21790 | 55.8% | 250 | 34.7% | 5326 | 83.8% | 5945 | 41.7% |
| No Barrier | 17258 | 44.2% | 471 | 65.3% | 1026 | 16.2% | 8317 | 58.3% |
| Total | 39048 | 100.0% | 721 | 100.0% | 6352 | 100.0% | 14262 | 100.0% |

Figure 23 Median Barrier Distribution

Table 39 Proportion Test Result for Roadways with Median Barrier

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 305 (<.0001 *) | - | - |
| Night Lighted | 366 (<.0001 *) | 598 (<.0001 *) | - |
| Night Unlighted | 2361 (<.0001 *) | 18.9 (<.0001 *) | 2319 (<.0001 *) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Horizontal Curve

This section compares the proportion of crashes on horizontal curves for the four crash categories. Horizontal curves were divided into roadways with horizontal curves and without horizontal curves according to crash reports. Table 40 and Figure 24 show the number of crashes on roadways on a curve and/or hill. Table 41 show the results of proportion tests.

- For nighttime crashes, the proportion of horizontal curve crashes is significantly higher under unlighted condition than lighted condition.

- Daytime and daytime low visibility crashes did not show statistically significant difference for the proportion of horizontal curve crashes.

Table 40 Horizontal Curve Summary

| Horizontal Curve | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Curve | 6851 | 14.1% | 109 | 13.2% | 1205 | 16.2% | 2544 | 10.3% |
| No Curve | 41607 | 85.9% | 719 | 86.8% | 6268 | 83.8% | 22163 | 89.7% |
| Total | 48458 | 100.0% | 828 | 100.0% | 7473 | 100.0% | 24707 | 100.0% |

Figure 24 Horizontal Curve Summary

Table 41  Proportion Test Result for Horizontal Curves

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 0.55 (.4548) | - | - |
| Night Lighted | 20.5 (<.0001*) | 4.7 (.0301*) | - |
| Night Unlighted | 215 (<.0001*) | 6.77 (.009*) | 188 (<.0001*) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Vertical Curve

This section compares the proportion of crashes on vertical curves among the four crash categories. Horizontal curves were divided into roadways with vertical curves and without vertical curves according to crash reports. Table 42 and Figure 25 show the number of crashes on roadways on a curve and/or hill. Table 43 show the results of proportion tests.

- For nighttime crashes, the proportion of crashes on vertical curves is significantly higher under unlighted conditions than lighted conditions.

- All other crash pairs are also statistically significant with respect to the proportion of vertical curve crashes.

Table 42 Vertical Curve Summary

| Vertical Curve | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Curve | 8445 | 17.4% | 186 | 22.5% | 1103 | 14.8% | 3151 | 12.7% |
| No Curve | 40013 | 82.6% | 642 | 77.5% | 6370 | 85.2% | 21556 | 87.3% |
| Total | 48458 | 100.0% | 828 | 100.0% | 7473 | 100.0% | 24707 | 100.0% |

Figure 25 Vertical Curve Summary

Table 43 Proportion Test Result for Vertical Curves

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 4.937 (.0263*) | - | - |
| Night Lighted | 0.039 (.8424) | 4.8 (.0285*) | - |
| Night Unlighted | 420 (<.0001*) | 53.1 (<.0001*) | 156 (<.0001*) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Roadway Conditions

This section compares the proportion of crashes under adverse roadway conditions (ice, snow, or wet) among the four crash categories. Roadway conditions are classified as normal, ice, snow and wet in the crash reports. Table 44 and Figure 26 show the roadway conditions distributions for the four crash categories. Approximately 40 percent of daytime and nighttime unlighted crashes occurred under adverse roadway conditions (snow, ice, wet), while around 55 percent of nighttime lighted and 60 percent of daytime low visibility crashes occurred under adverse roadway conditions.

Proportion tests were used to compare the percentages of normal and adverse roadway conditions crashes among the four crash categories. The results are shown in Table 45.

- For nighttime crashes, the proportion of adverse roadway condition crashes is significantly higher under unlighted condition than lighted condition.

- Daytime and daytime low visibility crashes did not show a statistically significant difference for the proportion of adverse roadway condition crashes.

Table 44 Roadway Conditions Distribution

| Roadway Condition | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 31158 | 64.4% | 335 | 41.2% | 4133 | 55.5% | 15799 | 64.1% |
| Ice | 3469 | 7.2% | 78 | 9.6% | 312 | 4.2% | 2907 | 11.8% |
| Snow | 6951 | 14.4% | 165 | 20.3% | 1260 | 16.9% | 4219 | 17.1% |
| Wet | 6771 | 14.0% | 236 | 29.0% | 1740 | 23.4% | 1741 | 7.1% |
| Total | 48349 | 100.0% | 814 | 100.0% | 7445 | 100.0% | 24666 | 100.0% |

Figure 26 Roadway Conditions Distribution

Table 45  Proportion Test Result for Adverse Roadway Conditions

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 193 (<.0001*) | - | - |
| Night Lighted | 224 (<.0001*) | 62.5 (<.0001*) | - |
| Night Unlighted | 2.9 (.0905) | 177 (<.0001*) | 169 (<.0001*) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value=0.05

### Speed Limit

This section compares the proportion of crashes under different speed limits among the four crash categories. Table 46 and Figure 27 show the speed limit distributions for the four crash categories, which were classified into two categories: a speed limit equal to or smaller than 55 mph and a speed limit higher than 55 mph.

- For nighttime crashes, the proportion of <= 55 mph roadway crashes is significantly higher under lighted conditions than unlighted conditions.

Table 46 Speed Limit Distribution

| Speed Limit | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <= 55 | 18339 | 42.2% | 166 | 22.2% | 5503 | 86.4% | 3136 | 13.1% |
| >55 | 25076 | 57.7% | 581 | 77.8% | 870 | 13.7% | 20873 | 86.9% |
| Total | 43415 | 100.0% | 747 | 100.0% | 6373 | 100.0% | 24009 | 100.0% |

Figure 27 Speed Limit Distribution

### Weather Conditions

This section compares the proportion of non-clear,/cloudy weather conditions among the four crash categories. Weather conditions are classified as clear, cloudy, fog, rain, sleet, snow and windy in the crash reports. Table 47 and Figure 28 show the weather condition distributions for the four crash categories. Table 48 show the proportion tests with respect to non-clear/cloudy conditions (fog, rain, sleet, snow and windy).

- For nighttime crashes, the proportion of non-clear/cloudy crashes is significantly higher under unlighted conditions than lighted conditions.

- Statistical test results showed that all pairs are statistically significantly different in the proportion of non-clear/cloudy crashes, with daytime low visibility crashes having the largest percent of crashes under adverse weather conditions.

Table 47 Weather Conditions Distribution

| Weather Conditions | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clear, Cloudy | 35834 | 74.1% | 322 | 39.5% | 5002 | 67.1% | 17863 | 72.4% |
| Non Clear, Cloudy | 12525 | 25.8% | 493 | 60.5% | 2452 | 33.0% | 6801 | 27.5% |
| Total | 48359 | 100.0% | 815 | 100.0% | 7454 | 100.0% | 24664 | 100.0% |

Figure 28 Weather Conditions Distribution

Table 48  Proportion Test Result for Non-clear/cloudy Weather Conditions

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 491 (<.0001*) | - | - |
| Night Lighted | 160 (<.0001*) | 242 (<.0001*) | - |
| Night Unlighted | 23.4 (<.0001*) | 416 (<.0001*) | 78 (<.0001*) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value = 0.05

### Driver Age

This section compares the distribution of driver ages among the four crash categories. Figure 29 shows the driver age distributions for the four crash categories. A K-S test was conducted to test the statistical significance of difference between each distribution. As can be seen from the figure, young drivers (<25) account for a large percentage. Age distributions among the four crash categories did not show any significant difference using the K-S test (p-value > .05).

D = 1.0737

p = .1155

Figure 29 Driver Age Distribution

### Driver Gender

This section compares the proportion of male drives among the four crash categories. Table 49 and Figure 30 show the driver gender distributions for the four crash categories. Approximately 60 percent of involved drivers were male, and 40 percent were female.

- For nighttime crashes, the proportion of male drivers is not significantly different under unlighted conditions vs. lighted conditions.

- Statistical test results showed statistically significant difference for only two pairs: daytime vs. nighttime lighted and daytime vs. nighttime unlighted.

Table 49 Driver Gender Distribution

| Driver Gender | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Male | 28274 | 59.8% | 495 | 63.0% | 4412 | 63.4% | 15682 | 64.6% |
| Female | 19007 | 40.2% | 291 | 37.0% | 2543 | 36.6% | 8586 | 35.4% |
| Total | 47281 | 100.0% | 786 | 100.0% | 6955 | 100.0% | 24268 | 100.0% |

Figure 30 Driver Gender Distribution

Table 50 Proportion Test Result for Driver Gender

| Crash Category | Chi-square Value |  |  |
| --- | --- | --- | --- |
|  | Daytime | Daytime Low Visibility | Night Lighted |
| Daytime Low Visibility | 3.1 (.0775) | - | - |
| Night Lighted | 33.3 (<.0001*) | 0.05 (.8303) | - |
| Night Unlighted | 157 (<.0001*) | 0.83 (.3629) | 3.2 (.0714) |

Note1: Chi-square Value (p-value)

Note2: *Denote statistically significant

Note3: Two-tailed test

Note4: Cut off p-value= 0.05

### Driver Contributing Factors

This section compares the distribution of driver contributing factors for daytime and nighttime crashes. Table 51 and Figure 31 show the percentage of crashes with specific driver contributing factors for the four crash categories. Failure to have control, too fast for conditions, inattentive driving and following too close are the top driver contributing factors for all four crash categories. Specifically, daytime low visibility crashes have a significant larger number of crashes that are due to “too fast for conditions” and “failure to yield.” Both unlighted and lighted nighttime crashes show a larger percentage of impaired driving due to alcohol or drugs.

Table 51 Driver Contributing Factors

| Crash Category | Daytime All |  | Daytime Low Visibility |  | Night Lighted |  | Night Unlighted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Speeding | 743 | 2.2% | 13 | 2.3% | 266 | 4.5% | 368 | 3.3% |
| Too Fast For Conditions | 9637 | 29.0% | 257 | 46.2% | 1475 | 24.7% | 5256 | 46.4% |
| Failure To Yield | 2749 | 8.3% | 100 | 18.0% | 468 | 7.8% | 469 | 4.1% |
| Inattentive Driving | 10772 | 32.5% | 135 | 24.3% | 1603 | 26.9% | 2317 | 20.5% |
| Following Too Close | 8536 | 25.7% | 115 | 20.7% | 1160 | 19.4% | 1164 | 10.3% |
| Improper Turn | 700 | 2.1% | 11 | 2.0% | 96 | 1.6% | 111 | 1.0% |
| Left of Center | 149 | 0.5% | 4 | 0.7% | 19 | 0.3% | 58 | 0.5% |
| Improper Overtaking | 973 | 2.9% | 15 | 2.7% | 145 | 2.4% | 211 | 1.9% |
| Failure to Have Control | 11682 | 35.2% | 231 | 41.6% | 2079 | 34.9% | 5654 | 49.9% |
| Impaired Driving | 993 | 3.0% | 20 | 3.6% | 696 | 11.7% | 1228 | 10.8% |

Note: The percentage adding up can be more than 1 since multiple factors may contribute to a crash.

Figure 31 Driver Contributing Factors

## Summary

Key findings from the crash rate, injury severity and crash characteristics analyses are summarized as follows:

- For freeway segments, unlighted segments have significantly higher crash rates than lighted segments.

- For interchange segments, only the lighting effect on a 6-lane segment is statistically significant.

- For ramps, the overall comparison between lighted and unlighted ramp shows statistically significant difference. However, the difference between 2-lane and 3-lane ramps is not statistically significant.

- Night unlighted crashes had significantly more injury crashes than night lighted crashes.

- Daytime crashes and nighttime lighted crashes did not show a significant difference for injury crashes.

- Daytime low visibility crashes and nighttime unlighted crashes did not show a significant difference for injury crashes.

- For nighttime crashes, the proportion of winter month crashes is significantly higher under lighted conditions than unlighted conditions.

- For nighttime crashes, the proportion of single vehicle crashes is significantly higher under unlighted conditions than lighted conditions.

- For nighttime crashes, the proportion of horizontal curve crashes is significantly higher under unlighted conditions than lighted conditions.

- For nighttime crashes, the proportion of vertical curve crashes is significantly higher under unlighted conditions than lighted conditions.

- For nighttime crashes, the proportion of adverse roadway condition crashes is significantly higher under unlighted conditions than lighted conditions.

- For nighttime crashes, the proportion of non-clear/cloudy crashes is significantly higher under unlighted conditions than lighted conditions.

- For nighttime crashes, the proportion of male drivers is not significantly different under unlighted conditions with lighted conditions.

# Development of Nighttime Safety Performance Functions

## List of SPFs

Safety performance functions (SPFs) for nighttime crashes provide quantitative understanding for highway safety performances under unlighted and lighted conditions. An SPF predicts the crash frequency (crash/year) on a certain type of roadway segment or ramp. Table 52 lists all SPFs that were developed in this research. These SPFs cover different subtypes under the three basic roadway types: (1) freeway segments, (2) interchange segments, and (3) interchange ramps.

Table 52 List of SPFs Developed

| Roadway Type | SPFs | Sample Size |
| --- | --- | --- |
| Freeway Segments | Lighted a | 92 |
|  | 4-Lane Unlighted | 1031 |
|  | 6-Lane Unlighted | 106 |
| Interchange Segments | Lighted b | 93 |
|  | 4-Lane Unlighted | 356 |
|  | 6-Lane Unlighted | 70 |
| Interchange Ramps c | 1-Lane Lighted Entry | 144 |
|  | 1-Lane Lighted Exit | 156 |
|  | 1-Lane Unlighted Entry | 733 |
|  | 1-Lane Unlighted Exit | 713 |
|  | 2-Lane Lighted Entry | 84 |
|  | 2-Lane Lighted Exit | 64 |
|  | 2-Lane Unlighted Entry | 102 |
|  | 2-Lane Unlighted Exit | 117 |

a. SPF for Lighted Freeway Segments involves both 4-lane and 6-lane data due to the lack of samples.

b. SPF for Lighted Interchange Segments involves both 4-lane and 6-lane data due to the lack of samples.

c. SPFs for 3-Lane Ramps were not developed due to lack of sufficient samples.

Table 53 Potential Independent Variables Considered in SPFs for Interchange Ramps

| Potential Independent Variable | Variable Type | Descriptive Statistics for Continuous Data or Possible Values for Categorical Data |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Mean | Std. Dev |
| AADT (veh/day) | Continuous | 80 | 32300 | 3803 | 3695 |
| Length (mi) | Continuous | 0.06 | 1.39 | 0.26 | 0.12 |
| Lane Width (ft) | Continuous | 11 | 30 | 14.71 | 3.03 |
| Right Shoulder Width (ft) | Continuous | 2 | 16 | 8.06 | 2.01 |
| Left Shoulder Width (ft) | Continuous | 2 | 17 | 6.21 | 2.31 |
| Ramp Meter | Categorical | With/Without |  |  |  |
| Ramp Type | Categorical | Diamond, ParClo Loop, Outer Connection etc. |  |  |  |
| Interchange Type | Categorical | Diamond, Cloverleaf, Trumpet, ParClo etc. |  |  |  |
| Area Type | Categorical | Urban/Rural |  |  |  |
| Number of Lanes | Categorical | 1, 2 or 3 |  |  |  |

Tables 53 through 55 summarize all potential variables considered during the SPF development process for the three types of roadways. All these listed variables may have a significant impact on the nighttime (either unlighted or lighted or both) crash frequency. The significance of each of these potential variables was tested during the SPF development. Only significant variables were included in the final SPF.

Table 54 Potential Independent Variables Considered in SPFs for Freeway Segments

| Potential Independent Variable | Variable Type | Descriptive Statistics for Continuous Data or Possible Values for Categorical Data |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Mean | Std. Dev |
| AADT (veh/day) | Continuous | 4500 | 159000 | 31070 | 28992 |
| Length (mi) | Continuous | 0.17 | 1.83 | 1.19 | 0.34 |
| Lane Width (ft) | Continuous | 11 | 20 | 12.03 | 0.33 |
| Right Shoulder Width (ft) | Continuous | 4 | 15 | 10.22 | 1.76 |
| Left Shoulder Width (ft) | Continuous | 4 | 13 | 7.24 | 1.93 |
| Median Width (ft) | Continuous | 10 | 290 | 59.91 | 26.67 |
| Curve a | Continuous | 0 | 1.00 | 0.048 | 0.069 |
| Median Barrier Proportion | Continuous | 0 | 1 | 0.12 | 0.32 |
| International Roughness Index (mm/m) | Continuous | 0.501 | 3.891 | 1.704 | 0.626 |
| Median Barrier Type | Categorical | Cable, Concrete, Guardrail, No barrier |  |  |  |
| Median Type | Categorical | Grass, Paved, Wood, Other |  |  |  |
| Speed Limit (mph) | Discrete | 45, 50, 55, 60 or 65 |  |  |  |
| Number of Lanes | Categorical | 4 or 6 |  |  |  |
| Area Type | Categorical | Urban/Rural |  |  |  |
| Weaving Type | Categorical | Type A weaving, Type B weaving, No weaving |  |  |  |

- Curve =pCurve*1.47*V232.2*R, where pCurve is proportion of curves; V is speed; and R is radius.

Table 55 Potential Independent Variables Considered in SPFs for Interchange Segments

| Potential Independent Variable | Variable Type | Descriptive Statistics for Continuous Data or Possible Values for Categorical Data |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Mean | Std. Dev |
| AADT (veh/day) | Continuous | 4600 | 157000 | 43076 | 35865 |
| Length (mi) | Continuous | 0.11 | 1.21 | 0.48 | 0.17 |
| Lane Width (ft) a | Continuous | 12 | 20 | 12.07 | 0.56 |
| Right Shoulder Width (ft) | Continuous | 6 | 16 | 10.31 | 1.55 |
| Left Shoulder Width (ft) | Continuous | 4 | 14 | 7.35 | 2.08 |
| Median Width (ft) | Continuous | 10 | 600 | 58.59 | 46.49 |
| Curve b | Continuous | 0 | 2.259 | 0.058 | 0.152 |
| Median Barrier Proportion | Continuous | 0 | 1 | 0.25 | 0.42 |
| International Roughness Index (mm/m) | Continuous | 0.548 | 3.891 | 1.722 | 0.572 |
| Median Barrier Type | Categorical | Cable, Concrete, Guardrail, No barrier |  |  |  |
| Median Type | Categorical | Grass, Paved, Wood, Other |  |  |  |
| Speed Limit (mph) | Discrete | 45, 50, 55, 60 or 65 |  |  |  |
| Number of Lanes | Categorical | 4 or 6 |  |  |  |
| Area Type | Categorical | Urban/Rural |  |  |  |
| Interchange Type | Categorical | Diamond, Cloverleaf, Trumpet, ParClo etc. |  |  |  |

- Lane width was not considered in the SPF development due to lack of samples. Only 16 out of 519 samples have a lane width that is different than 12 ft.

- Curve =pCurve*1.47*V232.2*R, where pCurve is proportion of curves; V is speed; and R is radius.

## SPF Development and Identified Significant Factors

A generalized linear model (GLM) with a negative binomial distribution of crash frequency was used. Since crash is a rare occurrence on roadway segments and demonstrates significant over-dispersion, utilization of a negative binomial distribution can accurately model the crash frequencies (47). A negative binomial distribution is also commonly used in crash frequency modeling. The probability density function is:

(17)

Where,

μ	= Mean,

θ	= Shape parameter, and

Γ()	= Gamma function.

The statistical modeling process was completed using R by employing a maximum likelihood method (48). A base model was specified and the final crash prediction models were developed using the stepwise regression by implementing Akaike Information Criterion (AIC) as the model selection criterion. AIC is a measure of the relative goodness of fit of a statistical model. This measure describes the trade-off between the accuracy and complexity of the model, with generally smaller AIC representing better models. AIC can be computed using the following equation:

(18)

Where,

k =	Number of parameters in the statistical model; and

L =	Maximized value of the likelihood function for the estimated model.

An SPF model is represented in the following form:

SPFi=SPFBase i*∏CMFk						(19)

SPFBase i=L*eβ0*AADTβ1					(20)

Where,

SPFi		= Predicted nighttime crash frequency for SPF Model i (crashes/year);

SPFBase ii	= Base SPF with consideration of AADT and segment length only;

∏CMFk	= Product of identified CMFs that increase or reduce the SPF;

L		= Length of segment (mi);

AADT		= AADT of the segment (veh/day);

β0,β1		= Model calibration coefficients, and

The SPF model can be split into two parts: the base SPF part and the crash modification factors (CMF) part. The base part is represented by Equation (20), which only takes AADT and segment length into account when predicting the crash frequency. Any significant independent variables in the final SPF model were further converted to a CMF. A CMF increases or reduces the crash frequency predicted by the base SPF by multiplying the CMF to the base SPF.  A full SPF is the product of the base SPF and all CMFs. Note that the segment length and AADT are included in all SPF base part. The segment length has a fixed coefficient of 1 due to the convention which assumes that the number of crashes is directly proportional to segment length.

An important step in the SPF development was performing the negative binomial regression analysis. Significant independent variables of the SPF were determined in the regression analysis. Table 56 lists all significant variables in the final SPFs for freeway segments. Except for AADT, all the significant variables were later converted into CMFs.

Table 56 Significant Factors in the Final SPFs for Freeway Segments

|  | Variable | Estimate | SE | z-Value | p(>¦z¦) |
| --- | --- | --- | --- | --- | --- |
| SPF for Lighted Freeway Segments | ln(AADT) | 0.851 | 0.138 | 6.164 | <.001 |
|  | Number of Lanes | -0.228 | 0.212 | -2.955 | .003 |
|  | Left Shoulder Width | -0.057 | 0.031 | -5.101 | <.001 |
|  | Right Shoulder Width | -0.011 | 0.034 | -3.277 | .001 |
| SPF for Unlighted Four-Lane Freeway Segments | ln(AADT) | 0.748 | 0.034 | 14.085 | <.001 |
|  | Left Shoulder Width | -0.025 | 0.012 | -2.015 | .044 |
|  | Right Shoulder Width | -0.016 | 0.012 | -2.079 | .037 |
|  | Curve | 0.773 | 0.317 | 2.443 | .015 |
|  | Median Type (Wood) | -0.191 | 0.077 | -2.479 | .013 |
|  | Median Type (Paved) | 0.054 | 0.209 | 2.325 | .044 |
| SPF for Unlighted Six-Lane Freeway Segments | ln(AADT) | 0.748 | 0.127 | 5.894 | <.001 |
|  | Left Shoulder Width | -0.056 | 0.016 | -3.545 | <.001 |
|  | Right Shoulder Width | -0.022 | 0.015 | -1.476 | .039 |
|  | Median Width | -0.002 | 0.001 | -1.492 | .036 |
|  | Curve | 1.099 | 1.005 | 3.381 | <.001 |

Note: Segment length is not listed as it is always included in the base SPF with a coefficient of 1.0.

Table 57 summarizes all significant independent variables in the final SPFs for interchange segments. Except for AADT, all the significant variables were later converted into CMFs.

Table 57 Significant Factors in the Final SPFs for Interchange Segments

|  | Variable | Estimate | SE | z-Value | p(>¦z¦) |
| --- | --- | --- | --- | --- | --- |
| SPF for Lighted Interchange Segments | ln(AADT) | 0.704 | 0.111 | 8.143 | <.001 |
|  | Number of Lanes | -0.287 | 0.149 | -2.587 | .009 |
|  | Left Shoulder Width | -0.009 | 0.025 | -4.421 | <.001 |
|  | Right Shoulder Width | -0.027 | 0.034 | -1.402 | .061 |
|  | Median Width | -0.002 | 0.001 | -2.231 | .026 |
| SPF for Unlighted Four-Lane Interchange Segments | ln(AADT) | 0.619 | 0.071 | 8.440 | <.001 |
|  | Median Barrier Proportion | 0.235 | 0.115 | 2.033 | .042 |
|  | Area Type (Rural) | -0.101 | 0.075 | -1.350 | .098 |
|  | Median Width | -0.003 | 0.001 | -3.289 | .001 |
| SPF for Unlighted Six-Lane Interchange Segments | ln(AADT) | 0.669 | 0.194 | 1.955 | .050 |
|  | Median Barrier Proportion | 0.237 | 0.107 | 2.215 | .027 |
|  | Area Type (Rural) | -0.114 | 0.139 | -2.244 | .025 |
|  | Curve | 1.372 | 0.895 | 1.533 | .100 |

Note: Segment length is not listed as it is always included in the base SPF with a coefficient of 1.0.

Table 58 lists all significant independent variables in the final SPFs for interchange ramps. Except for AADT, all the significant variables were later converted into CMFs.

Table 58 Significant Factors in the Final SPFs for Interchange Ramps

|  | Variable | Estimate | SE | z-Value | p(>¦z¦) |
| --- | --- | --- | --- | --- | --- |
| SPF for One-Lane Lighted Entry Ramps | ln(AADT) | 0.832 | 0.118 | 7.052 | <.001 |
|  | Left Shoulder Width | -0.043 | 0.506 | -3.623 | <.001 |
|  | Right Shoulder Width | -0.016 | 0.054 | -2.151 | .031 |
| SPF for One-Lane Lighted Exit Ramps | ln(AADT) | 0.614 | 0.169 | 3.632 | <.001 |
|  | Left Shoulder Width | -0.044 | 0.049 | -2.912 | .005 |
|  | Right Shoulder Width | -0.049 | 0.046 | -3.212 | .001 |
| SPF for One-Lane Unlighted Entry Ramps | ln(AADT) | 0.784 | 0.246 | 4.369 | <.001 |
|  | Left Shoulder Width | -0.023 | 0.038 | -3.258 | .001 |
|  | Right Shoulder Width | -0.032 | 0.041 | -2.229 | .026 |
|  | Area Type (Rural) | -0.160 | 0.202 | -1.285 | .098 |
| SPF for One-Lane Unlighted Exit Ramps | ln(AADT) | 0.784 | 0.116 | 6.764 | <.001 |
|  | Area Type (Rural) | -0.155 | 0.207 | -1.235 | .022 |
|  | Left Shoulder Width | -0.044 | 0.040 | -2.082 | .037 |
|  | Ramp Type (Direct Connection) | -0.083 | 0.435 | -1.898 | .058 |
|  | Ramp Type (Free Flow Loop) | -0.095 | 0.489 | -1.948 | .051 |
| SPF for Two-Lane Lighted Entry Ramps | ln(AADT) | 0.696 | 0.206 | 3.377 | <.001 |
|  | Left Shoulder Width | -0.019 | 0.065 | -2.895 | .004 |
|  | Right Shoulder Width | -0.027 | 0.064 | -2.747 | .006 |
| SPF for Two-Lane Lighted Exit Ramps | ln(AADT) | 0.696 | 0.243 | 3.370 | <.001 |
|  | Right Shoulder Width | -0.030 | 0.066 | -1.979 | .048 |
|  | Ramp Type (Direct Connection) | -0.099 | 0.541 | -1.846 | .065 |
|  | Ramp Type (Semi-Direct Connection) | 0.150 | 0.540 | 2.776 | .006 |
| SPF for Two-Lane Unlighted Entry Ramps | ln(AADT) | 0.784 | 0.271 | 2.860 | .004 |
|  | Left Shoulder Width | -0.027 | 0.086 | -3.072 | .002 |
|  | Right Shoulder Width | -0.031 | 0.059 | -1.531 | .026 |
| SPF for Two-Lane Unlighted Exit Ramps | ln(AADT) | 0.774 | 0.326 | 2.414 | .016 |
|  | Left Shoulder Width | -0.013 | 0.075 | -1.501 | .033 |
|  | Ramp Type (Direct Connection) | -0.128 | 0.299 | -2.762 | .006 |

Note: Segment length is not listed as it is always included in the base SPF with a coefficient of 1.0.

## Developed SPFs (Base SPF with CMFs)

This section lists all base SPFs and their corresponding CMFs. All base SPFs predict crash frequency under the following base conditions. Base conditions are considered to be the most commonly observed conditions on roadways. Most numbers refer to those used in the NCHRP 17-45 final report. For any conditions that are different from the base condition, apply the corresponding CMFs. Applying CMFs increases or reduces the crash frequency predicted for the base condition.

Base conditions include:

- Lane width: 12 ft.;

- Left shoulder Width: 6 ft.; (4 ft. for ramp)

- Right shoulder Width: 6 ft.; (6 ft. for ramp)

- Median width: 48 ft.;

- No curve present;

- No median barrier present;

- Median type: Grass;

- Area type: Urban;

- Ramp type: Diamond;

- No ramp meter.

### SPF for Lighted Four-Lane Freeway Segments

SPFBase  Lighted4Fwy=L*AADT0.851*e-8.039						(21)

CMFLShldW=e-0.057LShldW-6 								(22)

CMFRShldW=e-0.011RShldW-10			 		      			(23)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Lighted Six-Lane Freeway Segments

SPFBase  Lighted6Fwy=L*AADT0.851*e-8.267						(24)

CMFLShldW=e-0.057LShldW-6 								(25)

CMFRShldW=e-0.011RShldW-10			 		      			(26)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Unlighted Four-Lane Freeway Segments

SPFBase  Unlighted4Fwy=L*AADT0.748*e-6.625						(27)

CMFLShldW=e-0.025LShldW-6								(28)

CMFRShldW=e-0.016RShldW-10								(29)

CMFCurve=e0.773*pCurve*1.47*V232.2*R								  (30)

CMFMedianType=e-0.191*MedianWood+0.054*MedianPaved=1.00  if grass median0.83  if wood median1.05  if paved median	  (31)

Under unlighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Increasing the proportion of horizontal curve will increase the number of nighttime crashes.

- Increasing the horizontal curve radius will reduce the number of nighttime crashes.

- Wooded medians will reduce the number of nighttime crashes, compared to grass medians.

- Paved medians will increase the number of nighttime crashes, compared to grass medians.

### SPF for Unlighted Six-Lane Freeway Segments

SPFBase  Unlighted6Fwy=L*AADT0.748*e-6.843						(32)

CMFLShldW=e-0.056LShldW-6								(33)

CMFRShldW=e-0.022RShldW-10								(34)

CMFMedW=e-0.002MedW-48								(35)

CMFCurve=e1.099*pCurve*1.47*V232.2*R								(36)

Under unlighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Increasing the median width will reduce the number of nighttime crashes.

- Increasing the proportion of horizontal curve will increase the number of nighttime crashes.

- Increasing the horizontal curve radius will reduce the number of nighttime crashes.

### SPF for Lighted Four-Lane Interchange Segments

SPFBase  Lighted4Int=L*AADT0.704*e-5.549						(37)

CMFLShldW=e-0.009LShldW-6								(38)

CMFRShldW=e-0.027RShldW-10								(39)

CMFMedW=e-0.002MedW-48								(40)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Increasing the median width will reduce the number of nighttime crashes.

### SPF for Lighted Six-Lane Interchange Segments

SPFBase  Lighted6Int=L*AADT0.704*e-5.757						(41)

CMFLShldW=e-0.009LShldW-6								(42)

CMFRShldW=e-0.027RShldW-10								(43)

CMFMedW=e-0.002MedW-48								(44)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Increasing the median width will reduce the number of nighttime crashes.

### SPF for Unlighted Four-Lane Interchange Segments

SPFBase  Unlighted4Int=L*AADT0.619*e-4.361						(45)

CMFBarrierProp=e0.235*BarrierProp								(46)

CMFAreaType=e-0.101*AreaType=1.00  if urban0.90  if rural					(47)

CMFMedW=e-0.003MedW-48								(48)

Under unlighted conditions:

- Increasing the proportion of median barriers will increase the number of nighttime crashes.

- Compared to urban areas, rural areas have fewer nighttime crashes.

- Increasing the median width will reduce the number of nighttime crashes.

### SPF for Unlighted Six-Lane Interchange Segments

SPFBase  Unlighted6Int=L*AADT0.669*e-5.081						(49)

CMFBarrierProp=e0.237*BarrierProp								(50)

CMFAreaType=e-0.114*AreaType=1.00  if urban0.89   if rural					(51)

CMFCurve=e1.372*pCurve*1.47*V232.2*R								(52)

Under unlighted conditions:

- Increasing the proportion of median barriers will increase the number of nighttime crashes.

- Compared to urban areas, rural areas have fewer nighttime crashes.

- Increasing the proportion of horizontal curve will increase the number of nighttime crashes.

- Increasing the horizontal curve radius will reduce the number of nighttime crashes.

### SPF for Lighted One-Lane Entry Ramps

SPFBase  Lighted1EntryRamp=L*AADT0.832*e-6.850					(53)

CMFLShldW=e-0.043LShldW-4								(54)

CMFRShldW=e-0.016RShldW-6								(55)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Lighted One-Lane Exit Ramps

SPFBase  Lighted1ExitRamp=L*AADT0.614*e-4.789					(56)

CMFLShldW=e-0.044LShldW-4								(57)

CMFRShldW=e-0.049RShldW-6								(58)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Unlighted One-Lane Entry Ramps

SPFBase  Unlighted1EntryRamp=L*AADT0.784*e-5.880					(59)

CMFLShldW=e-0.023LShldW-4								(60)

CMFRShldW=e-0.032RShldW-6								(61)

CMFAreaType=e-0.160*AreaType=1.00  if urban0.85   if rural					(62)

Under unlighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Compared to urban areas, rural areas have fewer nighttime crashes.

### SPF for Unlighted One-Lane Exit Ramps

SPFBase  Unlighted1ExitRamp=L*AADT0.784*e-6.081					(63)

CMFAreaType=e-0.155*AreaType=1.00  if urban0.86   if rural					(64)

CMFLShldW=e-0.044LShldW-4								(65)

CMFRampType=e-0.083*RampDirCon-0.095*RampFreeFlowLoop=1.00             if diamond0.92  if direct connect0.91  if free flow loop	(66)

Under unlighted conditions:

- Compared to urban areas, rural areas have fewer nighttime crashes.

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Direct connection ramps will reduce the number of nighttime crashes, compared to diamond ramps.

- Free flow loops will reduce the number of nighttime crashes, compared to diamond ramps.

### SPF for Lighted Two-Lane Entry Ramps

SPFBase Lighted2EntryRamp=L*AADT0.696*e-5.525					(67)

CMFLShldW=e-0.019LShldW-4								(68)

CMFRShldW=e-0.027RShldW-6								(69)

Under lighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Lighted Two-Lane Exit Ramps

SPFBase  Lighted2ExitRamp=L*AADT0.696*e-5.935					(70)

CMFRShldW=e-0.030RShldW-6								(71)

CMFRampType=e-0.099*RampDirCon+0.150*RampSemiDirCon=1.00             if diamond0.91  if direct connect1.16  if semi direct con	(72)

Under lighted conditions:

- Increasing the right shoulder width will reduce the number of nighttime crashes.

- Direct connection ramps will reduce the number of nighttime crashes, compared to diamond ramps.

- Semi direct connection ramps will increase the number of nighttime crashes, compared to diamond ramp.

### SPF for Unlighted Two-Lane Entry Ramps

SPFBase  Unlighted2EntryRamp=L*AADT0.778*e-6.081					(73)

CMFLShldW=e-0.027LShldW-4								(74)

CMFRShldW=e-0.031RShldW-6								(75)

Under unlighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Increasing the right shoulder width will reduce the number of nighttime crashes.

### SPF for Unlighted Two-Lane Exit Ramps

SPFBase  Unlighted2ExitRamp=L*AADT0.774*e-6.336					(76)

CMFLShldW=e-0.013LShldW-4								(77)

CMFRampType=e-0.128*RampDirCon=1.00                  if diamond0.88 if direct connection				(78)

Under unlighted conditions:

- Increasing the left shoulder width will reduce the number of nighttime crashes.

- Direct connection ramps will reduce the number of nighttime crashes, compared to diamond ramp.

## Comparisons between Base SPF Models

Base SPF models are SPFs matching the base conditions mentioned in Section 6.3. Base models have only length and AADT as variables. This section compares different base SPF models.

### Freeway Segments

Figure 32 compares the predicted crash frequency for lighted and unlighted four-lane/six-lane freeway segments. Length was fixed to 1 mile as the predicted nighttime crash frequency grows linearly with segment length according to the form of the base SPF model. According to Figure 32, six-lane lighted freeway segments have the least number of crashes under the same traffic volume condition. Under the same AADT and lighting conditions, six-lane segments have fewer nighttime crashes than four-lane segments largely due to less traffic in each travel lane.

Figure 32 Base Model Comparisons for Freeway Segments

### Interchange Segments

Figure 33 compares the predicted crash frequency for lighted and unlighted four-lane/six-lane interchange segments. Length was also fixed to 1 mile. According to Figure 33, six-lane lighted interchange segments have the least number of nighttime crashes under the same traffic volume condition. Under the same AADT and lighting conditions, six-lane segments have fewer nighttime crashes than four-lane segments, which is largely due to the less traffic in each travel lane.

Figure 33 Base Model Comparisons for Interchange Segments

### One-Lane Interchange Ramps

Figure 34 compares the predicted crash frequency for lighted and unlighted one-lane entry and exit ramps with respect to AADT. Results showed that under the same traffic volume condition, unlighted entry ramp has the largest number of nighttime crashes for all volume ranges. Both unlighted entry and exit ramps have a higher predicted number of nighttime crashes than lighted conditions. One-lane exit lighted ramps have higher predicted crash frequencies than entry ramps at low traffic volume conditions.

Figure 34 Base Model Comparisons for One-Lane Interchange Ramps

### Two-Lane Interchange Ramps

This section compares the predicted crash frequency for lighted and unlighted two-lane entry and exit ramps with respect to AADT. The comparison result is illustrated in Figure 35. Under the same traffic condition, unlighted entry ramp unlighted have the highest number of crashes for all AADT values. Both unlighted entry and exit ramps have a higher predicted number of nighttime crashes than lighted conditions. Entry ramps have a higher predicted number of nighttime crashes than exit ramps under the same lighting and traffic conditions.

Figure 35 Base Model Comparisons for Two-Lane Interchange Ramps

## Sensitivity Analysis for Continuous CMFs

Each SPF contains one or more CMFs for adjusting the predicted nighttime crash frequency from base conditions. The following subsections present the sensitivity analysis results for each SPF.

### CMFs for Lighted Four-Lane Freeway Segments

This section presents the analysis of sensitivity of CMFs for lighted four-lane freeway segments. The CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 36. Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 4.5 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 0.9 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 36 CMFs for Lighted Four-Lane Freeway Segments

### CMFs for Lighted Six-Lane Freeway Segments

This section presents the analysis of sensitivity of CMFs for lighted six-lane freeway segments. The CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 37. Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 4.5 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 0.9 percent.

| - Left Shoulder Width | - Right Shoulder Width |
| --- | --- |

Figure 37 CMFs for Lighted Six-Lane Freeway Segments

### CMFs for Unlighted Four-Lane Freeway Segments

This section presents the analysis of sensitivity of CMFs for unlighted four-lane freeway segments. The CMFs analyzed include left shoulder width, right shoulder width and curve. The analysis results are shown in Figure 38.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 2.1 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 1.5 percent.

- For a curve radius of 5000 ft, by increasing the proportion of a segment with a curve (increasing curve length) by 10 percent, the predicted number of nighttime crashes increases by approximately 0.6 percent.

- For a segment with 50 percent of its length on a curve, by increasing its curve radius by 1000 ft, the predicted number of nighttime crashes decreases by approximately 0.6 percent.

| - Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |
| (c) Curve |  |

Figure 38 CMFs for Unlighted Four-Lane Freeway Segments

### CMFs for Unlighted Six-Lane Freeway Segments

This section presents the analysis of sensitivity of CMFs for unlighted six-lane freeway segments. CMFs analyzed include left shoulder width, right shoulder width, median width and curve. The analysis results are shown in Figure 39. Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 4.5 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 2.0 percent.

- By increasing median width by 10 ft, the predicted number of nighttime crashes decreases by approximately 2.0 percent.

- For a curve radius of 5000 ft, by increasing the proportion of a segment with a curve (increasing curve length) by 10 percent, the predicted number of nighttime crashes increases by approximately 0.7 percent.

- For a segment with 50 percent of its length on a curve, by increasing its curve radius by 1000 ft, the predicted number of nighttime crashes decreases by approximately 0.8 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |
| (c) Median Width | (d) Curve |

Figure 39 CMFs for Unlighted Six-Lane Freeway Segments

### CMFs for Lighted Four-Lane Interchange Segments

This section presents the analysis of sensitivity of CMFs for lighted four-lane interchange segments. CMFs analyzed include left shoulder width, right shoulder width and median width. The analysis results are shown in Figure 40.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 0.7 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 4.6 percent.

- By increasing median width by 10 ft, the predicted number of nighttime crashes decreases by approximately 2.0 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |
| (c) Median Width |  |

Figure 40 CMFs for Lighted Four-Lane Interchange Segments

### CMFs for Lighted Six-Lane Interchange Segments

This section presents the analysis of sensitivity of CMFs for lighted six-lane interchange segments. CMFs analyzed include left shoulder width, right shoulder width and median width. The analysis results are shown in Figure 41.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 0.7 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes decreases by approximately 4.6 percent.

- By increasing median width by 10 ft, the predicted number of nighttime crashes decreases by approximately 2.0 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |
| (c) Median Width |  |

Figure 41 CMFs for Lighted Six-Lane Interchange Segments

### CMFs for Unlighted Four-Lane Interchange Segments

This section presents the analysis of sensitivity of CMFs for unlighted four-lane interchange segments. CMFs analyzed include median width and barrier proportion. The analysis results are shown in Figure 42.  Key findings include:

- By increasing median width by 10 ft, the predicted number of nighttime crashes increases by approximately 2.5 percent.

- By increasing the proportion of a segment with a median barrier by 10 percent, the predicted number of nighttime crashes increases by approximately 2.6 percent.

| (a) Median Width | (b) Barrier Proportion |
| --- | --- |

Figure 42 CMFs for Unlighted Four-Lane Interchange Segments

### CMFs for Unlighted Six-Lane Interchange Segments

This section presents the analysis of sensitivity of CMFs for unlighted six-lane interchange segments. CMFs analyzed include curve and barrier proportion. The analysis results are shown in Figure 43.  Key findings include:

- By increasing the proportion of a segment with a median barrier by 10 percent, the predicted number of nighttime crashes increases by approximately 2.6 percent.

- For a curve radius of 5000 ft, by increasing the proportion of a segment with a curve (increasing curve length) by 10 percent, the predicted number of nighttime crashes increases by approximately 0.9 percent.

- For a segment with 50 percent of its length on a curve, by increasing its curve radius by 1000 ft, the predicted number of nighttime crashes decreases by approximately 1.0 percent.

| (a) Curve | (b) Barrier Proportion |
| --- | --- |

Figure 43 CMFs for Unlighted Six-Lane Interchange Segments

### CMFs for Lighted One-Lane Entry Ramps

This section presents the analysis of sensitivity of CMFs for lighted one-lane entry ramps. CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 44.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 3.5 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 1.4 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 44 CMFs for Lighted One-Lane Entry Ramps

### CMFs for Lighted One-Lane Exit Ramps

This section presents the analysis of sensitivity of CMFs for lighted one-lane exit ramps. CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 45.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 3.5 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 4.8 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 45 Lane Width CMFs for Lighted One-Lane Exit Ramps

### CMFs for Unlighted One-Lane Entry Ramps

This section presents the analysis of sensitivity of CMFs for unlighted one-lane entry ramps. CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 46.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.1 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.7 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 46 CMFs for Unlighted One-Lane Entry Ramps

### CMFs for Unlighted One-Lane Exit Ramps

This section presents the analysis of sensitivity of CMFs for unlighted one-lane exit ramps. CMFs analyzed only include left shoulder width. The analysis results are shown in Figure 47.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 3.5 percent.

Figure 47 CMFs for Unlighted One-Lane Exit Ramps

### CMFs for Lighted Two-Lane Entry Ramps

This section presents the analysis of sensitivity of CMFs for lighted two-lane entry ramps. CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 48.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 1.6 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.7 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 48 CMFs for Lighted Two-Lane Entry Ramps

### CMFs for Lighted Two-Lane Exit Ramps

This section presents the analysis of sensitivity of CMFs for lighted two-lane exit ramps. CMFs analyzed only include right shoulder width. The analysis results are shown in Figure 49.  Key findings include:

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.7 percent.

Figure 49 CMFs for Lighted Two-Lane Exit Ramps

### CMFs for Unlighted Two-Lane Entry Ramps

This section presents the analysis of sensitivity of CMFs for unlighted two-lane entry ramps. CMFs analyzed include left shoulder width and right shoulder width. The analysis results are shown in Figure 50.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.1 percent.

- By increasing right shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 2.7 percent.

| (a) Left Shoulder Width | (b) Right Shoulder Width |
| --- | --- |

Figure 50 CMFs for Unlighted Two-Lane Entry Ramps

### CMFs for Unlighted Two-Lane Exit Ramps

This section presents the analysis of sensitivity of CMFs for unlighted two-lane exit ramps. CMFs analyzed only include left shoulder width. The analysis results are shown in Figure 51.  Key findings include:

- By increasing left shoulder width by 1 ft, the predicted number of nighttime crashes increases by approximately 0.9 percent.

Figure 51 CMFs for Unlighted Two-Lane Exit Ramps

## Injury Severity Distribution Model

A Severity Distribution Function (SDF) is a discrete choice model that includes variables describing a segment’s geometric features and traffic characteristics, and is used to predict the proportion of crashes associated with each of the following crash severity levels for each segment (43).

- K (Fatal);

- A (Incapacitating injury);

- B (Non-incapacitating injury);

- C (Possible injury), and

- PDO (Property-damage only).

The multinomial logistic model (MNL) was selected as the basis for SDF development. The MNL model was derived assuming that the error components are extreme value distributed (49). The probability of outcome j is defined by the following equation. The utility function in the model was used to relate the crash severity with the geometric design and traffic variables using a linear regression model.

(79)

Where,

Pj = probability of outcome j;

Vj = utility function of outcome j; and

J   = total number of possible outcomes modeled.

When applied towards the distribution of crash severity, the crash outcomes can be represented by four severity levels (K, A, B, C), with property damage only (PDO) crashes used as the base scenario. Due to the small number of fatal crashes within the dataset, fatal and incapacitating injury crashes were aggregated as a single severity level. Equations 80 through 83 represent how the distribution of severity levels was computed.

(80)

(81)

(82)

(83)

As a result of applying Equations 81 through 84, the predicted percentage of different crash severities was calculated and summarized in Tables 59 through 61 for each segment type. Ideally, one injury severity distribution table could be developed for each corresponding SPF. However, the large amount of ramps with zero nighttime crashes resulted in a small number of samples for each ramp category. Therefore, ramp injury severity distribution was only modeled by the number of lanes by combining entry and exit ramps.

Table 59 Injury Severity Distribution for Freeway Segments

| Type | K | A | B | C | PDO |
| --- | --- | --- | --- | --- | --- |
| Lighted | 0.2% | 2.1% | 11.6% | 10.6% | 75.5% |
| Unlighted 4-Lane | 0.4% | 2.3% | 12.5% | 12.0% | 72.8% |
| Unlighted 6-Lane | 0.5% | 2.2% | 12.1% | 12.6% | 72.6% |

Table 60 Injury Severity Distribution for Interchange Segments

| Type | K | A | B | C | PDO |
| --- | --- | --- | --- | --- | --- |
| Lighted | 0.3% | 1.4% | 12.9% | 10.6% | 76.8% |
| Unlighted 4-Lane | 0.4% | 2.5% | 11.6% | 13.1% | 72.4% |
| Unlighted 6-Lane | 0.3% | 2.3% | 12.2% | 13.3% | 71.9% |

Table 61 Injury Severity Distribution for Interchange Ramps

| Type | K | A | B | C | PDO |
| --- | --- | --- | --- | --- | --- |
| Lighted 1-Lane | 0.2% | 1.8% | 7.5% | 17.0% | 73.5% |
| Lighted 2-Lane | 0.2% | 1.9% | 8.5% | 15.6% | 73.8% |
| Unlighted 1-Lane | 0.3% | 2.2% | 11.2% | 15.1% | 71.2% |
| Unlighted 2-Lane | 0.3% | 2.0% | 11.50 | 15.4% | 70.8% |

# Lighting Safety Analysis Tools

Based on the safety performance functions, three tools have been developed to assist engineers in quantitatively understanding the lighting benefits when they make decisions on lighting installations and other geometric safety treatments. The tools can be categorized as two types, namely responsive tool and proactive tool.

- Responsive tools were developed to identify locations where lighting is needed and the benefit that can be obtained, including a set of unlighted segments crash risk and lighting benefit maps.

- Proactive tools were developed for use in the planning phase to assist decision making of installing lighting and other safety treatments, including a crash reduction look-up table and an excel spreadsheet tool.

## Proactive Tool: Nighttime Crash and Crash Reduction Look-Up Table

For freeway geometry, there exist “sweet spots” where a specific geometric feature is assigned a range of possible values. Designing out of the boundaries of sweet spots would be unacceptable or too large to be significant to roadway safety. Almost all lower and upper bound values can be sourced from AASHTO A Policy on Geometric Design of Highways and Streets (The Green Book). Although no specific upper bounds for a curve radius were defined in the Green Book, research papers have identified that a curve radius of more 10,000 ft had little impact on safety and thus would not be considered as a curve. The table below summarizes the range of sweet spots for the four continuous geometric features appearing in the nighttime crash SPFs.

Table 62 Sweet Spots Ranges for Geometric Features

|  | Urban Freeway |  | Rural Freeway |  | Ramp |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
| Left Shoulder Width (ft) | 4 | 14 | 4 | 14 | 1 | 10 |
| Right Shoulder Width (ft) | 4 | 14 | 4 | 14 | 1 | 10 |
| Median Width (ft) | 10 | 100 | 36 | 100 | N/A | N/A |
| Horizontal Curve Radius (ft) | 2000 | 10,000 | 2000 | 10,000 | N/A | N/A |

By applying the lower bounds of all the four geometric features, worst-case scenarios of nighttime crash frequency are computed and summarized in Table 9 and 10. Specifically, a worst-case scenario includes no lighting installed, 4 ft left and right shoulder width (1 ft for ramp), 10 ft median width for urban freeway, 36 ft median width for rural freeway, and 2000 ft curve radius and 100% curve on segment. Values of the remaining variables used in computing the worst-case scenarios are base conditions.

    - Lane width: 12 ft.;

    - Left shoulder Width: 4 ft.; (1 ft. for ramp)

    - Right shoulder Width: 4 ft.; (1 ft. for ramp)

    - Median width: 10 ft.;

    - Curve proportion: 100%;

    - Curve radius: 2000 ft.

    - No median barrier present;

    - Median type: Grass;

    - Area type: Urban;

    - Ramp type: Diamond;

    - No ramp meter.

Table 63 Worst Case Nighttime Crash Frequency (Freeway & Interchange Segment)

| Roadway Segment Type (Unlighted) | Predicted Nighttime Crash Frequency (crash/mile/year) |  |  |
| --- | --- | --- | --- |
|  | AADT = 20,000 | AADT = 50,000 | AADT = 80,000 |
| 4-Lane Freeway Segment | 2.82 | 5.61 | 7.97 |
| 6-Lane Freeway Segment | 2.83 | 5.62 | 7.98 |
| 4-Lane Interchange Segment | 6.57 | 11.59 | 15.51 |
| 6-Lane Interchange Segment | 5.03 | 9.94 | 13.61 |

Table 64 Worst Case Nighttime Crash Frequency (Interchange Ramp)

| Ramp Type (Unlighted) | Predicted Nighttime Crash Frequency (crash/mile/year) |  |  |
| --- | --- | --- | --- |
|  | AADT = 2,000 | AADT = 5,000 | AADT = 10,000 |
| 1-Lane Entry Ramp | 1.45 | 2.97 | 5.12 |
| 1-Lane Exit Ramp | 1.02 | 2.09 | 3.60 |
| 2-Lane Entry Ramp | 1.17 | 2.40 | 4.13 |
| 2-Lane Exit Ramp | 0.66 | 1.34 | 2.30 |

CMFs provide quantitative understandings of how different factors further increase or reduce the crash frequency. The significant factors have been identified as the following:

- Lane width;

- Left shoulder width;

- Right shoulder width;

- Median width;

- Horizontal curve proportion;

- Horizontal curve radius;

- Median type;

- Urban/rural;

- Median barrier proportion;

- Ramp meter;

- Ramp type;

These factors are potential treatments to reducing nighttime crashes. If considering that installing lighting is also a treatment, then the safety benefit of installing lighting can be quantitatively compared to other treatments such as:

- Increasing lane width;

- Increasing left shoulder width;

- Increasing right shoulder width;

- Increasing median width;

- Reducing horizontal curve length;

- Increasing horizontal curve radius;

- Changing median type;

- Installing ramp meter;

- Changing ramp type;

Tables 65 through 72 help quantitatively understand the benefit in terms of reduction in crash frequency by applying different treatments with/without installing lighting. Note that the per unit crash reduction values shown in all tables were computed based on average per unit crash reduction on a range of values.

Table 65 Reduction of Nighttime Crashes by Treatments (4-Lane Freeway Segment)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 20,000 | AADT = 50,000 | AADT = 80,000 |
| Install lighting only |  | -1.06 | -1.75 | -2.22 |
| Increase n ft Left Shoulder (Up to 14 ft) | w/o Installing Lighting | -0.06*n | -0.12*n | -0.17*n |
|  | w/ Installing Lighting | -1.06-0.07*n | -1.75-0.16*n | -2.22-0.25*n |
| Increase n ft Right Shoulder (Up to 14 ft) | w/o Installing Lighting | -0.04*n | -0.08*n | -0.12*n |
|  | w/ Installing Lighting | -1.06-0.03*n | -1.75-0.04*n | -2.22-0.06*n |
| Increase n*1000-ft Curve Radius (Up to 10000 ft) | w/o Installing Lighting | -0.03*n | -0.03*n | -0.01*n |
|  | w/ Installing Lighting | -1.06 | -1.75 | -2.22 |
| Reduce n*10% Curves 1 | w/o Installing Lighting | -0.03*n | -0.06*n | -0.01*n |
|  | w/ Installing Lighting | -1.06 | -1.75 | -2.22 |
| Convert to Wood Median | w/o Installing Lighting | -0.48 | -0.94 | -1.36 |
|  | w/ Installing Lighting | -1.06 | -1.75 | -2.22 |
| Convert to Paved Median | w/o Installing Lighting | +0.15 | +0.28 | +0.40 |
|  | w/ Installing Lighting | -1.06 | -1.75 | -2.22 |

Table 66 Reduction of Nighttime Crashes by Treatments (6-Lane Freeway Segment)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 20,000 | AADT = 50,000 | AADT = 80,000 |
| Install lighting only |  | -1.42 | -2.55 | -3.41 |
| Increase n ft Left Shoulder (Up to 14 ft) | w/o Installing Lighting | -0.10*n | -0.22*n | -0.30*n |
|  | w/ Installing Lighting | -1.42-0.06*n | -2.55-0.14*n | -3.41-0.25*n |
| Increase n ft Right Shoulder (Up to 14 ft) | w/o Installing Lighting | -0.05*n | -0.07*n | -0.15*n |
|  | w/ Installing Lighting | -1.42-0.02*n | -2.55-0.03*n | -3.41-0.05*n |
| Increase n*10 ft Median Width (Up to 100 ft) | w/o Installing Lighting | -0.04*n | -0.08*n | -0.12*n |
|  | w/ Installing Lighting | -1.42 | -2.55 | -3.41 |
| Increase n*1000-ft Curve Radius (Up to 10000 ft) | w/o Installing Lighting | -0.04*n | -0.08*n | -0.10*n |
|  | w/ Installing Lighting | -1.42 | -2.55 | -3.41 |
| Decrease n*10% Curve | w/o Installing Lighting | -0.03*n | -0.07*n | -0.12*n |
|  | w/ Installing Lighting | -1.42 | -2.55 | -3.41 |

Table 67 Reduction of Nighttime Crashes by Treatments (4-Lane Interchange Segment)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 20,000 | AADT = 50,000 | AADT = 80,000 |
| Install lighting only |  | -1.21 (-0.56)* | -1.38 (-0.82) | -1.28 (-1.25) |
| Increase n ft Left Shoulder (Up to 14 ft) | w/o Installing Lighting | -0(-0) | -0(-0) | -0(-0) |
|  | w/ Installing Lighting | -1.21-0.05*n (-0.56-0.05*n) | -1.38-0.09*n (-0.82-0.09*n) | -1.28-0.12*n (-1.25-0.12*n) |
| Increase n ft Right Shoulder (Up to 14 ft) | w/o Installing Lighting | -0 (-0) | -0 (-0) | -0 (-0) |
|  | w/ Installing Lighting | -1.21-0.13*n (-0.56-0.13*n) | -1.38-0.24*n (-0.82-0.24*n) | -1.28-0.34*n (-1.25-0.34*n) |
| Increase n*10 ft Median Width (Up to 100 ft) | w/o Installing Lighting | -0.16*n (-0.16*n) | -0.27*n (-0.27*n) | -0.37*n (-0.37*n) |
|  | w/ Installing Lighting | -1.21-0.05*n (-0.56-0.05*n) | -1.38-0.17*n (-0.82-0.17*n) | -1.28-0.24*n (-1.25-0.24*n) |
| Reduce n*10% Median Barrier | w/o Installing Lighting | -0.10*n (-0.10*n) | -0.30*n (-0.30*n) | -0.40*n (-0.40*n) |
|  | w/ Installing Lighting | -1.21 (-0.56) | -1.38 (-0.82) | -1.28 (-1.25) |

* Numbers in parentheses show nighttime crash change in rural area.

Table 68 Reduction of Nighttime Crashes by Treatments (6-Lane Interchange Segment)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 20,000 | AADT = 50,000 | AADT = 80,000 |
| Install lighting only |  | -1.03 (-0.71)* | -1.64 (-1.05) | -2.06 (-1.25) |
| Increase n ft Left Shoulder (Up to 14 ft) | w/o Installing Lighting | -0 (-0) | -0 (-0) | -0 (-0) |
|  | w/ Installing Lighting | -1.03-0.04*n (-0.71-0.04*n) | -1.64-0.07*n (-1.05-0.07*n) | -2.06-0.10*n (-1.25-0.10*n) |
| Increase n ft Right Shoulder (Up to 14 ft) | w/o Installing Lighting | -0 (-0) | -0 (-0) | -0 (-0) |
|  | w/ Installing Lighting | -1.03-0.10*n (-0.71-0.10*n) | -1.64-0.19*n (-1.05-0.19*n) | -2.06-0.27*n (-1.25-0.27*n) |
| Increase n*10 ft Median Width (Up to 100 ft) | w/o Installing Lighting | -0 (-0) | -0 (-0) | -0 (-0) |
|  | w/ Installing Lighting | -1.03-0.07*n (-0.71-0.07*n) | -1.64-0.15*n (-1.05-0.15*n) | -2.06-0.19*n (-1.25-0.19*n) |
| Reduce n*10% Median Barrier | w/o Installing Lighting | -0.15*n (-0.15*n) | -0.28*n (-0.28*n) | -0.38*n (-0.38*n) |
|  | w/ Installing Lighting | -1.03 (-0.71) | -1.64 (-1.05) | -2.06 (-1.25) |
| Increase n*1000-ft Curve Radius (Up to 10000 ft) | w/o Installing Lighting | -0.08*n (-0.08*n) | -0.15*n (-0.15*n) | -0.20*n (-0.20*n) |
|  | w/ Installing Lighting | -1.03 (-0.71) | -1.64 (-1.05) | -2.06 (-1.25) |
| Decrease n*10% Curve | w/o Installing Lighting | -0.10*n (-0.02*n) | -0.19*n (-0.19*n) | -0.26*n (-0.26*n) |
|  | w/ Installing Lighting | -1.03 (-0.71) | -1.64 (-1.05) | -2.06 (-1.25) |

* Numbers in parentheses show nighttime crash change in rural area.

Table 69 Reduction of Nighttime Crashes by Treatments (1-Lane Entry Ramp)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 2,000 | AADT = 5,000 | AADT = 10,000 |
| Install lighting only |  | -0.72 (-0.51) * | -1.41 (-0.98) | -2.34 (-1.60) |
| Increase n ft Left Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.05*n(-0.05*n) | -0.10*n (-0.10*n) | -0.17*n (-0.17*n) |
|  | w/ Installing Lighting | -0.72-0.02*n (-0.51-0.02*n) | -1.41-0.05*n (-0.98-0.05*n) | -2.34-0.09*n (-1.60-0.09*n) |
| Increase n ft Right Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.04*n (-0.04*n) | -0.07*n (-0.07*n) | -0.13*n (-0.13*n) |
|  | w/ Installing Lighting | -0.72-0.01*n (-0.51-0.01*n) | -1.41-0.02*n (-0.98-0.02*n) | -2.34-0.04*n (-1.60-0.04*n) |

* Numbers in parentheses show nighttime crash change in rural area.

Table 70 Reduction of Nighttime Crashes by Treatments (1-Lane Exit Ramp)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 2,000 | AADT = 5,000 | AADT = 10,000 |
| Install lighting only |  | -0.00 (-0.00)* | -0.30 (-0.00) | -0.86 (-0.33) |
| Increase n ft Left Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.04*n (-0.04*n) | -0.08*n (-0.08*n) | -0.13*n (-0.13*n) |
|  | w/ Installing Lighting | -0.03*n (-0.03*n) | -0.30-0.06*n (-0.06*n) | -0.86-0.17*n (-0.33-0.17*n) |
| Increase n ft Right Shoulder (Up to 10 ft) | w/o Installing Lighting | -0 (-0) | -0 (-0) | -0 (-0) |
|  | w/ Installing Lighting | -0.02*n (-0.02*n) | -0.30-0.04*n (-0.04*n) | -0.86-0.07*n (-0.33-0.07*n) |
| Convert to Direct Connection Ramp | w/o Installing Lighting | -0.09 (-0.09) | -0.16 (-0.16) | -0.23 (-0.23) |
|  | w/ Installing Lighting | -0.00 (-0.00) | -0.30 (-0.00) | -0.86 (-0.33) |
| Convert to Free Flow Loop | w/o Installing Lighting | -0.09 (-0.09) | -0.16 (-0.16) | -0.23 (-0.23) |
|  | w/ Installing Lighting | -0.00 (-0.00) | -0.30 (-0.00) | -0.86 (-0.33) |

* Numbers in parentheses show nighttime crash change in rural area.

Table 71 Reduction of Nighttime Crashes by Treatments (2-Lane Entry Ramp)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 2,000 | AADT = 5,000 | AADT = 10,000 |
| Install lighting only |  | -0.21 | -0.58 | -1.19 |
| Increase n ft Left Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.04*n | -0.07*n | -0.13*n |
|  | w/ Installing Lighting | -0.21-0.04*n | -0.58-0.07*n | -1.19-0.11*n |
| Increase n ft Right Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.03*n | -0.06*n | -0.10*n |
|  | w/ Installing Lighting | -0.21-0.02*n | -0.58-0.04*n | -1.19-0.07*n |

Table 72 Reduction of Nighttime Crashes by Treatments (2-Lane Exit Ramp)

| Treatment |  | Change in Nighttime Crash (crash/mile/year) |  |  |
| --- | --- | --- | --- | --- |
|  |  | AADT = 2,000 | AADT = 5,000 | AADT = 10,000 |
| Install lighting only |  | -0.10 | -0.28 | -0.58 |
| Increase n ft Left Shoulder (Up to 10 ft) | w/o Installing Lighting | -0.01*n | -0.01*n | -0.02*n |
|  | w/ Installing Lighting | -0.10 | -0.28 | -0.58 |
| Increase n ft Right Shoulder (Up to 10 ft) | w/o Installing Lighting | -0 | -0 | -0 |
|  | w/ Installing Lighting | -0.10-0.01*n | -0.28-0.01*n | -0.58-0.02*n |
| Convert to Direct Connection Ramp | w/o Installing Lighting | -0.05 | -0.08 | -0.13 |
|  | w/ Installing Lighting | -0.10-0.04 | -0.28-0.06 | -0.58-0.09 |
| Convert to Semi-Direct Connection | w/o Installing Lighting | -0 | -0 | -0 |
|  | w/ Installing Lighting | -0.10+0.05 | -0.28+0.08 | -0.58+0.13 |

Case Study: How to Use Tables 65 Through 72

Consider the case that we are to estimate the nighttime crash frequency for an unlighted 4-lane freeway segment and then evaluate different treatments, including installing lighting, to address the nighttime safety issue.

- Step 1: Check Table 63 to identify the base crash frequency for an urban freeway segment. Assuming the AADT is 50,000, the estimated base nighttime crash frequency is then 5.61 crash/mile/year.

  - Note that the base nighttime crash frequency is under the worst case scenario conditions.

    - Lane width: 12 ft.;

    - Left shoulder Width: 4 ft.;

    - Right shoulder Width: 4 ft.;

    - Median width: 10 ft.;

    - Curve proportion: 100%;

    - Curve radius: 2000 ft.

    - No median barrier present;

    - Median type: Grass;

    - Area type: Urban;

    - Ramp type: Diamond;

    - No ramp meter.

- Step 2: Check Table 65 to compare the safety benefits from various treatments.

  - For installing lighting only, without applying any other treatments, the nighttime crash frequency is expected to be reduced by 1.75 crash/mile/year. The final expected nighttime crash frequency is 5.61 - 1.75 = 3.86 crash/mile/year.

  - If considering increasing the left shoulder width by 2 ft, the expected crash frequency without installing lighting is 5.61 - 2*0.12 = 5.37 crash/mile/year. If considering installing lighting and increasing the left shoulder width by 2 ft, the expected crash frequency without installing lighting is 5.61 - 1.75 -2*0.16 = 3.54 crash/mile/year..

  - If considering increasing the right shoulder width by 1 ft, the expected crash frequency is 5.61 - 0.08 = 5.53 crash/mile/year. If considering installing lighting AND widening the right shoulder by 1 additional ft, the expected crash frequency is 5.61 – 1.75 - 0.04 = 3.82 crash/mile/year.

- Comparing lighting with other treatments, for example increasing 5-ft left shoulder width, the benefit of installing lighting is 1.75/(0.12*5) =  approximately 3 times greater.

## Proactive Tool: Excel Spreadsheet Tool

The proactive tool uses the form of an Excel spreadsheet tool for estimating crash benefit from lighting based on inputs of roadway data (geometric, AADT, and etc.). The tool is for planning purposes for considering lighting, and can be considered as an advanced version of the previous proactive tool. All equations used for computation in this proactive tool are based on the SPFs development results.

This tool is organized into a main page and four worksheets.

- Main Page;

- Worksheet 1: Computing expected nighttime crash frequency for freeway segments;

- Worksheet 2: Computing expected nighttime crash frequency for interchange segments;

- Worksheet 3: Computing expected nighttime crash frequency for interchange ramps; and

- Worksheet 4: Empirical Bayes analysis.

A screenshot of the main page is presented in Figure 52. The main page contains instructions for using the tool and serves as a starting point of the analysis. The user needs to choose one roadways segment type which corresponds to a specific worksheet.

Figure 52 Illustration of the Proactive Tool – Main Page

Worksheets 1 through 3 are used for calculating the predicted nighttime crash frequency for freeway segments, interchange segments and interchange ramps, respectively. The three worksheets are similarly laid out and include several blocks, as shown in Figure 53. The input blocks include general information, basic roadway information and geometric information. Note that this tool is able to compare the crash frequencies under two different conditions: “with lighting” and “without lighting” scenarios. The “with lighting” and “without lighting” scenarios can have the same or different geometric characteristics to provide more flexibilities for the users. As an output, the worksheet displays the predicted nighttime crash frequencies under both scenarios with the units of nighttime crashes/year. A figure will then be automatically be generated for comparing the “with lighting” and “without lighting” outcomes.

Worksheet 4 was designed as a separate worksheet for Empirical Bayes (EB) before-after safety improvement analysis. The EB method increases the precision of safety performance estimation and corrects for the regression-to-mean bias (47). The EB method uses both the predicted crash frequency calculated from the SPFs and the field-observed before/after crash frequencies. In order to use worksheet 4, at least one of the worksheet 1-3 must be filled with output calculated. This worksheet will automatically input the outputs from the corresponding worksheet (i.e., worksheet 1, 2, or 3) into the corresponding cell. Users need to input the before and after crash frequencies and the number of before and after observation years. The worksheet through calculations will show whether this treatment is effective and whether the before and after comparison is statistically significant. A screenshot of worksheet 4 can be found in Figure 54.

Figure 53 Illustration of the Proactive Tool – Analysis Worksheet

Figure 54 Illustration of the Proactive Tool – Empirical Bayes Analysis

## Responsive Tool: Unlighted Crash Risk and Lighting Benefit Maps

The objective of the responsive tool is to identify high benefit locations for lighting installation. This tool is presented in the form of 6 maps: 3 crash risk maps and 3 lighting benefit maps. These maps include:

- Map 1: Nighttime crash risk map for unlighted freeway segments;

- Map 2: Nighttime crash risk map for unlighted interchange segments;

- Map 3: Nighttime crash risk map for unlighted interchange ramps;

- Map 4: Lighting benefit map for unlighted freeway segments;

- Map 5: Lighting benefit map for unlighted interchange segments;

- Map 6: Lighting benefit map for unlighted interchange ramps;

The nighttime crash risk maps (Figures 55 through 57) use color to present the expected number of nighttime crash frequency on unit length (crashes/mile/year) for each unlighted segment/ramp. The nighttime crash frequency is computed by using the developed SPFs rather than using actual observed crash frequencies. These maps help visualize the existing nighttime crash risk for all unlighted STN segments in Wisconsin. Appendix C provides top 100 locations for each segment type that have the highest nighttime crash risk as predicted by the SPFs.

The lighting benefit maps represent the expected reduction in nighttime crash frequency on unit length (crashes/mile/year) if lighting is installed for each unlighted segment/ramp. The lighting benefit is computed by using the following equation:

Lighting Benefit=SPFUnlighted-SPFLighted			(84)

Note that for all maps, segments already lighted are not included; only currently unlighted segments are shown. These maps help identify of candidate locations for installing lighting, where the highest safety benefit can be obtained if lighting is installed. The maps are shown in Figures 58 through 60. Appendix C provides top 100 locations for each segment type that have the highest lighting benefit.

Map 1

Crash Risk Map for Unlighted

Freeway Segments

*Expressway segments 500 ft. away from at grade intersection included as freeway segments

Figure 55 Nighttime Crash Risk Map for Unlighted Freeway Segments

Figure 56 Nighttime Crash Risk Map for Unlighted Interchange Segments

Figure 57 Nighttime Crash Risk Map for Unlighted Interchange Ramps

Map 4

Lighting Benefit Map for Unlighted Freeway Segments

*Expressway segments 500 ft. away from at grade intersection included as freeway segments

Figure 58 Lighting Benefit Map for Unlighted Freeway Segments

Figure 59 Lighting Benefit Map for Unlighted Interchange Segments

Figure 60 Lighting Benefit Map for Unlighted Interchange Ramps

# Benefit Cost Analysis

## Crash Costs and Lighting Benefits

This section presents a cost-benefit study based on the crash and lighting cost data. Specifically, benefits were calculated as the difference in crash costs between the before-lighting and after-lighting scenarios. Crash costs were computed based on the unit costs currently being used by WisDOT which were adapted from the National Safety Council’s “Estimating the Costs of Unintentional Injuries” and are shown in Table 73. The benefits and costs of different lighting types were not separately analyzed in this section for the following reasons. First, the safety benefits of different types of lighting (LED vs. traditional) may be slightly different, which are not separately modeled in Section 6. Also, no lighting type data is available to identify the specific type used. Furthermore, the cost for installing and operating newer lighting types (e.g. LEDs) is not available.

Table 73 National Safety Council Crash Unit Costs

| Injury Severity | Unit Cost (2012 $) |
| --- | --- |
| Fatal | $4,538,000 |
| Incapacitating Injury (A) | $230,000 |
| Non-Incapacitating Injury (B) | $58,700 |
| Possible Injury (C) | $28,000 |
| Property Damage Only (PDO) | $2,500 |

Lighting benefits were calculated based on the difference between before-lighting crash costs and after lighting crash costs. The corresponding injury severity distributions for nighttime lighted and unlighted conditions were used. Nighttime crash reduction values were obtained using the developed SPF models and were organized in different scenarios in terms of traffic volume and segment types. All other geometry data were the base conditions. Appendix A includes an explanation of base conditions. The benefit analysis results are presented in Tables 74 and 75, with the following calculation criteria.

- 10-year crash reduction;

- 1-mile segment for each segment type; and

- Roadway geometry based on base scenario (most commonly seen).

The procedure for computing lighting benefits is shown in the following steps.

- Use lighted SPFs to predict crash frequency based on AADT and other geometric variables;

- Obtain corresponding lighted injury severity distribution;

- Calculate exact crash frequency for each injury severity under lighted condition; and

- Compute the total crash cost for lighted condition using the following equations.

TC= Ci*Ni						(85)

Where,

TC	= Total crash cost under lighted/unlighted conditions;

Ci	= Crash unit costs for a specific injury severity type; and

Ni	= Crash frequency for a specific injury severity type.

- Repeat steps 1 to 4 to obtain total crash cost for unlighted conditions;

- Compute the lighting benefit using the following equation;

BLighting=TCUnlighted-TCLighted 				(86)

Where,

BLighting	= Lighting benefit;

TCUnlighted	= Total crash cost for unlighted conditions; and

TCLighted	= Total crash cost for lighted conditions.

- For each AADT and segment type combination, a lighting benefit value is computed.

Table 74 Ten-Year Lighting Benefits for Freeway and Interchange Segments

| Segment Type | AADT=20,000 | AADT=50,000 | AADT=80,000 |
| --- | --- | --- | --- |
| 4-Lane Freeway Segment | 385,404 | 693,408 | 931,306 |
| 6-Lane Freeway Segment | 314,488 | 564,798 | 758,148 |
| 4-Lane Interchange Segment | 968,150 | 1,593,600 | 2,029,444 |
| 6-Lane Interchange Segment | 1,045,862 | 1,381,470 | 1,853,580 |

Table 75 Ten-Year Lighting Benefits for Interchange Ramps

| Segment Type | AADT=2,000 | AADT=5,000 | AADT=10,000 |
| --- | --- | --- | --- |
| 1-Lane Entry Ramp | 224,348 | 447,132 | 753,534 |
| 1-Lane Exit Ramp | 85,894 | 241,836 | 485,930 |
| 2-Lane Entry Ramp | 110,408 | 254,092 | 476,124 |
| 2-Lane Exit Ramp | 91,182 | 198,038 | 360,370 |

## Lighting Costs

Lighting costs were comprised of costs for multiple components pertaining to a lighting system. These costs include the installation cost in addition to the associated operation costs. Repair costs were not available and thus were not included in computing lighting costs. Ten-year lighting costs were computed as a basis for benefit/cost ratios using the following equations:

LightingCost5-Year=NumPolesPerMile*PerPoleCost5-Year 		(87)

Where,

PerPoleCost5-Year=Installation+10*Yearly Operation 			(88)

NumPolePerMile=5280/LightPoleSpacing 					(89)

The total lighting cost for a 1-mile segment can be computed using the per pole cost multiplying by the number of light poles needed per mile. Based upon roadway image and Texas Illumination Manual, a 240-ft light pole spacing was used, which corresponds to 5280/240 = 22 light poles per mile. This is also based on the observation that most freeway and interchange segments use light poles placed in medians with twin-luminaries which can cover both directions of travel, as shown in Figure 61, while ramp light poles are only placed on one side with a single luminary.

Figure 61 Freeway Median Light Poles with Twin Luminaires (Image Source: Wikipedia)

With the lack of comprehensive and complete lighting installation, as well as operation and repair costs data, the cost values used for computation are mainly based on a combination of assumptions, literature provided values and available WisDOT lighting cost data. Installation cost was sourced from 2010 Highway Construction Cost Estimation Manual, which is $9,500 per pole. Therefore, the total cost for installing 22 light poles on a 1-mile of freeway would be $209,000.

Yearly operational cost for a light pole is computed using the following equation:

YearOperation= Hours*Watt*EC*NumLuminaries 		(90)

Where,

Hours			=Number of operating hours per year per pole;

Watt			=Average power of light pole luminary;

EC			=Average cost of per kWh electricity; and

NumLuminaries	=Number of luminaries, 1 or 2.

Number of operating hours per year was estimated using U.S. Naval Observatory Sunrise/Sunset data, the same data used for separating daytime/nighttime crashes. The total number of operating hours is computed as 4,297 hours out of 8,760 total hours in a calendar year. The average power of a lighting pole luminary is 250 watts or 0.25 kW. The average cost per kWh electricity for Wisconsin transportation industry is estimated at $0.07 per kWh based on WisDOT data and U.S. Energy Information Administration data (48). The number of luminaries is determined to be 2 for freeway/interchange segments, and 1 for ramps. Thus the yearly energy cost for a light pole is computed to be $150 for freeway/interchange segments and $75 for ramps. 1-mile of lighting would cost $3,300 annually for freeway/interchange segments and $1,650 for ramps.

## Benefit/Cost Ratios

Benefit/Cost (B/C) ratios were computed on a 10-year basis by segment type, number of lanes, and traffic volume level. A benefit cost ratio greater than 1.0 signifies that the benefits outweigh the costs; a B/C ratio equal to 1.0 signifies that the benefits equal the costs; and a B/C ratio less than 1.0 signifies that the costs outweigh the benefits. Tables 76 and 77 present the B/C ratios for freeway and interchange segments and interchange ramps, respectively. Red cells represent less than 1.0 b/c ratio, the darker green color represent a high b/c ratio.

Table 76 Ten-Year B/C Ratio for Freeway and Interchange Segments

| Segment Type | AADT=20,000 | AADT=50,000 | AADT=80,000 |
| --- | --- | --- | --- |
| 4-Lane Freeway Segment | 1.59 | 2.87 | 3.85 |
| 6-Lane Freeway Segment | 1.30 | 2.33 | 3.13 |
| 4-Lane Interchange Segment | 4.00 | 6.59 | 8.39 |
| 6-Lane Interchange Segment | 4.32 | 5.71 | 7.66 |

Table 77 Ten-Year B/C Ratio for Interchange Ramps

| Segment Type | AADT=2,000 | AADT=5,000 | AADT=10,000 |
| --- | --- | --- | --- |
| 1-Lane Entry Ramp | 0.99 | 1.98 | 3.34 |
| 1-Lane Exit Ramp | 0.38 | 1.07 | 2.15 |
| 2-Lane Entry Ramp | 0.49 | 1.13 | 2.11 |
| 2-Lane Exit Ramp | 0.40 | 0.88 | 1.60 |

As shown in Tables 76 and 77, all of the B/C ratios for freeway and interchange segments are larger than 1.0. The highest B/C ratio occurs under both 4 and 6-lane interchange segments with high AADT, especially for interchange segments, which signifies the largest expected benefits compared with its costs.

For interchange ramps, most low and medium ramps scenarios have less than 1.0 B/C ratio which may imply its low cost effectiveness of installing lighting. However, when considering the service life of light poles exceeds a 10-year range, the benefit cost ratio will certainly be much larger in the long run.

# Lighting Safety Guidelines

Current lighting requirements in Wisconsin indicated that roundabout, signalized intersections and Milwaukee area freeways are always installed with roadway lighting. Other STN roadways are not lighted in many cases. Most freeway and interchanges are not lighted. Based on the analysis and modeling results from Chapter 6, this section proposes guidelines for installing lighting on STN freeways and interchanges.

Federal level warrants for freeway and interchange lighting can be found in the AASHTO Lighting Design Guide. It specifies warrant conditions for three types of freeway/interchange lighting: Complete freeway lighting (CFL), complete interchange lighting (CIL) and partial interchange lighting (PIL). However, after applying the warrant conditions to existing unlighted freeway in Wisconsin, this criterion seems to meet more than half of all freeways/interchanges, thus may not be appropriate to implement. As a reference, the type of AASHTO criteria met by each segment is included in Appendix C.

The Highway Safety Manual includes a CMF which quantifies the reduction in nighttime crashes through installation of roadway lighting. This CMF can be applied to all roadway types. However, that CMF is a single number. The SPFs based method developed through this project provides specific nighttime crash predictions with respect to different segment types. Additionally, the SPFs enable the comparison of safety benefits of lighting with other geometric improvements, which are reflected in the proactive spreadsheet tool.

The procedure for determining scenarios under which lighting installation is recommended is summarized as follows:

- First, for each segment type and number of lane combinations, a lighting benefit table was created using the SPF along with its corresponding CMFs results. The tables include all possible levels for categorical variables, and three different levels (low, medium, high) for continuous variables. Note that only AADT and one other variable are considered at a time, with other variables fixed at base conditions (See Appendix A for explanations). Lighting benefit was computed as the reduction of nighttime crashes through lighting.

- Second, the computed values were colored according to its values, with red showing smaller lighting benefits, and green representing larger benefits. In addition, the top 20 percent lighting benefits in each table were shown in bold red numbers. Scenarios with the top 20 percent lighting benefits are recommended to install lighting. The following subsections present the lighting benefit tables along with its recommended lighting installation scenarios.

## Freeway Segments

### Four-Lane Freeway Segments

For four-lane freeway/segments, lighting is recommended under the following scenarios:

- All conditions with AADT > 90,000;

- AADT > 60,000 with 4-ft left shoulder;

- AADT > 80,000 with 4-ft right shoulder;

- AADT > 65,000 with 3000-ft radius curve;

- AADT > 75,000 with 6000-ft radius curve;

- AADT > 60,000 with paved median.

Table 78 Lighting Benefit for Four-Lane Freeway Segments

| 4-Lane | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Curve |  |  | Median Type |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | No | 50%/3000ft | 50%/6000ft | Grass | Wood | Paved |
| 10,000 | 0.53 | 0.48 | 0.45 | 0.51 | 0.48 | 0.44 | 0.48 | 0.53 | 0.51 | 0.48 | 0.26 | 0.55 |
| 15,000 | 0.68 | 0.61 | 0.56 | 0.64 | 0.61 | 0.55 | 0.61 | 0.67 | 0.64 | 0.61 | 0.31 | 0.70 |
| 20,000 | 0.80 | 0.71 | 0.65 | 0.75 | 0.71 | 0.64 | 0.71 | 0.79 | 0.75 | 0.71 | 0.34 | 0.82 |
| 25,000 | 0.92 | 0.80 | 0.72 | 0.85 | 0.80 | 0.72 | 0.80 | 0.90 | 0.85 | 0.80 | 0.36 | 0.93 |
| 30,000 | 1.02 | 0.88 | 0.78 | 0.93 | 0.88 | 0.79 | 0.88 | 0.99 | 0.93 | 0.88 | 0.38 | 1.03 |
| 35,000 | 1.12 | 0.95 | 0.83 | 1.00 | 0.95 | 0.85 | 0.95 | 1.07 | 1.01 | 0.95 | 0.38 | 1.12 |
| 40,000 | 1.21 | 1.01 | 0.88 | 1.07 | 1.01 | 0.90 | 1.01 | 1.15 | 1.08 | 1.01 | 0.39 | 1.20 |
| 45,000 | 1.29 | 1.07 | 0.92 | 1.14 | 1.07 | 0.95 | 1.07 | 1.22 | 1.14 | 1.07 | 0.39 | 1.27 |
| 50,000 | 1.37 | 1.12 | 0.96 | 1.19 | 1.12 | 0.99 | 1.12 | 1.29 | 1.20 | 1.12 | 0.39 | 1.34 |
| 55,000 | 1.44 | 1.17 | 0.99 | 1.25 | 1.17 | 1.03 | 1.17 | 1.35 | 1.26 | 1.17 | 0.38 | 1.41 |
| 60,000 | 1.51 | 1.22 | 1.02 | 1.30 | 1.22 | 1.07 | 1.22 | 1.40 | 1.31 | 1.22 | 0.37 | 1.47 |
| 65,000 | 1.58 | 1.26 | 1.05 | 1.34 | 1.26 | 1.11 | 1.26 | 1.46 | 1.36 | 1.26 | 0.36 | 1.52 |
| 70,000 | 1.64 | 1.30 | 1.07 | 1.39 | 1.30 | 1.14 | 1.30 | 1.51 | 1.40 | 1.30 | 0.35 | 1.58 |
| 75,000 | 1.70 | 1.34 | 1.09 | 1.43 | 1.34 | 1.17 | 1.34 | 1.55 | 1.44 | 1.34 | 0.34 | 1.63 |
| 80,000 | 1.76 | 1.37 | 1.11 | 1.46 | 1.37 | 1.19 | 1.37 | 1.60 | 1.48 | 1.37 | 0.32 | 1.68 |
| 85,000 | 1.82 | 1.40 | 1.12 | 1.50 | 1.40 | 1.22 | 1.40 | 1.64 | 1.52 | 1.40 | 0.30 | 1.73 |
| 90,000 | 1.87 | 1.43 | 1.14 | 1.53 | 1.43 | 1.24 | 1.43 | 1.68 | 1.56 | 1.43 | 0.29 | 1.77 |
| 95,000 | 1.93 | 1.46 | 1.15 | 1.57 | 1.46 | 1.27 | 1.46 | 1.72 | 1.59 | 1.46 | 0.27 | 1.81 |
| 100,000 | 1.98 | 1.49 | 1.16 | 1.60 | 1.49 | 1.29 | 1.49 | 1.76 | 1.62 | 1.49 | 0.25 | 1.85 |

### Six-Lane Freeway Segments

For six-lane freeway/segments, lighting is recommended under the following scenarios:

- All conditions with AADT > 90,000;

- AADT > 70,000 with 4-ft left shoulder;

- AADT > 70,000 with 4-ft right shoulder;

- AADT > 60,000 with 3000-ft radius curve;

- AADT > 70,000 with 6000-ft radius curve;

- AADT > 75,000 with 10-ft median.

Table 79 Lighting Benefit for Six-Lane Freeway Segments

| 6-Lane | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Curve |  |  | Median Width (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | No | 50%/3000ft | 50%/6000ft | 10 | 36 | 80 |
| 10,000 | 0.44 | 0.40 | 0.32 | 0.43 | 0.40 | 0.34 | 0.40 | 0.45 | 0.42 | 0.43 | 0.41 | 0.37 |
| 15,000 | 0.56 | 0.50 | 0.40 | 0.54 | 0.50 | 0.42 | 0.50 | 0.57 | 0.54 | 0.54 | 0.51 | 0.47 |
| 20,000 | 0.65 | 0.58 | 0.47 | 0.64 | 0.58 | 0.49 | 0.58 | 0.68 | 0.63 | 0.63 | 0.60 | 0.55 |
| 25,000 | 0.73 | 0.66 | 0.53 | 0.72 | 0.66 | 0.54 | 0.66 | 0.77 | 0.71 | 0.71 | 0.67 | 0.62 |
| 30,000 | 0.81 | 0.72 | 0.58 | 0.79 | 0.72 | 0.59 | 0.72 | 0.85 | 0.79 | 0.78 | 0.74 | 0.68 |
| 35,000 | 0.87 | 0.78 | 0.63 | 0.86 | 0.78 | 0.64 | 0.78 | 0.93 | 0.85 | 0.84 | 0.80 | 0.73 |
| 40,000 | 0.93 | 0.84 | 0.67 | 0.92 | 0.84 | 0.68 | 0.84 | 0.99 | 0.91 | 0.90 | 0.86 | 0.78 |
| 45,000 | 0.98 | 0.88 | 0.71 | 0.98 | 0.88 | 0.71 | 0.88 | 1.06 | 0.97 | 0.95 | 0.91 | 0.83 |
| 50,000 | 1.03 | 0.93 | 0.75 | 1.03 | 0.93 | 0.75 | 0.93 | 1.12 | 1.02 | 1.00 | 0.95 | 0.87 |
| 55,000 | 1.08 | 0.97 | 0.78 | 1.08 | 0.97 | 0.77 | 0.97 | 1.17 | 1.07 | 1.05 | 0.99 | 0.91 |
| 60,000 | 1.12 | 1.01 | 0.82 | 1.12 | 1.01 | 0.80 | 1.01 | 1.22 | 1.11 | 1.09 | 1.03 | 0.95 |
| 65,000 | 1.16 | 1.05 | 0.85 | 1.17 | 1.05 | 0.83 | 1.05 | 1.27 | 1.16 | 1.13 | 1.07 | 0.98 |
| 70,000 | 1.20 | 1.08 | 0.87 | 1.21 | 1.08 | 0.85 | 1.08 | 1.32 | 1.20 | 1.16 | 1.11 | 1.01 |
| 75,000 | 1.23 | 1.11 | 0.90 | 1.24 | 1.11 | 0.87 | 1.11 | 1.36 | 1.24 | 1.20 | 1.14 | 1.04 |
| 80,000 | 1.27 | 1.14 | 0.92 | 1.28 | 1.14 | 0.89 | 1.14 | 1.40 | 1.27 | 1.23 | 1.17 | 1.07 |
| 85,000 | 1.30 | 1.17 | 0.95 | 1.31 | 1.17 | 0.90 | 1.17 | 1.45 | 1.30 | 1.26 | 1.20 | 1.10 |
| 90,000 | 1.33 | 1.19 | 0.97 | 1.34 | 1.19 | 0.92 | 1.19 | 1.48 | 1.34 | 1.29 | 1.22 | 1.12 |
| 95,000 | 1.35 | 1.22 | 0.99 | 1.37 | 1.22 | 0.93 | 1.22 | 1.52 | 1.37 | 1.32 | 1.25 | 1.14 |
| 100,000 | 1.38 | 1.24 | 1.01 | 1.40 | 1.24 | 0.95 | 1.24 | 1.56 | 1.40 | 1.34 | 1.27 | 1.17 |

## Interchange Segments

### Four-Lane Interchange Segments

Table 80 Lighting Benefit for Four-Lane Urban Interchange Segments

| 4-Lane Urban | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Median Width (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | 10 | 36 | 80 |
| 10,000 | 1.36 | 1.27 | 1.23 | 1.50 | 1.27 | 1.15 | 1.53 | 1.35 | 1.08 |
| 15,000 | 1.64 | 1.52 | 1.46 | 1.82 | 1.52 | 1.36 | 1.85 | 1.62 | 1.28 |
| 20,000 | 1.86 | 1.72 | 1.64 | 2.09 | 1.72 | 1.52 | 2.10 | 1.83 | 1.44 |
| 25,000 | 2.05 | 1.88 | 1.79 | 2.32 | 1.88 | 1.65 | 2.31 | 2.01 | 1.56 |
| 30,000 | 2.21 | 2.02 | 1.92 | 2.51 | 2.02 | 1.75 | 2.49 | 2.16 | 1.67 |
| 35,000 | 2.36 | 2.14 | 2.03 | 2.69 | 2.14 | 1.85 | 2.66 | 2.30 | 1.76 |
| 40,000 | 2.49 | 2.25 | 2.13 | 2.86 | 2.25 | 1.92 | 2.80 | 2.42 | 1.84 |
| 45,000 | 2.61 | 2.35 | 2.21 | 3.01 | 2.35 | 1.99 | 2.94 | 2.52 | 1.91 |
| 50,000 | 2.71 | 2.43 | 2.29 | 3.14 | 2.43 | 2.05 | 3.06 | 2.62 | 1.98 |
| 55,000 | 2.81 | 2.51 | 2.36 | 3.27 | 2.51 | 2.11 | 3.17 | 2.71 | 2.03 |
| 60,000 | 2.90 | 2.59 | 2.42 | 3.39 | 2.59 | 2.15 | 3.28 | 2.79 | 2.08 |
| 65,000 | 2.99 | 2.65 | 2.48 | 3.51 | 2.65 | 2.20 | 3.37 | 2.87 | 2.13 |
| 70,000 | 3.07 | 2.72 | 2.53 | 3.61 | 2.72 | 2.23 | 3.46 | 2.94 | 2.17 |
| 75,000 | 3.14 | 2.77 | 2.58 | 3.72 | 2.77 | 2.27 | 3.55 | 3.00 | 2.21 |
| 80,000 | 3.21 | 2.83 | 2.63 | 3.81 | 2.83 | 2.30 | 3.63 | 3.06 | 2.24 |
| 85,000 | 3.28 | 2.87 | 2.67 | 3.91 | 2.87 | 2.32 | 3.70 | 3.12 | 2.27 |
| 90,000 | 3.34 | 2.92 | 2.70 | 3.99 | 2.92 | 2.34 | 3.77 | 3.17 | 2.30 |
| 95,000 | 3.40 | 2.96 | 2.74 | 4.08 | 2.96 | 2.36 | 3.84 | 3.22 | 2.32 |
| 100,000 | 3.46 | 3.00 | 2.77 | 4.16 | 3.00 | 2.38 | 3.90 | 3.27 | 2.35 |

Table 81 Lighting Benefit for Four-Lane Rural Interchange Segments

| 4-Lane Rural | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Median Width (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | 10 | 36 | 80 |
| 10,000 | 0.98 | 0.89 | 0.84 | 1.12 | 0.89 | 0.77 | 1.10 | 0.95 | 0.73 |
| 15,000 | 1.15 | 1.03 | 0.97 | 1.33 | 1.03 | 0.87 | 1.30 | 1.11 | 0.84 |
| 20,000 | 1.28 | 1.13 | 1.05 | 1.50 | 1.13 | 0.93 | 1.44 | 1.22 | 0.90 |
| 25,000 | 1.38 | 1.21 | 1.12 | 1.64 | 1.21 | 0.97 | 1.55 | 1.31 | 0.95 |
| 30,000 | 1.46 | 1.27 | 1.17 | 1.76 | 1.27 | 1.00 | 1.65 | 1.38 | 0.99 |
| 35,000 | 1.53 | 1.31 | 1.20 | 1.86 | 1.31 | 1.02 | 1.73 | 1.44 | 1.01 |
| 40,000 | 1.59 | 1.35 | 1.23 | 1.96 | 1.35 | 1.02 | 1.79 | 1.48 | 1.03 |
| 45,000 | 1.64 | 1.38 | 1.24 | 2.04 | 1.38 | 1.02 | 1.85 | 1.52 | 1.03 |
| 50,000 | 1.68 | 1.40 | 1.26 | 2.11 | 1.40 | 1.02 | 1.90 | 1.55 | 1.04 |
| 55,000 | 1.72 | 1.42 | 1.26 | 2.18 | 1.42 | 1.01 | 1.94 | 1.57 | 1.04 |
| 60,000 | 1.75 | 1.43 | 1.27 | 2.24 | 1.43 | 1.00 | 1.98 | 1.59 | 1.03 |
| 65,000 | 1.77 | 1.44 | 1.26 | 2.29 | 1.44 | 0.98 | 2.01 | 1.61 | 1.02 |
| 70,000 | 1.80 | 1.44 | 1.26 | 2.34 | 1.44 | 0.96 | 2.03 | 1.62 | 1.01 |
| 75,000 | 1.81 | 1.44 | 1.25 | 2.39 | 1.44 | 0.94 | 2.06 | 1.63 | 1.00 |
| 80,000 | 1.83 | 1.44 | 1.24 | 2.43 | 1.44 | 0.91 | 2.08 | 1.63 | 0.98 |
| 85,000 | 1.84 | 1.44 | 1.23 | 2.47 | 1.44 | 0.88 | 2.09 | 1.63 | 0.97 |
| 90,000 | 1.85 | 1.43 | 1.21 | 2.50 | 1.43 | 0.86 | 2.10 | 1.63 | 0.95 |
| 95,000 | 1.86 | 1.42 | 1.20 | 2.54 | 1.42 | 0.82 | 2.11 | 1.63 | 0.93 |
| 100,000 | 1.87 | 1.41 | 1.18 | 2.57 | 1.41 | 0.79 | 2.12 | 1.62 | 0.90 |

For four-lane interchange segments, lighting is recommended under the following scenarios:

- All conditions with AADT > 80,000

- AADT > 70,000 with 4-ft left shoulder

- AADT > 50,000 with 4-ft right shoulder

- AADT > 50,000 with 10-ft median

### Six-Lane Interchange Segments

For six-lane interchange segments, lighting is recommended under the following scenarios:

- All conditions with AADT > 80,000

- AADT > 65,000 with 4-ft right shoulder

- AADT > 70,000 with 10-ft median

- AADT > 45,000 with 3000 ft radius curve

- AADT > 65,000 with 6000 ft radius curve

Table 82 Lighting Benefit for Six-Lane Urban Interchange Segments

| 6-Lane Urban | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Median Width (ft) |  |  | Curve |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | 10 | 36 | 80 | No | 50%/3000 | 50%/6000 |
| 10,000 | 0.95 | 0.88 | 0.84 | 1.06 | 0.88 | 0.78 | 1.01 | 0.83 | 0.71 | 0.88 | 1.29 | 1.08 |
| 15,000 | 1.21 | 1.11 | 1.06 | 1.36 | 1.11 | 0.98 | 1.28 | 1.05 | 0.90 | 1.11 | 1.65 | 1.37 |
| 20,000 | 1.43 | 1.31 | 1.25 | 1.62 | 1.31 | 1.15 | 1.52 | 1.23 | 1.05 | 1.31 | 1.96 | 1.63 |
| 25,000 | 1.64 | 1.50 | 1.42 | 1.85 | 1.50 | 1.31 | 1.74 | 1.40 | 1.18 | 1.50 | 2.25 | 1.86 |
| 30,000 | 1.82 | 1.66 | 1.58 | 2.06 | 1.66 | 1.45 | 1.94 | 1.55 | 1.31 | 1.66 | 2.51 | 2.07 |
| 35,000 | 1.99 | 1.81 | 1.72 | 2.26 | 1.81 | 1.57 | 2.12 | 1.69 | 1.42 | 1.81 | 2.76 | 2.27 |
| 40,000 | 2.15 | 1.96 | 1.86 | 2.45 | 1.96 | 1.69 | 2.30 | 1.83 | 1.53 | 1.96 | 2.99 | 2.46 |
| 45,000 | 2.31 | 2.10 | 1.99 | 2.63 | 2.10 | 1.81 | 2.46 | 1.95 | 1.62 | 2.10 | 3.21 | 2.63 |
| 50,000 | 2.45 | 2.22 | 2.11 | 2.80 | 2.22 | 1.92 | 2.62 | 2.07 | 1.72 | 2.22 | 3.42 | 2.80 |
| 55,000 | 2.59 | 2.35 | 2.22 | 2.96 | 2.35 | 2.02 | 2.77 | 2.18 | 1.81 | 2.35 | 3.62 | 2.97 |
| 60,000 | 2.72 | 2.47 | 2.33 | 3.12 | 2.47 | 2.11 | 2.92 | 2.29 | 1.89 | 2.47 | 3.82 | 3.12 |
| 65,000 | 2.85 | 2.58 | 2.44 | 3.27 | 2.58 | 2.21 | 3.06 | 2.39 | 1.97 | 2.58 | 4.01 | 3.27 |
| 70,000 | 2.98 | 2.69 | 2.54 | 3.42 | 2.69 | 2.30 | 3.20 | 2.49 | 2.05 | 2.69 | 4.19 | 3.42 |
| 75,000 | 3.10 | 2.80 | 2.64 | 3.56 | 2.80 | 2.39 | 3.33 | 2.59 | 2.12 | 2.80 | 4.37 | 3.56 |
| 80,000 | 3.22 | 2.90 | 2.74 | 3.70 | 2.90 | 2.47 | 3.45 | 2.68 | 2.19 | 2.90 | 4.54 | 3.69 |
| 85,000 | 3.33 | 3.00 | 2.83 | 3.84 | 3.00 | 2.55 | 3.58 | 2.77 | 2.26 | 3.00 | 4.71 | 3.83 |
| 90,000 | 3.44 | 3.10 | 2.92 | 3.97 | 3.10 | 2.63 | 3.70 | 2.86 | 2.33 | 3.10 | 4.87 | 3.96 |
| 95,000 | 3.55 | 3.19 | 3.01 | 4.10 | 3.19 | 2.71 | 3.82 | 2.95 | 2.40 | 3.19 | 5.03 | 4.08 |
| 100,000 | 3.66 | 3.29 | 3.10 | 4.22 | 3.29 | 2.78 | 3.93 | 3.03 | 2.46 | 3.29 | 5.19 | 4.21 |

Table 83 Lighting Benefit for Six-Lane Rural Interchange Segments

| 6-Lane Rural | Lighting Safety Benefits (reduction in nighttime crashes/year) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Median Width (ft) |  |  | Curve |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | 10 | 36 | 80 | No | 50%/3000 | 50%/6000 |
| 10,000 | 0.62 | 0.58 | 0.55 | 0.77 | 0.58 | 0.38 | 0.71 | 0.53 | 0.42 | 0.58 | 0.95 | 0.76 |
| 15,000 | 0.78 | 0.73 | 0.68 | 0.97 | 0.73 | 0.45 | 0.90 | 0.66 | 0.51 | 0.73 | 1.21 | 0.96 |
| 20,000 | 0.91 | 0.85 | 0.79 | 1.15 | 0.85 | 0.51 | 1.06 | 0.76 | 0.58 | 0.85 | 1.43 | 1.13 |
| 25,000 | 1.02 | 0.95 | 0.88 | 1.31 | 0.95 | 0.56 | 1.20 | 0.86 | 0.64 | 0.95 | 1.63 | 1.28 |
| 30,000 | 1.13 | 1.05 | 0.97 | 1.45 | 1.05 | 0.60 | 1.32 | 0.94 | 0.69 | 1.05 | 1.81 | 1.42 |
| 35,000 | 1.22 | 1.13 | 1.04 | 1.58 | 1.13 | 0.64 | 1.44 | 1.01 | 0.74 | 1.13 | 1.98 | 1.54 |
| 40,000 | 1.31 | 1.21 | 1.11 | 1.71 | 1.21 | 0.67 | 1.55 | 1.08 | 0.78 | 1.21 | 2.14 | 1.66 |
| 45,000 | 1.40 | 1.29 | 1.18 | 1.82 | 1.29 | 0.70 | 1.66 | 1.14 | 0.82 | 1.29 | 2.29 | 1.77 |
| 50,000 | 1.47 | 1.36 | 1.24 | 1.94 | 1.36 | 0.73 | 1.76 | 1.20 | 0.85 | 1.36 | 2.44 | 1.88 |
| 55,000 | 1.55 | 1.43 | 1.30 | 2.04 | 1.43 | 0.75 | 1.85 | 1.26 | 0.88 | 1.43 | 2.57 | 1.98 |
| 60,000 | 1.62 | 1.49 | 1.36 | 2.14 | 1.49 | 0.77 | 1.94 | 1.31 | 0.91 | 1.49 | 2.71 | 2.08 |
| 65,000 | 1.69 | 1.55 | 1.41 | 2.24 | 1.55 | 0.79 | 2.03 | 1.36 | 0.94 | 1.55 | 2.83 | 2.17 |
| 70,000 | 1.75 | 1.61 | 1.46 | 2.34 | 1.61 | 0.80 | 2.11 | 1.41 | 0.96 | 1.61 | 2.96 | 2.26 |
| 75,000 | 1.82 | 1.66 | 1.51 | 2.43 | 1.66 | 0.82 | 2.19 | 1.46 | 0.99 | 1.66 | 3.08 | 2.35 |
| 80,000 | 1.88 | 1.72 | 1.55 | 2.52 | 1.72 | 0.83 | 2.27 | 1.50 | 1.01 | 1.72 | 3.19 | 2.43 |
| 85,000 | 1.93 | 1.77 | 1.60 | 2.60 | 1.77 | 0.85 | 2.35 | 1.54 | 1.03 | 1.77 | 3.30 | 2.51 |
| 90,000 | 1.99 | 1.82 | 1.64 | 2.69 | 1.82 | 0.86 | 2.42 | 1.58 | 1.05 | 1.82 | 3.41 | 2.59 |
| 95,000 | 2.04 | 1.86 | 1.68 | 2.77 | 1.86 | 0.87 | 2.49 | 1.62 | 1.07 | 1.86 | 3.52 | 2.67 |
| 100,000 | 2.10 | 1.91 | 1.72 | 2.85 | 1.91 | 0.88 | 2.56 | 1.66 | 1.08 | 1.91 | 3.62 | 2.74 |

## Interchange Ramps

### One-Lane Entry Ramps

For one-lane entry ramps, lighting is recommended under the following scenarios:

- All conditions with AADT > 17,000

- AADT > 16,000 with 4-ft left shoulder

- AADT > 16,000 with 6-ft left shoulder

- AADT > 15,000 with 4-ft right shoulder

- AADT > 16,000 with 6-ft right shoulder

Table 84 Lighting Benefit for 1-Lane Entry Urban Interchange Ramps

| 1-Lane Entry Urban | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 |
| 1,000 | 0.29 | 0.30 | 0.30 | 0.32 | 0.30 | 0.25 |
| 2,000 | 0.49 | 0.49 | 0.49 | 0.54 | 0.49 | 0.41 |
| 3,000 | 0.66 | 0.66 | 0.65 | 0.72 | 0.66 | 0.55 |
| 4,000 | 0.81 | 0.81 | 0.80 | 0.89 | 0.81 | 0.68 |
| 5,000 | 0.96 | 0.95 | 0.94 | 1.04 | 0.95 | 0.80 |
| 6,000 | 1.09 | 1.09 | 1.08 | 1.19 | 1.09 | 0.91 |
| 7,000 | 1.23 | 1.21 | 1.20 | 1.33 | 1.21 | 1.01 |
| 8,000 | 1.35 | 1.34 | 1.32 | 1.46 | 1.34 | 1.12 |
| 9,000 | 1.47 | 1.45 | 1.43 | 1.59 | 1.45 | 1.21 |
| 10,000 | 1.59 | 1.57 | 1.55 | 1.71 | 1.57 | 1.31 |
| 11,000 | 1.70 | 1.68 | 1.65 | 1.84 | 1.68 | 1.40 |
| 12,000 | 1.81 | 1.79 | 1.76 | 1.95 | 1.79 | 1.49 |
| 13,000 | 1.92 | 1.89 | 1.86 | 2.07 | 1.89 | 1.57 |
| 14,000 | 2.03 | 1.99 | 1.96 | 2.18 | 1.99 | 1.66 |
| 15,000 | 2.13 | 2.09 | 2.06 | 2.29 | 2.09 | 1.74 |
| 16,000 | 2.23 | 2.19 | 2.15 | 2.40 | 2.19 | 1.82 |
| 17,000 | 2.33 | 2.29 | 2.25 | 2.51 | 2.29 | 1.90 |
| 18,000 | 2.43 | 2.38 | 2.34 | 2.61 | 2.38 | 1.98 |
| 19,000 | 2.53 | 2.48 | 2.43 | 2.71 | 2.48 | 2.05 |
| 20,000 | 2.62 | 2.57 | 2.52 | 2.81 | 2.57 | 2.13 |

Table 85 Lighting Benefit for 1-Lane Entry Rural Interchange Ramps

| 1-Lane Entry Rural | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 |
| 1,000 | 0.21 | 0.20 | 0.20 | 0.22 | 0.20 | 0.17 |
| 2,000 | 0.34 | 0.33 | 0.32 | 0.36 | 0.33 | 0.27 |
| 3,000 | 0.46 | 0.44 | 0.42 | 0.48 | 0.44 | 0.36 |
| 4,000 | 0.56 | 0.53 | 0.51 | 0.59 | 0.53 | 0.43 |
| 5,000 | 0.65 | 0.62 | 0.60 | 0.69 | 0.62 | 0.51 |
| 6,000 | 0.74 | 0.70 | 0.67 | 0.78 | 0.70 | 0.57 |
| 7,000 | 0.83 | 0.78 | 0.75 | 0.86 | 0.78 | 0.63 |
| 8,000 | 0.91 | 0.86 | 0.82 | 0.95 | 0.86 | 0.69 |
| 9,000 | 0.99 | 0.93 | 0.88 | 1.03 | 0.93 | 0.75 |
| 10,000 | 1.07 | 0.99 | 0.94 | 1.10 | 0.99 | 0.80 |
| 11,000 | 1.14 | 1.06 | 1.01 | 1.18 | 1.06 | 0.85 |
| 12,000 | 1.21 | 1.12 | 1.07 | 1.25 | 1.12 | 0.90 |
| 13,000 | 1.28 | 1.19 | 1.12 | 1.32 | 1.19 | 0.95 |
| 14,000 | 1.35 | 1.25 | 1.18 | 1.39 | 1.25 | 1.00 |
| 15,000 | 1.41 | 1.31 | 1.23 | 1.45 | 1.31 | 1.05 |
| 16,000 | 1.48 | 1.36 | 1.28 | 1.52 | 1.36 | 1.09 |
| 17,000 | 1.54 | 1.42 | 1.34 | 1.58 | 1.42 | 1.14 |
| 18,000 | 1.60 | 1.47 | 1.39 | 1.64 | 1.47 | 1.18 |
| 19,000 | 1.66 | 1.53 | 1.44 | 1.70 | 1.53 | 1.22 |
| 20,000 | 1.72 | 1.58 | 1.48 | 1.76 | 1.58 | 1.26 |

### One-Lane Exit Ramps

For one-lane exit ramps, lighting is recommended under the following scenarios:

- All conditions for diamond ramps AADT > 15,000

- AADT > 17,000 with 4-ft left shoulder;

- AADT > 16,000 with 6-ft left shoulder;

- AADT > 16,000 with 10-ft left shoulder;

- AADT > 15,000 with 4-ft right shoulder;

- AADT > 16,000 with 6-ft right shoulder;

- AADT > 12,000 with ramp metering.

Table 86 Lighting Benefit for 1-Lane Exit Urban Interchange Ramps

| 1-Lane Exit Urban | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Ramp Type |  |  | Ramp Meter |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | Diamond | Direct Connection | Free Flow Loop | Without | With |
| 1,000 | 0.30 | 0.30 | 0.29 | 0.32 | 0.30 | 0.25 | 0.30 | 0.25 | 0.24 | 0.30 | 0.35 |
| 2,000 | 0.49 | 0.49 | 0.49 | 0.54 | 0.49 | 0.41 | 0.49 | 0.40 | 0.39 | 0.49 | 0.59 |
| 3,000 | 0.65 | 0.66 | 0.66 | 0.72 | 0.66 | 0.55 | 0.66 | 0.54 | 0.53 | 0.66 | 0.79 |
| 4,000 | 0.80 | 0.81 | 0.81 | 0.89 | 0.81 | 0.68 | 0.81 | 0.66 | 0.64 | 0.81 | 0.98 |
| 5,000 | 0.94 | 0.95 | 0.96 | 1.04 | 0.95 | 0.80 | 0.95 | 0.78 | 0.75 | 0.95 | 1.16 |
| 6,000 | 1.08 | 1.09 | 1.09 | 1.19 | 1.09 | 0.91 | 1.09 | 0.88 | 0.86 | 1.09 | 1.32 |
| 7,000 | 1.20 | 1.21 | 1.23 | 1.33 | 1.21 | 1.01 | 1.21 | 0.98 | 0.95 | 1.21 | 1.48 |
| 8,000 | 1.32 | 1.34 | 1.35 | 1.46 | 1.34 | 1.12 | 1.34 | 1.08 | 1.05 | 1.34 | 1.64 |
| 9,000 | 1.43 | 1.45 | 1.47 | 1.59 | 1.45 | 1.21 | 1.45 | 1.17 | 1.14 | 1.45 | 1.78 |
| 10,000 | 1.55 | 1.57 | 1.59 | 1.71 | 1.57 | 1.31 | 1.57 | 1.26 | 1.22 | 1.57 | 1.93 |
| 11,000 | 1.65 | 1.68 | 1.70 | 1.84 | 1.68 | 1.40 | 1.68 | 1.35 | 1.31 | 1.68 | 2.07 |
| 12,000 | 1.76 | 1.79 | 1.81 | 1.95 | 1.79 | 1.49 | 1.79 | 1.43 | 1.39 | 1.79 | 2.21 |
| 13,000 | 1.86 | 1.89 | 1.92 | 2.07 | 1.89 | 1.57 | 1.89 | 1.52 | 1.47 | 1.89 | 2.34 |
| 14,000 | 1.96 | 1.99 | 2.03 | 2.18 | 1.99 | 1.66 | 1.99 | 1.60 | 1.55 | 1.99 | 2.47 |
| 15,000 | 2.06 | 2.09 | 2.13 | 2.29 | 2.09 | 1.74 | 2.09 | 1.67 | 1.62 | 2.09 | 2.60 |
| 16,000 | 2.15 | 2.19 | 2.23 | 2.40 | 2.19 | 1.82 | 2.19 | 1.75 | 1.69 | 2.19 | 2.73 |
| 17,000 | 2.25 | 2.29 | 2.33 | 2.51 | 2.29 | 1.90 | 2.29 | 1.82 | 1.77 | 2.29 | 2.85 |
| 18,000 | 2.34 | 2.38 | 2.43 | 2.61 | 2.38 | 1.98 | 2.38 | 1.90 | 1.84 | 2.38 | 2.97 |
| 19,000 | 2.43 | 2.48 | 2.53 | 2.71 | 2.48 | 2.05 | 2.48 | 1.97 | 1.91 | 2.48 | 3.09 |
| 20,000 | 2.52 | 2.57 | 2.62 | 2.81 | 2.57 | 2.13 | 2.57 | 2.04 | 1.98 | 2.57 | 3.21 |

Table 87 Lighting Benefit for 1-Lane Exit Rural Interchange Ramps

| 1-Lane Exit Rural | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Ramp Type |  |  | Ramp Meter |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | Diamond | Direct Connection | Free Flow Loop | Without | With |
| 1,000 | 0.20 | 0.20 | 0.21 | 0.22 | 0.20 | 0.17 | 0.20 | 0.16 | 0.15 | 0.20 | 0.26 |
| 2,000 | 0.32 | 0.33 | 0.34 | 0.36 | 0.33 | 0.27 | 0.33 | 0.26 | 0.25 | 0.33 | 0.42 |
| 3,000 | 0.42 | 0.44 | 0.46 | 0.48 | 0.44 | 0.36 | 0.44 | 0.34 | 0.32 | 0.44 | 0.57 |
| 4,000 | 0.51 | 0.53 | 0.56 | 0.59 | 0.53 | 0.43 | 0.53 | 0.41 | 0.39 | 0.53 | 0.70 |
| 5,000 | 0.60 | 0.62 | 0.65 | 0.69 | 0.62 | 0.51 | 0.62 | 0.47 | 0.45 | 0.62 | 0.82 |
| 6,000 | 0.67 | 0.70 | 0.74 | 0.78 | 0.70 | 0.57 | 0.70 | 0.53 | 0.51 | 0.70 | 0.94 |
| 7,000 | 0.75 | 0.78 | 0.83 | 0.86 | 0.78 | 0.63 | 0.78 | 0.58 | 0.56 | 0.78 | 1.05 |
| 8,000 | 0.82 | 0.86 | 0.91 | 0.95 | 0.86 | 0.69 | 0.86 | 0.64 | 0.61 | 0.86 | 1.15 |
| 9,000 | 0.88 | 0.93 | 0.99 | 1.03 | 0.93 | 0.75 | 0.93 | 0.69 | 0.66 | 0.93 | 1.26 |
| 10,000 | 0.94 | 0.99 | 1.07 | 1.10 | 0.99 | 0.80 | 0.99 | 0.73 | 0.70 | 0.99 | 1.36 |
| 11,000 | 1.01 | 1.06 | 1.14 | 1.18 | 1.06 | 0.85 | 1.06 | 0.78 | 0.75 | 1.06 | 1.45 |
| 12,000 | 1.07 | 1.12 | 1.21 | 1.25 | 1.12 | 0.90 | 1.12 | 0.82 | 0.79 | 1.12 | 1.54 |
| 13,000 | 1.12 | 1.19 | 1.28 | 1.32 | 1.19 | 0.95 | 1.19 | 0.87 | 0.83 | 1.19 | 1.64 |
| 14,000 | 1.18 | 1.25 | 1.35 | 1.39 | 1.25 | 1.00 | 1.25 | 0.91 | 0.87 | 1.25 | 1.72 |
| 15,000 | 1.23 | 1.31 | 1.41 | 1.45 | 1.31 | 1.05 | 1.31 | 0.95 | 0.90 | 1.31 | 1.81 |
| 16,000 | 1.28 | 1.36 | 1.48 | 1.52 | 1.36 | 1.09 | 1.36 | 0.99 | 0.94 | 1.36 | 1.90 |
| 17,000 | 1.34 | 1.42 | 1.54 | 1.58 | 1.42 | 1.14 | 1.42 | 1.03 | 0.98 | 1.42 | 1.98 |
| 18,000 | 1.39 | 1.47 | 1.60 | 1.64 | 1.47 | 1.18 | 1.47 | 1.06 | 1.01 | 1.47 | 2.06 |
| 19,000 | 1.44 | 1.53 | 1.66 | 1.70 | 1.53 | 1.22 | 1.53 | 1.10 | 1.04 | 1.53 | 2.14 |
| 20,000 | 1.48 | 1.58 | 1.72 | 1.76 | 1.58 | 1.26 | 1.58 | 1.13 | 1.08 | 1.58 | 2.22 |

### Two-Lane Entry Ramps

For two-lane entry ramps, lighting is recommended under the following scenarios:

- AADT > 15,000 with 4-ft left shoulder;

- AADT > 16,000 with 6-ft left shoulder;

- AADT > 20,000 with 10-ft left shoulder;

- AADT > 15,000 with 4-ft right shoulder;

- AADT > 16,000 with 6-ft right shoulder;

- AADT > 19,000 with 10-ft right shoulder.

Table 88 Lighting Benefit for 2-Lane Entry Interchange Ramps

| 2-Lane Entry | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 |
| 1,000 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 |
| 2,000 | 0.07 | 0.06 | 0.03 | 0.07 | 0.06 | 0.04 |
| 3,000 | 0.13 | 0.11 | 0.07 | 0.13 | 0.11 | 0.08 |
| 4,000 | 0.20 | 0.17 | 0.11 | 0.19 | 0.17 | 0.13 |
| 5,000 | 0.27 | 0.23 | 0.16 | 0.26 | 0.23 | 0.18 |
| 6,000 | 0.33 | 0.29 | 0.21 | 0.32 | 0.29 | 0.23 |
| 7,000 | 0.40 | 0.35 | 0.26 | 0.39 | 0.35 | 0.28 |
| 8,000 | 0.47 | 0.41 | 0.31 | 0.46 | 0.41 | 0.33 |
| 9,000 | 0.54 | 0.47 | 0.36 | 0.52 | 0.47 | 0.39 |
| 10,000 | 0.60 | 0.53 | 0.41 | 0.59 | 0.53 | 0.44 |
| 11,000 | 0.67 | 0.60 | 0.46 | 0.66 | 0.60 | 0.49 |
| 12,000 | 0.74 | 0.66 | 0.51 | 0.72 | 0.66 | 0.54 |
| 13,000 | 0.81 | 0.72 | 0.56 | 0.79 | 0.72 | 0.59 |
| 14,000 | 0.87 | 0.78 | 0.61 | 0.86 | 0.78 | 0.65 |
| 15,000 | 0.94 | 0.84 | 0.66 | 0.92 | 0.84 | 0.70 |
| 16,000 | 1.01 | 0.90 | 0.71 | 0.99 | 0.90 | 0.75 |
| 17,000 | 1.08 | 0.96 | 0.76 | 1.05 | 0.96 | 0.80 |
| 18,000 | 1.14 | 1.02 | 0.81 | 1.12 | 1.02 | 0.85 |
| 19,000 | 1.21 | 1.09 | 0.86 | 1.19 | 1.09 | 0.90 |
| 20,000 | 1.28 | 1.15 | 0.91 | 1.25 | 1.15 | 0.96 |

### Two-Lane Exit Ramps

For two-lane exit ramps, lighting is recommended under the following scenarios:

- AADT > 15,000 with 4-ft left shoulder;

- AADT > 16,000 with 6-ft left shoulder;

- AADT > 19,000 with 10-ft left shoulder;

- AADT > 12,000 with 4-ft right shoulder;

- AADT > 16,000 with 6-ft right shoulder;

- AADT > 18,000 with 10-ft right shoulder;

- AADT > 16,000 for diamond ramps;

- AADT > 20,000 for direct connection ramps.

Table 89 Lighting Benefit for 2-Lane Exit Interchange Ramps

| 2-Lane Exit | Lighting Safety Benefits (crashes/year) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT (veh/day) | Left Shoulder (ft) |  |  | Right Shoulder (ft) |  |  | Ramp Type |  |  |
|  | 4 | 6 | 10 | 4 | 6 | 10 | Diamond | Direct Connection | Semi Direct Connection |
| 1,000 | 0.06 | 0.05 | 0.03 | 0.08 | 0.05 | 0.03 | 0.05 | 0.03 | 0.00 |
| 2,000 | 0.13 | 0.11 | 0.08 | 0.17 | 0.11 | 0.08 | 0.11 | 0.08 | 0.03 |
| 3,000 | 0.20 | 0.17 | 0.13 | 0.25 | 0.17 | 0.13 | 0.17 | 0.13 | 0.06 |
| 4,000 | 0.27 | 0.24 | 0.18 | 0.33 | 0.24 | 0.18 | 0.24 | 0.18 | 0.10 |
| 5,000 | 0.33 | 0.30 | 0.23 | 0.41 | 0.30 | 0.24 | 0.30 | 0.23 | 0.14 |
| 6,000 | 0.40 | 0.36 | 0.29 | 0.49 | 0.36 | 0.29 | 0.36 | 0.28 | 0.18 |
| 7,000 | 0.47 | 0.42 | 0.34 | 0.56 | 0.42 | 0.34 | 0.42 | 0.33 | 0.22 |
| 8,000 | 0.53 | 0.48 | 0.39 | 0.64 | 0.48 | 0.40 | 0.48 | 0.38 | 0.26 |
| 9,000 | 0.60 | 0.54 | 0.44 | 0.71 | 0.54 | 0.45 | 0.54 | 0.43 | 0.30 |
| 10,000 | 0.66 | 0.60 | 0.49 | 0.78 | 0.60 | 0.50 | 0.60 | 0.48 | 0.34 |
| 11,000 | 0.72 | 0.66 | 0.54 | 0.85 | 0.66 | 0.55 | 0.66 | 0.53 | 0.38 |
| 12,000 | 0.79 | 0.72 | 0.59 | 0.92 | 0.72 | 0.61 | 0.72 | 0.58 | 0.43 |
| 13,000 | 0.85 | 0.78 | 0.64 | 0.99 | 0.78 | 0.66 | 0.78 | 0.63 | 0.47 |
| 14,000 | 0.91 | 0.83 | 0.69 | 1.06 | 0.83 | 0.71 | 0.83 | 0.67 | 0.51 |
| 15,000 | 0.97 | 0.89 | 0.74 | 1.13 | 0.89 | 0.76 | 0.89 | 0.72 | 0.55 |
| 16,000 | 1.03 | 0.95 | 0.79 | 1.20 | 0.95 | 0.81 | 0.95 | 0.77 | 0.59 |
| 17,000 | 1.09 | 1.00 | 0.84 | 1.27 | 1.00 | 0.86 | 1.00 | 0.81 | 0.63 |
| 18,000 | 1.15 | 1.06 | 0.88 | 1.33 | 1.06 | 0.91 | 1.06 | 0.86 | 0.67 |
| 19,000 | 1.21 | 1.12 | 0.93 | 1.40 | 1.12 | 0.96 | 1.12 | 0.91 | 0.71 |
| 20,000 | 1.27 | 1.17 | 0.98 | 1.47 | 1.17 | 1.01 | 1.17 | 0.95 | 0.76 |

## Steps to Prioritize Locations for Lighting Installation

Considering that there are still a number of locations that meet the AADT criteria as described in Sections 9.1 through 9.3, while funds available for lighting installation is limited,  prioritizing locations for lighting installation is necessary. The following paragraphs summarize the recommended steps to complete this prioritization process.

Step 1: Look for red labeled freeway/interchange segments or ramps within the study region on the Crash Risk Map (Section 7.3 Maps 1-3). Crash risk concerns the absolute number of nighttime crashes. Red colored segments/ramps represent those which have the largest crash risks.

Step 2: Confirm in the Top 100 risk table (Appendix C) in terms of high rank. Check whether geometric deficiencies exist. Geometric deficiencies are highlighted in the labels.

Step 3: Check whether the lighting benefit is also high by using the lighting benefit map (Section 7.3 Maps 4-6). Benefits represent the expected reduction in nighttime crash frequency if lighting is installed. Red colored segments/ramps represent those which have the largest lighting benefits.

Step 4: Confirm in the Top 100 benefit table in terms of high rank (Appendix D).

Step 5: Use Tools Proactive Table Tool (Section 7.1) and Proactive Spreadsheet Tool (Section 7.2) to compare the benefit of installing lighting and benefit of changing geometrics.

Step 6: By comprehensive consideration of crash risk, lighting benefit, and comparison of benefits between geometric treatments and installation lighting, locations of lighting installation will be prioritized in order.

# Uniqueness, Benefit and Limitations

## Uniqueness

The uniqueness of the project and tools developed through the project is reflected by the numerical modeling of nighttime safety for various freeway segment types: freeway segments, interchange segments, interchange ramp segments. Therefore, decisions on lighting installation can be made on a quantitative safety basis.  Previous national guidelines for lighting installation are either qualitative or only for a certain type of roadway segment.

The AASHTO Lighting Design Guide specifies warrant conditions for three types of freeway/interchange lighting: Complete freeway lighting (CFL), complete interchange lighting (CIL) and partial interchange lighting (PIL). However, after applying the warrant conditions to existing unlighted freeway in Wisconsin, this criterion can meet more than half of all freeways/interchanges, thus may not be appropriate to implement in Wisconsin. The guidelines developed in Chapter 9, especially the unique six-step method to prioritize lighting installation helps identify the priority lighting installation locations that have the most safety benefit.

FHWA Lighting Handbook provides a lighting installation warranting method as described in the Highway Safety Manual. The method uses a CMF which quantifies the reduction in nighttime crashes through installation of roadway lighting. This CMF can be applied to all roadway types. However, that CMF is a single number. The SPFs based method developed through this project provides specific nighttime crash predictions with respect to different segment types. Additionally, the SPFs enable the comparison of safety benefits of lighting with other geometric improvements, which are reflected in the proactive spreadsheet tool.

## Benefit

The uniqueness of the project and tools developed through the project is reflected by the numerical modeling of nighttime safety for various freeway segment types: freeway segments, interchange segments, interchange ramp segments. Therefore, decisions on lighting installation can be made on a quantitative safety basis.  Previous national guidelines for lighting installation are either qualitative or only for a certain type of roadway segment.

## Limitations

What the study can be used for?

The tools developed through the project can be used to quantitatively estimate the benefit of lighting installation in terms of nighttime crash frequency for three different types of freeway segments.  Lighting benefit can be quantitatively estimated using the tools, and compared to the benefit of making geometric improvements, thus helping making decision on lighting installations. The resulted benefit cost analysis as described in the study further assists the decision making process. From a long run, the study gives support for the need to change State Law to allow lighting to be installed for safety.

What the tools cannot do?

The tools are not a one-click solution that directly warrants the locations for installing lighting. Rather, they are assistance tools that help analyze safety benefit of installing lighting with and without combination of making geometric changes, and prioritize candidate locations for lighting installations. Warrants like the AASHTO and FHWA warrants typically result in a number of warranted locations what are not appropriate with consideration of limited safety improvement funds.

What is the next step?

As the state presently do not have enough information on the signing and marking inventories, the tools do not evaluate the safety benefit of add signings and markings. This shows the benefit of better asset management. In future projects, it is desired that estimation of the benefit of signings and markings can be modeled and added to tool after the signing and marking inventories become available.

This study only covers freeways segments with uninterrupted traffic flow. As nighttime traffic safety issues exist on many expressway segments with intersections, project is needed to quantitatively look into nighttime safety issues at expressway intersections.

# Conclusions

This project performed both qualitative assessment and quantitative modeling of the safety benefits of lighting on freeways and interchanges in Wisconsin. Particularly, safety performance functions that predict nighttime crash frequency under unlighted and lighted conditions were developed. With the developed model and analysis results, analysis tools to implement the obtained results were established to facilitate the decision making on lighting installation. Finally, generalized guidelines on lighting installation on freeways and interchanges were proposed.

Key findings from the qualitative crash analysis and comparisons include:

- For freeway segments, unlighted segments have significantly higher crash rates than lighted segments.

- For ramps, the overall comparison between lighted and unlighted ramp shows statistically significant difference. However, the difference between two-lane and three-lane ramps is not statistically significant.

- Nighttime unlighted crashes had significantly more fatal + injury crashes than nighttime lighted crashes.

- Daytime low visibility crashes, which usually occur under fog, rain or snow conditions, share many common characteristics with nighttime unlighted crashes.

A total of 16 SPFs were developed for predicting and comparing nighttime crash frequency for different segment types and lighting conditions. These models also incorporated various roadways geometric and traffic related variables. Results show that under the same traffic and roadway geometry conditions:

- Lighting reduces nighttime crash frequency for all segment types, though with different magnitude.

- Interchange segments have higher nighttime crash frequencies than non-interchange freeway segments.

- Six-lane freeway segments and interchange segments have lower nighttime crash frequencies than four-lane freeway segments and interchange segments.

- The reduction of nighttime crash frequency by lighting generally increases as AADT increases.

- Entry ramps have higher nighttime crash frequencies than exit ramps under either unlighted or lighted conditions.

A benefit cost analysis was also conducted to determine the high benefit-cost scenarios. Benefit/cost ratios were computed based on 10-year lighting benefits with lighting installation and 10-year operation cost. Key findings are summarized as follows,

- All of the B/C ratios for freeway and interchange segments are greater than 1.0.

- Most low and some medium AADT ramps scenarios have a B/C ratio of less than 1.0.

Lighting guidelines were developed based on the safety benefits of lighting under different scenarios for different segment types. Scenarios of top 20 percentile of lighting benefits are recommended for lighting installation. General conditions under which lighting installation is recommended are:

- Freeway segments: In most cases, segments with more than 90,000 AADT would be recommended for lighting. In some cases, segments with lower AADT combining with certain roadway geometry were also recommended for lighting.

- Interchange segments: In most cases, segments with more than 80,000 AADT would be recommended for lighting. In some cases, segments with lower AADT combining with certain roadway geometry were also recommended for lighting.

- Interchange Ramps: In most cases, segments with more than 15,000 AADT would generally be recommended for lighting. In some cases, certain ramp type, ramp metering and geometry factor combinations will lower the AADT threshold for lighting installation.

Considering that there are still a number of locations that meet the AADT criteria as described in Sections 9.1 through 9.3, while funds available for lighting installation is limited,  a six-step method for prioritizing locations for lighting installation is provided.

# References

- FHWA Lighting Handbook. Federal Highway Administration, U.S. Department of Transportation. 2012.

- Fatality Analysis Reporting System (FARS). National Highway Traffic Safety Administration. 2012. http://www-fars.nhtsa.dot.gov/Crashes/CrashesTime.aspx. Accessed May 2014.

- Analysis of Safety Effects for the Presence of Roadway Lighting. NCHRP Project No. 5-19. Pennsylvania Transportation Institute. 2009.

- Traffic Safety Fundamentals Handbook. Minnesota Department of Transportation. 2008.

- Federal Highway Administration. The 1996 Annual Report on Highway Safety Improvement Programs. Publication No. FHWA-SA-96-040. 1996.

- AASHTO, Roadway Lighting Design Guide, American Association of State Highway and Transportation Officials, Washington, DC, 2005.

- California Traffic Manual. California Department of Transportation. 2006.

- Colorado Lighting Design Guide. Clanton & Associates, Inc. 2006.

- Delaware Lighting Design Guidelines. Delaware Department of Transportation. 2012.

- Illinois Bureau of Design and Environment Manual. Illinois Department of Transportation. 2010.

- Minnesota DOT Roadway Lighting Design Manual. Minnesota Department of Transportation. 2010.

- Minnesota Traffic Engineering Manual. Minnesota Department of Transportation. 2009.

- New Hampshire Traffic Design Manual. New Hampshire Department of Transportation of Transportation. 2010.

- NJDOT Design Manual. New Jersey Department of Transportation. 2013.

- Oregon Lighting Policy and Guidelines. Oregon Department of Transportation. 2003.

- Texas Highway Illumination Manual. Texas Department of Department of Transportation. 2003.

- WSDOT Design Manual. Washington Department of Transportation. 2009.

- Facilities Development Manual. Wisconsin Department of Transportation. 2014.

- Lee C. Gonzalez-Velez E. Comparing Street Lighting Levels with Nighttime Crash Histories -VOLUME I. University of South Florida. 2009.

- Roadway Lighting and Driver Safety. Kentucky Transportation Center. 2003.

- Safety Impact of Street Lighting at Isolated Rural Intersections. Minnesota Department of Transportation. 1999.

- Safety Impacts Of Street Lighting At Isolated Rural Intersections, Part II, Year 1 Report. Center for Transportation Research and Education. Iowa State University. 2004.

- Isebrands, H., Hallmark, S., Hans, Z., McDonald, T., Preston, H., Storm, R., 2004. Safety impacts of street lighting at isolated rural intersections: Part II, Year 1 report. Center for Transportation Research and Education, Iowa State University, Ames, IA.

- Preston, H., Schoenecker, T., 1999. Safety impacts of street lighting at rural intersections. Report No. 1999-17. Minnesota Department of Transportation, St. Paul, MN.

- Elvik, R., Vaa, T., 2004. The Handbook of Road Safety Measures. Elsevier Science.

- Highway Safety Manual 1st edition. American Association of State Traffic and Highway Officials. 2010.

- Wanvik P. Effects of road lighting: An analysis based on Dutch accident statistics 1987–2006. Accident Analysis and Prevention. 41. pp. 123-128. 2009.

- Bruneau J., Morin D. and Pouliot M. Safety of Motorway Lighting. Transportation Research Record 1758, Journal of Transportation Research Board, National Research Council, Washington, D.C. 2001.

- Review of the Safety Benefits and Other Effects of Roadway Lighting. NCHRP Project No. 5-19. Rensselaer Polytechnic University. 2009.

- Donnell E., Porter R., Shankar V. A framework for estimating the safety effects of roadway lighting at intersections, Safety Science, Volume 48, Issue 10, December 2010, Pages 1436-1444.

- Gibbons R., Edwards C., Bhagavathula R., Carlson P., Owens A. Development of Visual Model for Exploring Relationship Between Nighttime Driving Behavior and Roadway Visibility Features. In Transportation Research Record: Journal of the Transportation Research Board, No. 2298, Transportation Research Board of the National Academies, Washington, D.C., 2012, pp. 96-103.

- New Lighting Technologies and Roadway Lighting: An Informational Brochure. Lighting Research Center, Rensselaer Polytechnic Institute. 2012.

- Terry T. and Gibbons R. Evaluation of Guardrail Embedded Lighting System in Trinidad, Colorado. Colorado Department of Transportation. 2014.

- Design Criteria for Adaptive Roadway Lighting. Federal Highway Administration. 2014.

- Janoff, M.S. and McCunney, W. “Economic Analysis of Roadway Lighting.” Journal of Illuminating Engineering Society, Vol. 8, No. 4. 1979.

- Mcfarland, W.F., and Walton, N.E. “Economic and Accident Potential Analysis of Roadway Lighting Alternatives.” Highway Research Record 377, pp. 92-102. 1971.

- Box, P.C. Major Road Accident Reduction by Illumination. Transportation Research Record 1247, Journal of Transportation Research Board, National Research Council, Washington, D.C., pp. 32-38. 1989.

- Highway Safety Manual Case Study 4: Development of Safety Performance Functions for Network Screening in Illinois. Federal Highway Administration. U.S. Department of Transportation.

- Linear Regression Crash Prediction Models: Issues and Proposed Solutions. Virginia Tech Transportation Institute. 2010.

- Kulmala, R. Safety at Rural three and Four-arm Junctions: Development of Accident Prediction Models, Espoo: Technical Research Centre of Finland, VTT Publications 233, 1995.

- A Policy on Geometric Design of Highways and Streets. American Association of State Traffic and Highway Officials. 2011.

- Highway Capacity Manual. American Association of State Traffic and Highway Officials. 2010.

- Safety Prediction Methodology and Analysis Tool for Freeways and Interchanges. NCHRP Project 17-45. National Cooperative Highway Research Program Transportation Research Board of the National Academies. 2012.

- Designated Freeways and Expressways. SEC. 84.29 & 84.295 Wisconsin Statutes. 2013.

- Crash Data Query Tool. WisTransPortal. University of Wisconsin-Madison. http://transportal.cee.wisc.edu/. Accessed June 2014.

- United States (U.S.) Naval Observatory Sunrise and Sunset Data Retrieval. http://aa.usno.navy.mil/data/docs/RS_OneYear.php. Accessed May 2014.

- Persaud, B. N., Statistical Methods in Highway Safety Analysis, National Cooperative Highway Research Program Synthesis 295, Transportation Research Board, 2001.

- R Core Team. R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria. ISBN 3-900051-07-0. http://www.R-project.org/. 2013.

- Croissant Y. Estimation of Multinomial Logit models in R: The mlogit Packages. 2012.

- Hauer E, Harwood D., Council F. and Griffith M. Estimating Safety by the Empirical Bayes Method: A Tutorial. http://www.ctre.iastate.edu/educweb/ce552/docs/Bayes_tutor_hauer.pdf

- Average Retail Price of Electricity to Ultimate Customers by End-Use Sector. Electric Power Monthly. U.S. Energy Information Administration.  http://www.eia.gov/electricity/monthly/epm_table_grapher.cfm?t=epmt_5_6_a

# Appendix A Glossary

Freeway (Full Access Controlled Highway):

Designed exclusively for high-speed, unhindered vehicular traffic, with no traffic signals, intersections, or property access. These highways are free of any at-grade crossings with other roads or railroads, which instead use overpasses and underpasses to cross the highway. Entrance and exit to the highway is provided by ramps at interchanges.

Expressway (Limited Access Highway):

A highway or arterial road for high-speed traffic which has many characteristics of a Controlled-Access highway (see above), including limited or no access to adjacent property, some degree of separation of opposing traffic flow, the use of grade-separated interchanges to some extent, and few or no intersecting roadways.

Interchange

A system of interconnecting roadways in conjunction with one or more grade separations that provides for the movement of traffic between two or more roadways or highways on different levels.

SPF Base Conditions: Base conditions are considered to be the most commonly observed conditions on roadways. Most numbers refer to those used in the NCHRP 17-45 final report.

- Lane width: 12 ft.;

- Left shoulder Width: 6 ft.; (4 ft for ramp)

- Right shoulder Width: 10 ft.; (6 ft for ramp)

- Median width: 48 ft.;

- No curve present;

- No median barrier present;

- Median type: Grass;

- Area type: Urban;

- Ramp type: Diamond;

- No ramp meter.

Worst Case Conditions

- Lane width: 12 ft.;

- Left shoulder Width: 4 ft.;

- Right shoulder Width: 4 ft.;

- Median width: 10 ft.;

- Curve proportion: 100%;

- Curve radius: 2000 ft.

- No median barrier present;

- Median type: Grass;

- Area type: Urban;

- Ramp type: Diamond;

- No ramp meter.

Interchange Types

| Diamond | Full Cloverleaf | ParClo |
| --- | --- | --- |
| Example: I43 at W Good Hope Rd | Example: I43 at STH 100 | Example: I 894 at W National Ave |
| Trumpet | Three-Leg Directional | All Directional |
| Example: I94 at STH 119 | Example: I 894 at I43 | Example: I 94 at Miller Parkway |

Ramp Types

| Diamond | Free Flow Loop | ParClo Loop |
| --- | --- | --- |
| Apply to all diamond interchange ramps | Apply to loops in full cloverleaf and trumpet interchanges | Apply to loops in ParClo interchanges |
| Outer Connection | Direct Connection | Semi-Direct Connection |
| Apply to non-loop ramps in ParClo, cloverleaf and trumpet interchanges | Apply to right turn ramps in three-leg and all directional interchanges | Apply to left-turn ramps in three-leg and all directional interchanges |

Nighttime Crash Frequency

The number of nighttime crashes occurring on a segment/ramp per year, with nighttime crashes/year as unit.

Nighttime Crash Density

The number of nighttime crashes occurring on a segment/ramp per year per mile, with nighttime crashes/year/mile as unit. Using nighttime crash density could facilitate the comparison of segments with different lengths.

# Appendix B Data Summary by County

Table B-1 Summary of Freeway Segment and Interchange Information by County

(Descending order according to total freeway length)

| Region | County ID | County Name | Freeway Segment Length (mi) | Number of Interchanges | Number of Freeway Segments | Number of Ramps | Number of Nighttime Crashes (5-year period) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SW | 13 | Dane | 124.70 | 53 | 150 | 224 | 2943 |
| NC | 37 | Marathon | 86.51 | 26 | 86 | 96 | 1302 |
| SE | 40 | Milwaukee | 69.94 | 78 | 137 | 304 | 5791 |
| NE | 5 | Brown | 68.47 | 30 | 78 | 124 | 1614 |
| NW | 9 | Chippewa | 65.24 | 18 | 65 | 72 | 689 |
| NC | 49 | Portage | 60.33 | 17 | 56 | 67 | 564 |
| NE | 70 | Winnebago | 58.88 | 21 | 66 | 84 | 1530 |
| SE | 67 | Waukesha | 58.18 | 31 | 66 | 118 | 1920 |
| NC | 58 | Shawano | 55.30 | 7 | 47 | 26 | 801 |
| SE | 64 | Walworth | 50.07 | 14 | 47 | 60 | 600 |
| SW | 41 | Monroe | 46.21 | 10 | 39 | 38 | 583 |
| NE | 42 | Oconto | 43.06 | 8 | 40 | 28 | 281 |
| SE | 66 | Washington | 41.95 | 16 | 43 | 63 | 1063 |
| NW | 27 | Jackson | 41.16 | 5 | 33 | 22 | 453 |
| NW | 16 | Douglas | 41.07 | 3 | 35 | 9 | 237 |
| NW | 55 | St Croix | 40.15 | 12 | 40 | 45 | 822 |
| SW | 11 | Columbia | 39.29 | 10 | 36 | 44 | 616 |
| NE | 59 | Sheboygan | 38.21 | 12 | 42 | 52 | 642 |
| NW | 18 | Eau Claire | 37.95 | 10 | 37 | 44 | 726 |
| NE | 20 | Fond Du Lac | 37.92 | 13 | 38 | 7 | 595 |
| NW | 65 | Washburn | 37.89 | 2 | 31 | 52 | 258 |
| SW | 53 | Rock | 37.09 | 9 | 35 | 48 | 583 |
| NE | 36 | Manitowoc | 33.86 | 8 | 32 | 32 | 406 |
| NW | 3 | Barron | 33.64 | 4 | 25 | 16 | 146 |
| SW | 29 | Juneau | 33.57 | 5 | 26 | 20 | 396 |
| SW | 14 | Dodge | 33.09 | 10 | 35 | 40 | 456 |
| NC | 35 | Lincoln | 31.71 | 7 | 31 | 28 | 338 |
| SW | 32 | La Crosse | 31.50 | 12 | 36 | 55 | 658 |
| NW | 10 | Clark | 30.14 | 5 | 25 | 18 | 283 |
| NW | 17 | Dunn | 29.02 | 5 | 26 | 20 | 499 |
| SW | 25 | Iowa | 28.74 | 5 | 26 | 20 | 228 |
| SE | 45 | Ozaukee | 27.41 | 8 | 26 | 30 | 472 |
| NC | 68 | Waupaca | 26.25 | 7 | 26 | 28 | 486 |
| SW | 28 | Jefferson | 25.22 | 4 | 20 | 14 | 393 |
| NC | 39 | Marquette | 23.84 | 4 | 20 | 14 | 192 |
| NE | 44 | Outagamie | 22.76 | 14 | 32 | 56 | 695 |
| SW | 22 | Grant | 21.41 | 8 | 21 | 29 | 217 |
| NC | 69 | Waushara | 18.80 | 3 | 17 | 12 | 122 |
| NE | 38 | Marinette | 18.40 | 3 | 17 | 12 | 112 |
| SW | 56 | Sauk | 15.75 | 3 | 13 | 14 | 315 |
| SE | 51 | Racine | 12.24 | 7 | 14 | 26 | 340 |
| SE | 30 | Kenosha | 11.81 | 6 | 13 | 27 | 399 |
| SW | 33 | La Fayette | 11.35 | 1 | 9 | 4 | 96 |
| NC | 43 | Oneida | 6.35 | 0 | 4 | 0 | 29 |
| NW | 61 | Trempealeau | 4.18 | 1 | 4 | 4 | 75 |
| NE | 8 | Calumet | 2.58 | 1 | 3 | 4 | 58 |
| NC | 1 | Adams | 0.00 | 0 | 0 | 0 | 0 |
| NW | 2 | Ashland | 0.00 | 0 | 0 | 0 | 0 |
| NW | 4 | Bayfield | 0.00 | 0 | 0 | 0 | 0 |
| NW | 6 | Buffalo | 0.00 | 0 | 0 | 0 | 0 |
| NW | 7 | Burnett | 0.00 | 0 | 0 | 0 | 0 |
| SW | 12 | Crawford | 0.00 | 0 | 0 | 0 | 0 |
| NE | 15 | Door | 0.00 | 0 | 0 | 0 | 0 |
| NC | 19 | Florence | 0.00 | 0 | 0 | 0 | 0 |
| NC | 21 | Forest | 0.00 | 0 | 0 | 0 | 0 |
| SW | 23 | Green | 0.00 | 0 | 0 | 0 | 0 |
| NC | 24 | Green Lake | 0.00 | 0 | 0 | 0 | 0 |
| NC | 26 | Iron | 0.00 | 0 | 0 | 0 | 0 |
| NE | 31 | Kewaunee | 0.00 | 0 | 0 | 0 | 0 |
| NC | 34 | Langlade | 0.00 | 0 | 0 | 0 | 0 |
| NW | 46 | Pepin | 0.00 | 0 | 0 | 0 | 0 |
| NW | 47 | Pierce | 0.00 | 0 | 0 | 0 | 0 |
| NW | 48 | Polk | 0.00 | 0 | 0 | 0 | 0 |
| NC | 50 | Price | 0.00 | 0 | 0 | 0 | 0 |
| SW | 52 | Richland | 0.00 | 0 | 0 | 0 | 0 |
| NW | 54 | Rusk | 0.00 | 0 | 0 | 0 | 0 |
| NW | 57 | Sawyer | 0.00 | 0 | 0 | 0 | 0 |
| NW | 60 | Taylor | 0.00 | 0 | 0 | 0 | 0 |
| SW | 62 | Vernon | 0.00 | 0 | 0 | 0 | 0 |
| NC | 63 | Vilas | 0.00 | 0 | 0 | 0 | 0 |
| NC | 71 | Wood | 0.00 | 0 | 0 | 0 | 0 |
| NC | 73 | Menominee | 0.00 | 0 | 0 | 0 | 0 |

Table B-2 Summary of Nighttime Crash Data by County

(In descending order according to total length of freeway)

| Region | County | County ID | Freeway Segment Length (mi) | Total Nighttime Crashes (5-year period) | Nighttime Crashes on Freeway Segments (5-year period) | Nighttime Crashes on Interchange Segments (5-year period) | Nighttime Crashes on Ramps (5-year period) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SW | 13 | Dane | 124.70 | 2943 | 1540 | 950 | 453 |
| NC | 37 | Marathon | 86.51 | 1302 | 832 | 332 | 138 |
| SE | 40 | Milwaukee | 69.94 | 5791 | 2249 | 2686 | 856 |
| NE | 5 | Brown | 68.47 | 1614 | 770 | 590 | 254 |
| NW | 9 | Chippewa | 65.24 | 689 | 516 | 138 | 35 |
| NC | 49 | Portage | 60.33 | 564 | 350 | 165 | 49 |
| NE | 70 | Winnebago | 58.88 | 1530 | 720 | 615 | 195 |
| SE | 67 | Waukesha | 58.18 | 1920 | 856 | 767 | 297 |
| NC | 58 | Shawano | 55.30 | 801 | 705 | 83 | 13 |
| SE | 64 | Walworth | 50.07 | 600 | 393 | 153 | 54 |
| SW | 41 | Monroe | 46.21 | 583 | 484 | 93 | 6 |
| NE | 42 | Oconto | 43.06 | 281 | 218 | 46 | 17 |
| SE | 66 | Washington | 41.95 | 1063 | 572 | 372 | 119 |
| NW | 27 | Jackson | 41.16 | 453 | 408 | 38 | 7 |
| NW | 16 | Douglas | 41.07 | 237 | 201 | 27 | 9 |
| NW | 55 | St Croix | 40.15 | 822 | 655 | 112 | 55 |
| SW | 11 | Columbia | 39.29 | 616 | 425 | 138 | 53 |
| NE | 59 | Sheboygan | 38.21 | 642 | 406 | 199 | 37 |
| NW | 18 | Eau Claire | 37.95 | 726 | 461 | 184 | 81 |
| NE | 20 | Fond Du Lac | 37.92 | 595 | 348 | 208 | 39 |
| NW | 65 | Washburn | 37.89 | 258 | 233 | 17 | 8 |
| SW | 53 | Rock | 37.09 | 583 | 421 | 122 | 40 |
| NE | 36 | Manitowoc | 33.86 | 406 | 302 | 86 | 18 |
| NW | 3 | Barron | 33.64 | 146 | 129 | 14 | 3 |
| SW | 29 | Juneau | 33.57 | 396 | 337 | 50 | 9 |
| SW | 14 | Dodge | 33.09 | 456 | 323 | 104 | 29 |
| NC | 35 | Lincoln | 31.71 | 338 | 255 | 76 | 7 |
| SW | 32 | La Crosse | 31.50 | 658 | 337 | 249 | 72 |
| NW | 10 | Clark | 30.14 | 283 | 255 | 22 | 6 |
| NW | 17 | Dunn | 29.02 | 499 | 420 | 59 | 20 |
| SW | 25 | Iowa | 28.74 | 228 | 183 | 35 | 10 |
| SE | 45 | Ozaukee | 27.41 | 472 | 325 | 89 | 58 |
| NC | 68 | Waupaca | 26.25 | 486 | 345 | 121 | 20 |
| SW | 28 | Jefferson | 25.22 | 393 | 338 | 34 | 21 |
| NC | 39 | Marquette | 23.84 | 192 | 171 | 18 | 3 |
| NE | 44 | Outagamie | 22.76 | 695 | 286 | 264 | 145 |
| SW | 22 | Grant | 21.41 | 217 | 151 | 56 | 10 |
| NC | 69 | Waushara | 18.80 | 122 | 95 | 23 | 4 |
| NE | 38 | Marinette | 18.40 | 112 | 95 | 13 | 4 |
| SW | 56 | Sauk | 15.75 | 315 | 220 | 72 | 23 |
| SE | 51 | Racine | 12.24 | 340 | 167 | 142 | 31 |
| SE | 30 | Kenosha | 11.81 | 399 | 154 | 192 | 53 |
| SW | 33 | La Fayette | 11.35 | 96 | 90 | 4 | 2 |
| NC | 43 | Oneida | 6.35 | 29 | 29 | 0 | 0 |
| NW | 61 | Trempealeau | 4.18 | 75 | 64 | 4 | 7 |
| NE | 8 | Calumet | 2.58 | 58 | 24 | 14 | 20 |
| NC | 1 | Adams | 0.00 | 0 | 0 | 0 | 0 |
| NW | 2 | Ashland | 0.00 | 0 | 0 | 0 | 0 |
| NW | 4 | Bayfield | 0.00 | 0 | 0 | 0 | 0 |
| NW | 6 | Buffalo | 0.00 | 0 | 0 | 0 | 0 |
| NW | 7 | Burnett | 0.00 | 0 | 0 | 0 | 0 |
| SW | 12 | Crawford | 0.00 | 0 | 0 | 0 | 0 |
| NE | 15 | Door | 0.00 | 0 | 0 | 0 | 0 |
| NC | 19 | Florence | 0.00 | 0 | 0 | 0 | 0 |
| NC | 21 | Forest | 0.00 | 0 | 0 | 0 | 0 |
| SW | 23 | Green | 0.00 | 0 | 0 | 0 | 0 |
| NC | 24 | Green Lake | 0.00 | 0 | 0 | 0 | 0 |
| NC | 26 | Iron | 0.00 | 0 | 0 | 0 | 0 |
| NE | 31 | Kewaunee | 0.00 | 0 | 0 | 0 | 0 |
| NC | 34 | Langlade | 0.00 | 0 | 0 | 0 | 0 |
| NW | 46 | Pepin | 0.00 | 0 | 0 | 0 | 0 |
| NW | 47 | Pierce | 0.00 | 0 | 0 | 0 | 0 |
| NW | 48 | Polk | 0.00 | 0 | 0 | 0 | 0 |
| NC | 50 | Price | 0.00 | 0 | 0 | 0 | 0 |
| SW | 52 | Richland | 0.00 | 0 | 0 | 0 | 0 |
| NW | 54 | Rusk | 0.00 | 0 | 0 | 0 | 0 |
| NW | 57 | Sawyer | 0.00 | 0 | 0 | 0 | 0 |
| NW | 60 | Taylor | 0.00 | 0 | 0 | 0 | 0 |
| SW | 62 | Vernon | 0.00 | 0 | 0 | 0 | 0 |
| NC | 63 | Vilas | 0.00 | 0 | 0 | 0 | 0 |
| NC | 71 | Wood | 0.00 | 0 | 0 | 0 | 0 |
| NC | 73 | Menominee | 0.00 | 0 | 0 | 0 | 0 |

# Appendix C Top 100 Unlighted Segments Concerning Crash Risk

Table C-1 Top 100 Unlighted Freeway Segments Concerning Crash Risk (crash/mile/year)

| Risk Rank | Light Ben Rank | Area Type | County Name | Route Number | Predicted Crash Dens (crash/mile/year) | Actual Crash Dens (crash/mile/year) | From Interchange or Cross Rd | To Interchange or Cross Rd | Number of Lanes | Length (mi) | AADT | AADT Pctla | Curve | Lane Width (ft) | Left Shld Width (ft) | Right Shld Width (ft) | Med Width (ft) | AASHTO Standardb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Urban | Dane | US 12,14,18,151 | 8.03 | 8.60 | Fish Hatchery Rd | US 14 (Park St) | 6 | 0.349 | 137420 | 99.8 | Y | 12 | 7 | 8 | 24 | CFL |
| 2 | 8 | Urban | Dane | US 12,14,18,151 | 7.57 | 15.23 | US 18,151 (Verona Rd) | Seminole Hwy | 6 | 0.565 | 134450 | 99.6 | Y | 12 | 7 | 10 | 48 | CFL |
| 3 | 2 | Urban | Waukesha | I 94 | 7.09 | 12.10 | S Brookfield Rd | CTH O | 6 | 1.355 | 141000 | 100.0 | Y | 12 | 10 | 10 | 48 | CFL |
| 4 | 4 | Urban | Waukesha | I 94 | 6.95 | 10.18 | CTH Y (S Barker Rd) | S Brookfield Rd | 6 | 1.355 | 141000 | 100.0 | Y | 12 | 10 | 10 | 26 | CFL |
| 5 | 6 | Urban | Waukesha | I 94,ST 164 | 6.48 | 7.39 | CTH F (Redford Blvd) | CTH Y (S Barker Rd) | 6 | 1.597 | 127000 | 99.6 | Y | 12 | 10 | 10 | 15 | CFL |
| 6 | 11 | Urban | Dane | US 12,18 | 6.34 | 7.06 | US 14 (Park St) | Rimrock Rd | 6 | 0.482 | 126810 | 99.5 | N | 12 | 10 | 10 | 80 | CFL |
| 7 | 7 | Urban | Waukesha | I 94,ST 16 | 6.22 | 3.54 | STH 164 (Pewaukee Rd) | CTH F (Redford Blvd) | 6 | 0.508 | 122000 | 99.4 | Y | 12 | 10 | 12 | 26 | CFL |
| 8 | 3 | Urban | Dane | US 12,18 | 6.18 | 1.98 | John Nolen Dr | S Towne Dr | 6 | 0.404 | 134990 | 99.7 | Y | 12 | 12 | 12 | 100 | CFL |
| 9 | 20 | Urban | Dane | US 12,14 | 6.14 | 3.69 | Gammon Rd | Whitney Way | 4 | 1.245 | 79480 | 97.3 | Y | 12 | 7 | 10 | 80 | CFL |
| 10 | 16 | Urban | Waukesha | I 94 | 5.96 | 5.12 | STH 16 | STH 164 (Pewaukee Rd) | 6 | 0.547 | 122000 | 99.4 | N | 12 | 10 | 12 | 100 | CFL |
| 11 | 47 | Urban | Waukesha | I 94 | 5.88 | 2.53 | CTH SS | CTH G | 4 | 0.474 | 72500 | 96.3 | Y | 12 | 6 | 10 | 50 | CFL |
| 12 | 5 | Urban | Dane | US 12,18 | 5.85 | 2.05 | S Towne Dr | Monona Dr | 6 | 0.684 | 120050 | 99.2 | Y | 12 | 12 | 10 | 100 | CFL |
| 13 | 9 | Urban | Dane | US 12,18 | 5.79 | 2.51 | Monona Dr | US 51 (Stoughton Rd) | 6 | 0.239 | 120050 | 99.2 | N | 12 | 12 | 10 | 100 | CFL |
| 14 | 66 | Urban | Brown | US 41,ST 32 | 5.64 | 3.84 | STH 29 | STH 32 | 4 | 0.573 | 70300 | 96.0 | Y | 12 | 6 | 10 | 60 | CFL |
| 15 | 42 | Urban | Brown | US 41 | 5.50 | 6.44 | CTH C (Wilmot) | CTH Q | 4 | 0.683 | 64600 | 95.5 | Y | 12 | 6 | 10 | 48 | CFL |
| 16 | 83 | Urban | Dane | US 12,14 | 5.47 | 0.71 | Old Sauk Rd | Mineral Point Rd | 4 | 0.562 | 68100 | 95.7 | Y | 12 | 6 | 10 | 100 | CFL |
| 17 | 14 | Urban | Racine | I 94,US 41 | 5.43 | 3.31 | 7 Mile Rd | 5 1/2 Mile Rd | 6 | 0.968 | 109000 | 99.0 | N | 12 | 11 | 11 | 26 | CFL |
| 18 | 15 | Urban | Racine | I 94,US 41 | 5.36 | 2.68 | 5 1/2 Mile Rd | CTH K | 6 | 1.418 | 107000 | 98.9 | N | 12 | 11 | 11 | 26 | CFL |
| 19 | 79 | Rural | Winnebago | US 41 | 5.27 | 3.05 | CTH GG | STH 76 | 4 | 1.574 | 64000 | 95.3 | Y | 12 | 6 | 10 | 60 |  |
| 20 | 95 | Urban | Brown | US 41 | 5.26 | 2.69 | STHH 172 | S Oneida St | 4 | 0.372 | 64600 | 95.5 | N | 12 | 6 | 10 | 80 | CFL |
| 21 | 10 | Urban | Winnebago | US 41 | 5.25 | 9.05 | US 10 | CTH B | 6 | 0.685 | 87800 | 98.2 | Y | 12 | 10 | 8 | 48 | CFL |
| 22 | 17 | Urban | Dane | I 39,90 | 5.12 | 4.45 | I 94 | Cottage Grove Rd | 6 | 1.034 | 92570 | 98.8 | Y | 12 | 10 | 10 | 50 | CFL |
| 23 | 75 | Rural | Rock | I 39,90 | 5.08 | 3.49 | US 14 Bus | STH 11 | 4 | 1.031 | 57900 | 93.6 | N | 12 | 6 | 7 | 60 |  |
| 24 | 21 | Urban | Dane | I 39,90 | 5.07 | 4.02 | E Buckeye Rd | US 12,18 (Beltline Hwy) | 6 | 1.591 | 92570 | 98.8 | Y | 12 | 10 | 10 | 50 | CFL |
| 25 | 19 | Urban | Kenosha | I 94,US 41 | 5.03 | 4.93 | CTH Q | Illinois State Line | 6 | 1.502 | 96300 | 98.9 | N | 12 | 11 | 10 | 26 | CFL |
| 26 | 28 | Urban | Dane | I 39,90 | 5.01 | 2.65 | Cottage Grove Rd | E Buckeye Rd | 6 | 0.756 | 92570 | 98.8 | N | 12 | 10 | 10 | 50 | CFL |
| 27 | 45 | Urban | Outagamie | US 41 | 4.99 | 3.29 | CTH OO | CTH A | 4 | 1.275 | 55600 | 92.6 | Y | 12 | 6 | 10 | 60 | CFL |
| 28 | 122 | Urban | Brown | US 41 | 4.89 | 3.69 | US 141 | STHH 29 | 4 | 1.029 | 58600 | 93.8 | N | 12 | 6 | 10 | 60 | CFL |
| 29 | 123 | Urban | Outagamie | US 41 | 4.88 | 3.06 | STH 47 | CTH E | 4 | 1.568 | 58500 | 93.8 | N | 12 | 6 | 10 | 60 | CFL |
| 30 | 112 | Urban | Waukesha | I 94 | 4.88 | 4.45 | CTH C | STH 83 | 4 | 1.259 | 58900 | 93.9 | Y | 12 | 6 | 11 | 60 | CFL |
| 31 | 124 | Urban | Dane | US 12,14 | 4.87 | 0.44 | Greenway Blvd | Old Sauk Rd | 4 | 0.459 | 58280 | 93.7 | N | 12 | 6 | 10 | 50 | CFL |
| 32 | 22 | Rural | Kenosha | I 94,US 41 | 4.86 | 3.85 | STH 142 (Burlington Rd) | STH 158 | 6 | 1.144 | 91900 | 98.5 | N | 12 | 11 | 10 | 26 |  |
| 33 | 23 | Urban | Kenosha | I 94,US 41 | 4.86 | 3.60 | STH 158 | STH 50 (75th St) | 6 | 0.944 | 91900 | 98.5 | N | 12 | 11 | 10 | 100 | CFL |
| 34 | 96 | Urban | Dane | I 39,90 | 4.83 | 6.48 | US 12,18 (Beltline Hwy) | Siggelkow Rd | 4 | 1.234 | 56520 | 93.3 | Y | 12 | 6 | 10 | 60 | CFL |
| 35 | 107 | Urban | Dane | I 39,90 | 4.80 | 2.26 | Gaston Rd | Norton Dr | 4 | 1.591 | 56520 | 93.3 | Y | 12 | 6 | 10 | 60 | CFL |
| 36 | 114 | Urban | Dane | I 39,90 | 4.79 | 4.08 | Siggelkow Rd | CTH MN | 4 | 1.472 | 56520 | 93.3 | Y | 12 | 6 | 10 | 60 | CFL |
| 37 | 130 | Urban | Ozaukee | I 43,ST 32,57 | 4.72 | 3.17 | West Bonniwell Rd | West Dandelion Ln | 4 | 1.263 | 56500 | 93.1 | Y | 12 | 6 | 11 | 60 | CFL |
| 38 | 131 | Urban | Ozaukee | I 43,ST 32,57 | 4.72 | 2.85 | CTH OO | STH 57,167 | 4 | 1.263 | 56500 | 93.1 | Y | 12 | 6 | 11 | 70 | CFL |
| 39 | 40 | Urban | Dane | US 12,18 | 4.72 | 2.35 | US 51 (Stoughton Rd) | I 39,90 | 6 | 0.765 | 85500 | 97.8 | Y | 12 | 10 | 10 | 24 | CFL |
| 40 | 51 | Urban | Ozaukee | I 43,ST 32 | 4.71 | 2.72 | Donges Bay Rd | W County Line Rd | 4 | 0.956 | 62900 | 95.0 | Y | 12 | 9 | 11 | 50 | CFL |
| 41 | 81 | Urban | Brown | US 41 | 4.71 | 4.04 | CTH G (Main Ave) | CTH F (Scheuring Rd) | 4 | 1.434 | 52200 | 90.9 | Y | 12 | 6 | 8 | 48 | CFL |
| 42 | 142 | Urban | Outagamie | US 41 | 4.70 | 4.23 | W Liebau Rd | STHH 57 | 4 | 1.275 | 55600 | 92.6 | N | 12 | 6 | 10 | 48 | CFL |
| 43 | 18 | Urban | Dane | I 39,90,94 | 4.70 | 4.52 | US 151 (E Washington Ave) | I 94 | 6 | 1.592 | 90820 | 98.3 | N | 12 | 12 | 10 | 60 | CFL |
| 44 | 13 | Urban | Milwaukee | I 94,US 41 | 4.68 | 7.17 | W Puetz Rd | STHH 100 (W Ryan Rd) | 6 | 1.283 | 89700 | 98.2 | N | 12 | 13 | 8 | 48 | CFL |
| 45 | 157 | Urban | Ozaukee | I 43,ST 32,57 | 4.68 | 2.85 | West Dandelion Ln | W Liebau Rd | 4 | 1.263 | 56500 | 93.1 | N | 12 | 6 | 11 | 60 | CFL |
| 46 | 26 | Rural | Kenosha | I 94,US 41 | 4.66 | 2.83 | CTH E | STHH 142 (Burlington Rd) | 6 | 1.200 | 87000 | 98.1 | N | 12 | 11 | 10 | 26 |  |
| 47 | 180 | Urban | Brown | US 41 | 4.65 | 3.66 | STHH 32 | CTH VK (Lombardi Ave) | 6 | 0.600 | 73400 | 96.6 | N | 12 | 6 | 10 | 60 | CFL |
| 48 | 181 | Urban | Brown | US 41 | 4.65 | 5.80 | CTH VK (Lombardi Ave) | STHH 172 | 6 | 1.035 | 73400 | 96.6 | N | 12 | 6 | 10 | 48 | CFL |
| 49 | 148 | Urban | Rock | I 39,90 | 4.63 | 3.25 | US 14 (Humes Rd) | Mt Zion Ave | 4 | 1.355 | 54500 | 91.5 | N | 12 | 6 | 10 | 60 | CFL |
| 50 | 195 | Rural | Rock | I 39,90 | 4.62 | 2.51 | CTH DM | CTH V | 4 | 1.036 | 57900 | 93.6 | N | 12 | 6 | 13 | 60 |  |
| 51 | 149 | Urban | Outagamie | US 41 | 4.62 | 0.84 | CTH OO | CTH CE | 4 | 0.477 | 54300 | 91.0 | N | 12 | 6 | 10 | 100 | CFL |
| 52 | 32 | Urban | Ozaukee | I 43,ST 32 | 4.59 | 5.12 | STHH 57 | Donges Bay Rd | 4 | 0.899 | 62900 | 95.0 | Y | 12 | 10 | 12 | 60 | CFL |
| 53 | 35 | Urban | Racine | I 94,US 41 | 4.58 | 3.74 | CTH K | 2 Mile Rd | 6 | 1.444 | 86900 | 98.0 | N | 12 | 11 | 11 | 26 | CFL |
| 54 | 36 | Urban | Racine | I 94,US 41 | 4.58 | 5.12 | 2 Mile Rd | STHH 20 (Washington Ave) | 6 | 1.444 | 86900 | 98.0 | N | 12 | 11 | 11 | 60 | CFL |
| 55 | 100 | Urban | Outagamie | ST 441 | 4.56 | 5.46 | CTH OO (E Northland Ave) | CTH CE | 4 | 1.503 | 51700 | 90.5 | Y | 12 | 6 | 10 | 64 | CFL |
| 56 | 60 | Urban | Rock | I 39,90 | 4.56 | 3.19 | CTH A | STHH 47 | 4 | 0.314 | 49500 | 89.3 | Y | 12 | 6 | 10 | 60 | CFL |
| 57 | 102 | Urban | Winnebago | US 41 | 4.54 | 3.61 | STH 76 | CTH Y | 4 | 1.329 | 51400 | 90.1 | Y | 12 | 6 | 10 | 48 | CFL |
| 58 | 34 | Urban | Washington | US 41,45 | 4.54 | 6.45 | US 45 | STH 167 (Holy Hill Rd) | 6 | 0.992 | 80700 | 97.4 | Y | 12 | 10 | 12 | 60 | CFL |
| 59 | 49 | Urban | Winnebago | US 41 | 4.52 | 3.60 | CTH B | Oakridge Rd | 6 | 0.500 | 80700 | 97.4 | N | 12 | 10 | 10 | 26 | CFL |
| 60 | 33 | Urban | Milwaukee | I 94,US 41 | 4.52 | 5.63 | Winchester Rd | Main St | 6 | 0.923 | 83400 | 97.7 | Y | 12 | 11 | 10 | 48 | CFL |
| 61 | 62 | Rural | Dane | I 39,90 | 4.50 | 3.64 | CTH B | E Church Rd | 4 | 1.153 | 48700 | 88.9 | Y | 12 | 6 | 10 | 60 |  |
| 62 | 24 | Urban | Waukesha | I 94 | 4.48 | 4.87 | STHH 83 | CTH E | 4 | 1.521 | 62000 | 94.7 | Y | 12 | 11 | 11 | 60 | CFL |
| 63 | 27 | Urban | Winnebago | US 41 | 4.47 | 7.85 | CTH BB (W Prospect Ave) | US 10 | 6 | 1.324 | 75600 | 96.7 | Y | 12 | 10 | 10 | 26 | CFL |
| 64 | 12 | Urban | Milwaukee | I 94,US 41 | 4.46 | 5.48 | STHH 100 (W Ryan Rd) | W Oakwood Rd | 6 | 1.169 | 83400 | 97.7 | Y | 12 | 13 | 10 | 36 | CFL |
| 65 | 78 | Rural | Dane | I 39,90 | 4.45 | 3.02 | CTH N | Church St | 4 | 1.591 | 48700 | 88.9 | Y | 12 | 6 | 10 | 60 |  |
| 66 | 162 | Urban | Winnebago | US 41 | 4.43 | 1.66 | CTH Y | US 45 | 4 | 1.329 | 51400 | 90.1 | N | 12 | 6 | 10 | 100 | CFL |
| 67 | 52 | Urban | Winnebago | US 41 | 4.42 | 3.05 | Oakridge Rd | STHH 114 (Winneconne Ave) | 6 | 0.394 | 78400 | 97.2 | N | 12 | 10 | 10 | 26 | CFL |
| 68 | 86 | Rural | Dane | I 39,90 | 4.42 | 3.90 | Church St | Drotning Rd | 4 | 1.591 | 48700 | 88.9 | Y | 12 | 6 | 10 | 60 |  |
| 69 | 39 | Urban | Racine | I 94,US 41 | 4.42 | 4.92 | Main St | STHH 114 (Winneconne Ave) | 6 | 1.342 | 80900 | 97.5 | N | 12 | 11 | 10 | 26 | CFL |
| 70 | 210 | Urban | Rock | I 39,90 | 4.41 | 3.11 | E Milwaukee St | US 14 Bus | 4 | 1.349 | 54500 | 91.5 | N | 12 | 6 | 13 | 60 | CFL |
| 71 | 50 | Urban | Waukesha | I 43 | 4.40 | 3.04 | CTH O | S 124th St | 4 | 1.643 | 60500 | 94.4 | Y | 12 | 10 | 12 | 70 | CFL |
| 72 | 116 | Urban | Rock | I 39,90 | 4.39 | 1.82 | Colley Rd | Illinois State Line | 4 | 1.206 | 49300 | 89.1 | Y | 12 | 6 | 10 | 60 | CFL |
| 73 | 106 | Rural | Dane | I 39,90 | 4.38 | 4.48 | Drotning Rd | CTH W | 4 | 1.072 | 48700 | 88.9 | Y | 12 | 6 | 10 | 60 |  |
| 74 | 43 | Urban | Washington | US 41,45,ST 167 | 4.38 | 9.25 | STH 167 (Holy Hill Rd) | CTH F (Freistadt Rd) | 6 | 1.362 | 77100 | 97.1 | Y | 12 | 10 | 12 | 60 | CFL |
| 75 | 55 | Urban | Washington | US 41,45,ST 167 | 4.37 | 6.31 | CTH F (Freistadt Rd) | STH 167 (Lannon Rd) | 6 | 1.362 | 77100 | 97.1 | N | 12 | 10 | 10 | 24 | CFL |
| 76 | 90 | Urban | Waukesha | I 94 | 4.37 | 2.41 | CTH G | CTH T | 6 | 1.080 | 73200 | 96.4 | Y | 12 | 8 | 12 | 22 | CFL |
| 77 | 91 | Rural | Rock | I 39,90 | 4.36 | 4.89 | STH 59 | E Knutson Rd | 4 | 1.553 | 47700 | 88.2 | Y | 12 | 6 | 10 | 60 |  |
| 78 | 87 | Urban | Brown | US 41,141 | 4.34 | 5.05 | N Lakeview Dr | I 43 | 4 | 0.832 | 47200 | 86.9 | Y | 12 | 6 | 10 | 60 | CFL |
| 79 | 25 | Urban | Waukesha | I 43 | 4.34 | 1.57 | S Calhoun Rd | CTH O | 4 | 1.145 | 54500 | 91.5 | Y | 12 | 10 | 10 | 100 | CFL |
| 80 | 59 | Urban | Dane | US 12,14 | 4.33 | 6.79 | US 14 (University Ave) | Greenway Blvd | 4 | 0.265 | 55120 | 92.0 | N | 12 | 9 | 10 | 48 | CFL |
| 81 | 109 | Rural | Rock | I 39,90 | 4.31 | 3.21 | CTH M | N Newville Rd | 4 | 1.560 | 47700 | 88.2 | Y | 12 | 6 | 10 | 60 |  |
| 82 | 57 | Urban | Outagamie | US 41 | 4.30 | 10.42 | STHH 125 (W College Ave) | W Prospect Ave | 6 | 0.806 | 75600 | 96.7 | N | 12 | 10 | 10 | 26 | CFL |
| 83 | 170 | Urban | Rock | I 39,90 | 4.30 | 3.15 | I 43 | Colley Rd | 4 | 1.206 | 49300 | 89.1 | Y | 12 | 6 | 10 | 60 | CFL |
| 84 | 173 | Rural | Dane | I 39,90 | 4.26 | 5.39 | CTH W | CTH B | 4 | 1.150 | 48700 | 88.9 | N | 12 | 6 | 10 | 60 |  |
| 85 | 133 | Urban | Outagamie | US 41 | 4.25 | 2.82 | STH 55 | Rose Hill Rd | 4 | 0.852 | 47300 | 87.2 | Y | 12 | 6 | 10 | 60 | CFL |
| 86 | 175 | Urban | Winnebago | US 10,ST 441 | 4.25 | 13.63 | Racine Rd | Midway Rd | 4 | 0.382 | 48600 | 88.3 | N | 12 | 6 | 10 | 38 | CFL |
| 87 | 44 | Urban | Waukesha | I 94 | 4.25 | 4.67 | CTH P | CTH C | 4 | 1.456 | 55500 | 92.4 | Y | 12 | 10 | 11 | 60 | CFL |
| 88 | 105 | Rural | Brown | US 41 | 4.23 | 3.55 | Little Rapids Rd | CTH S | 4 | 1.182 | 46000 | 86.7 | Y | 12 | 6 | 10 | 60 |  |
| 89 | 159 | Rural | Rock | I 39,90 | 4.22 | 2.99 | CTH J | CTH S | 4 | 1.539 | 47400 | 87.5 | Y | 12 | 6 | 10 | 60 |  |
| 90 | 48 | Urban | Waukesha | I 43 | 4.21 | 2.62 | CTH Y | S Calhoun Rd | 4 | 1.145 | 54500 | 91.5 | Y | 12 | 10 | 10 | 70 | CFL |
| 91 | 158 | Rural | Brown | US 41 | 4.21 | 1.07 | Golden Glow Rd | CTH U | 4 | 1.314 | 46000 | 86.7 | N | 12 | 6 | 8 | 60 |  |
| 92 | 174 | Rural | Rock | I 39,90 | 4.20 | 5.64 | STH 26 | E M H Townline Rd | 4 | 1.560 | 47700 | 88.2 | Y | 12 | 6 | 10 | 60 |  |
| 93 | 74 | Urban | Winnebago | US 10,ST 441 | 4.20 | 3.94 | Midway Rd | STH 47 | 4 | 0.456 | 43900 | 85.9 | Y | 12 | 6 | 10 | 48 | CFL |
| 94 | 182 | Rural | Rock | I 39,90 | 4.19 | 2.05 | E Manogue Rd | CTH M | 4 | 1.560 | 47700 | 88.2 | N | 12 | 6 | 10 | 60 |  |
| 95 | 183 | Rural | Rock | I 39,90 | 4.19 | 3.08 | E Manogue Rd | E M H Townline Rd | 4 | 1.560 | 47700 | 88.2 | N | 12 | 6 | 10 | 60 |  |
| 96 | 184 | Rural | Rock | I 39,90 | 4.17 | 3.07 | STH 11 | E Woodman Rd | 4 | 1.562 | 47400 | 87.5 | N | 12 | 6 | 10 | 60 |  |
| 97 | 185 | Rural | Rock | I 39,90 | 4.17 | 2.82 | E Woodman Rd | CTH J | 4 | 1.562 | 47400 | 87.5 | N | 12 | 6 | 10 | 60 |  |
| 98 | 31 | Urban | Sauk | I 90,94 | 4.17 | 3.82 | STH 13 | Trout Rd | 4 | 0.995 | 36800 | 80.6 | Y | 12 | 6 | 5 | 100 | CFL |
| 99 | 186 | Urban | Outagamie | US 41 | 4.16 | 2.35 | CTH N | Rose Hill Rd | 4 | 0.852 | 47300 | 87.2 | N | 12 | 6 | 10 | 60 | CFL |
| 100 | 187 | Urban | Outagamie | US 41 | 4.16 | 3.16 | Holland Rd | STH 441 | 4 | 0.887 | 47300 | 87.2 | N | 12 | 6 | 10 | 48 | CFL |

a. AADT Pctl refer to the percentile of AADT of a particular segment compared with all segments of its type in Wisconsin.

b. AASHTO Standard refer to which lighting type is warranted based on AASHTO criteria. The lighting types include CFL (Continuous Freeway Lighting), CIL (Complete Interchange Lighting) and PIL (Partial Interchange Lighting). Blank indicates lighting is not warranted.

c. The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 4.10 |  | 1.40 |  |
|  | 2.92 | 4.09 | 1.03 | 1.39 |
|  | 2.10 | 2.91 | 0.78 | 1.02 |
|  | 1.45 | 2.09 | 0.57 | 0.77 |
|  |  | 1.44 |  | 0.56 |

Table C-2 Top 100 Unlighted Interchange Segments Concerning Crash Risk (crashes/mile/year)

| Risk Rank | Light Ben Rank | Area Type | County Name | On Road | Crossing Road | Interchange Type | Predicted Crash Dens (crash/mile/year) | Actual Crash Dens (crash/mile/year) | Number of Lanes | Length (mi) | AADT | AADT Pctla | Curve | Lane Width (ft) | Left Shld Width (ft) | Right Shld Width (ft) | Med Width (ft) | AASHTO Standardb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Urban | Dane | US 12,18 | John Nolen Dr | Trumpet | 25.27 | 26.94 | 6 | 0.267 | 119890 | 98.8 | Y | 12 | 10 | 10 | 24 | CFL |
| 2 | 3 | Urban | Waukesha | I 94 | CT F (Redford Blvd) | Diamond | 24.10 | 21.84 | 6 | 0.403 | 122000 | 99.3 | Y | 12 | 10 | 12 | 26 | CFL |
| 3 | 9 | Urban | Dane | US 12,18 | S Towne Dr | Diamond | 21.90 | 6.48 | 6 | 0.494 | 134990 | 100.0 | Y | 12 | 10 | 10 | 24 | CFL |
| 4 | 1 | Urban | Waukesha | I 94 | ST 164 (Pewaukee Rd) | Diamond | 21.84 | 20.71 | 6 | 0.357 | 122000 | 99.3 | Y | 12 | 7 | 10 | 420 | CFL |
| 5 | 10 | Urban | Dane | US 12,18 | Rimrock Rd | Diamond | 20.43 | 19.09 | 6 | 0.346 | 126810 | 99.8 | N | 12 | 10 | 10 | 24 | CFL |
| 6 | 5 | Urban | Dane | US 12,18 | Monona Dr | Diamond | 20.37 | 7.48 | 6 | 0.589 | 126200 | 99.5 | N | 12 | 12 | 10 | 100 | CFL |
| 7 | 4 | Urban | Winnebago | US 41 | ST 114 (Winneconne Ave) | Diamond | 20.10 | 12.25 | 6 | 0.604 | 78400 | 94.6 | Y | 12 | 10 | 10 | 26 | CFL |
| 8 | 7 | Urban | Winnebago | US 41 | CT II (Winchester Rd) | Diamond | 18.90 | 47.53 | 6 | 0.454 | 87800 | 96.7 | Y | 12 | 10 | 10 | 26 | CFL |
| 9 | 6 | Urban | Outagamie | US 41 | CT BB (W Prospect Ave) | Diamond | 18.12 | 18.66 | 6 | 0.418 | 75600 | 93.4 | Y | 12 | 10 | 10 | 26 | CFL |
| 10 | 8 | Urban | Dane | US 12,18 | US 51 (Stoughton Rd) | Diamond | 18.06 | 7.69 | 6 | 0.572 | 85500 | 96.0 | Y | 12 | 6 | 11 | 60 | CFL |
| 11 | 12 | Urban | Kenosha | I 94,US 41 | ST 165 | ParClo | 17.00 | 8.48 | 6 | 0.991 | 96300 | 98.1 | N | 12 | 11 | 11 | 26 | CFL |
| 12 | 17 | Rural | Racine | I 94,US 41 | 7 Mile Rd | Diamond | 16.47 | 8.19 | 6 | 0.537 | 109000 | 98.6 | N | 12 | 11 | 10 | 60 | CIL |
| 13 | 16 | Urban | Waukesha | I 94 | CT G | Diamond | 16.37 | 8.39 | 4 | 0.453 | 72500 | 92.5 | Y | 12 | 6 | 10 | 50 | CFL |
| 14 | 27 | Rural | Racine | I 94,US 41 | CT G | Diamond | 16.27 | 8.20 | 6 | 0.512 | 107000 | 98.4 | N | 12 | 11 | 11 | 26 |  |
| 15 | 11 | Urban | Waukesha | I 94 | ST 16 | Three Leg Directional | 16.11 | 8.77 | 6 | 0.547 | 88900 | 96.9 | N | 12 | 10 | 12 | 60 | CFL |
| 16 | 14 | Urban | Dane | I 39,90,94 | I 94 | All Directional | 15.55 | 4.99 | 6 | 0.641 | 92570 | 97.7 | Y | 12 | 10 | 12 | 60 | CFL |
| 17 | 28 | Urban | Dane | I 39,90,94 | High Crossing Blvd | Half-Diamond | 15.51 | 9.01 | 6 | 0.244 | 90820 | 97.2 | Y | 12 | 12 | 8 | 38 | CFL |
| 18 | 15 | Urban | Milwaukee | I 94,US 41 | ST 100 (W Ryan Rd) | Diamond | 15.44 | 17.37 | 6 | 0.461 | 83400 | 95.8 | N | 12 | 13 | 10 | 36 | CFL |
| 19 | 20 | Urban | Winnebago | US 41 | Oakridge Rd (Main St) | ParClo | 15.10 | 9.77 | 6 | 0.307 | 80700 | 95.1 | N | 12 | 10 | 10 | 26 | CFL |
| 20 | 22 | Urban | Washington | US 41,45 | ST 167 (Holly Hill Rd) | Diamond | 15.10 | 14.74 | 6 | 0.421 | 80700 | 95.1 | N | 12 | 10 | 10 | 24 | CFL |
| 21 | 37 | Rural | Kenosha | I 94,US 41 | ST 142 (Burlington Rd) | Diamond | 15.07 | 9.41 | 6 | 0.659 | 95400 | 97.9 | N | 12 | 11 | 10 | 26 | CIL |
| 22 | 30 | Rural | Kenosha | I 94,US 41 | ST 158 (52nd St) | Diamond | 14.70 | 6.97 | 6 | 0.545 | 91900 | 97.4 | N | 12 | 11 | 10 | 60 | CIL |
| 23 | 13 | Urban | Washington | US 41,45 | ST 167 (Lannon Rd) | Diamond | 14.65 | 14.01 | 6 | 0.499 | 77100 | 94.1 | N | 12 | 10 | 12 | 60 | CFL |
| 24 | 23 | Urban | Kenosha | I 94,US 41 | ST 50 (75th St) | Diamond | 14.58 | 12.98 | 6 | 0.601 | 76600 | 93.9 | N | 12 | 11 | 10 | 26 | CFL |
| 25 | 25 | Urban | Winnebago | US 41 | US 10,ST 441 | ParClo | 14.45 | 17.32 | 6 | 0.866 | 75600 | 93.4 | N | 12 | 10 | 10 | 26 | CFL |
| 26 | 18 | Urban | Waukesha | I 94 | CT SS (Prospect Ave) | Diamond | 14.42 | 9.54 | 4 | 0.399 | 62000 | 89.2 | Y | 12 | 11 | 11 | 60 | CFL |
| 27 | 41 | Rural | Kenosha | I 94,US 41 | CT E (12th St) | Diamond | 14.17 | 11.23 | 6 | 0.570 | 87000 | 96.5 | N | 12 | 11 | 10 | 26 | CIL |
| 28 | 42 | Rural | Racine | I 94,US 41 | CT K | Diamond | 14.16 | 7.25 | 6 | 0.745 | 86900 | 96.2 | N | 12 | 11 | 10 | 26 | CIL |
| 29 | 19 | Urban | Waukesha | I 94 | CT T (N Grandview Blvd) | Diamond | 14.15 | 9.02 | 6 | 0.488 | 73200 | 92.7 | N | 12 | 10 | 12 | 22 | CFL |
| 30 | 24 | Urban | Brown | ST 172 | S Webster Ave | Diamond | 14.09 | 9.29 | 6 | 0.732 | 77400 | 94.4 | Y | 12 | 10 | 10 | 60 | CFL |
| 31 | 26 | Urban | Ozaukee | I 43,ST 32,57 | ST 57 (Mequon Rd) | Diamond | 14.03 | 7.65 | 4 | 0.340 | 56500 | 86.9 | Y | 12 | 6 | 11 | 50 | CFL |
| 32 | 29 | Urban | Waukesha | I 94 | ST 83 | Diamond | 13.97 | 20.74 | 4 | 0.482 | 58900 | 88.5 | N | 12 | 6 | 11 | 60 | CFL |
| 33 | 21 | Urban | Dane | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | 13.51 | 5.16 | 6 | 0.621 | 67670 | 90.6 | Y | 12 | 10 | 10 | 60 | CFL |
| 34 | 45 | Rural | Racine | I 94,US 41 | ST 11 (Durand Ave) | ParClo | 13.50 | 4.84 | 6 | 0.248 | 80900 | 95.5 | N | 12 | 11 | 10 | 26 | CIL |
| 35 | 46 | Rural | Racine | I 94,US 41 | CT KR (1st Ave) | Diamond | 13.50 | 4.25 | 6 | 0.518 | 80900 | 95.5 | N | 12 | 11 | 10 | 26 | CIL |
| 36 | 31 | Urban | Waukesha | I 94 | CT C (Genesee St) | ParClo | 13.46 | 11.42 | 4 | 0.455 | 55500 | 86.2 | Y | 12 | 6 | 11 | 60 | CFL |
| 37 | 32 | Urban | Winnebago | US 10,ST 441 | Racine Rd | Trumpet | 13.25 | 13.61 | 4 | 0.485 | 48600 | 81.0 | Y | 12 | 6 | 10 | 38 | CFL |
| 38 | 48 | Rural | Kenosha | I 94,US 41 | CT C (Wilmot Rd) | Diamond | 13.01 | 8.79 | 6 | 0.660 | 76600 | 93.9 | N | 12 | 11 | 10 | 26 | CIL |
| 39 | 44 | Urban | Dane | I 39,90,94 | US 51 (Stoughton Rd) | ParClo | 12.69 | 6.41 | 6 | 0.530 | 70040 | 91.8 | Y | 12 | 10 | 10 | 38 | CFL |
| 40 | 82 | Urban | Brown | US 41 | CT VK (Hazelwood Ln) | Diamond | 12.66 | 16.04 | 4 | 0.449 | 73400 | 93.0 | N | 12 | 6 | 10 | 60 | CFL |
| 41 | 35 | Urban | Outagamie | US 41 | ST 125 (W College Ave) | Diamond | 12.55 | 7.09 | 6 | 0.395 | 61200 | 89.0 | N | 12 | 10 | 10 | 26 | CFL |
| 42 | 85 | Urban | Brown | US 41 | ST 32,54 (W Mason St) | Diamond | 12.32 | 11.38 | 4 | 0.387 | 70300 | 92.0 | N | 12 | 6 | 10 | 60 | CFL |
| 43 | 86 | Urban | Winnebago | US 41 | Breezewood Ln (W Bell St) | Diamond | 12.20 | 19.84 | 4 | 0.363 | 69200 | 91.1 | N | 12 | 6 | 10 | 60 | CFL |
| 44 | 49 | Rural | Racine | I 94,US 41 | ST 20 (Washington Ave) | Diamond | 12.17 | 10.32 | 6 | 0.950 | 69300 | 91.3 | N | 12 | 11 | 11 | 26 | CIL |
| 45 | 39 | Urban | Outagamie | US 41 | ST 96 (W Wisconsin Ave) | Diamond | 12.09 | 17.87 | 6 | 0.291 | 57900 | 87.6 | N | 12 | 10 | 10 | 26 | CFL |
| 46 | 156 | Urban | Dane | US 12,14 | Old Sauk Rd | Diamond | 12.08 | 5.96 | 4 | 0.570 | 68100 | 90.8 | N | 12 | 6 | 8 | 60 | CFL |
| 47 | 165 | Rural | Winnebago | US 41 | ST 76 (Jackson St) | Diamond | 11.85 | 25.78 | 4 | 0.489 | 64000 | 89.7 | N | 12 | 6 | 10 | 20 |  |
| 48 | 88 | Urban | Brown | US 41 | ST 172 ( Airport Dr) | ParClo | 11.84 | 9.32 | 4 | 0.901 | 65900 | 90.4 | N | 12 | 6 | 10 | 60 | CFL |
| 49 | 40 | Urban | Dane | US 12,14 | Mineral Point Rd | Diamond | 11.68 | 2.31 | 6 | 0.346 | 62150 | 89.4 | Y | 12 | 7 | 12 | 60 | CFL |
| 50 | 33 | Urban | Dane | US 151 | American Pkwy | ParClo | 11.61 | 6.28 | 6 | 0.573 | 54470 | 84.5 | N | 12 | 10 | 14 | 16 | CFL |
| 51 | 36 | Urban | Fond Du Lac | US 41 | S Military Rd | Diamond | 11.48 | 15.99 | 4 | 0.400 | 34500 | 67.4 | Y | 12 | 6 | 10 | 15 | CFL |
| 52 | 51 | Rural | Ozaukee | I 43,ST 32,57 | CT C (Pioneer Rd) | Diamond | 11.47 | 12.84 | 4 | 0.280 | 47600 | 80.5 | Y | 12 | 6 | 11 | 48 | CIL |
| 53 | 43 | Urban | Ozaukee | I 43,ST 32,57 | ST 60 (Washington St) | Diamond | 11.33 | 5.88 | 4 | 0.374 | 42000 | 76.3 | Y | 12 | 6 | 11 | 60 | CFL |
| 54 | 34 | Urban | Dane | US 151 | Reiner Rd | Diamond | 11.02 | 9.95 | 6 | 0.483 | 50380 | 82.6 | N | 12 | 10 | 12 | 60 | CFL |
| 55 | 47 | Urban | Fond Du Lac | US 41 | US 151 | Diamond | 11.01 | 7.81 | 4 | 0.666 | 38200 | 72.1 | Y | 12 | 10 | 8 | 50 | CFL |
| 56 | 96 | Urban | Brown | US 41 | ST-29 (Shawano Ave) | Diamond | 11.01 | 15.75 | 4 | 0.584 | 58600 | 88.3 | N | 12 | 6 | 10 | 60 | CFL |
| 57 | 97 | Urban | Outagamie | US 41 | ST 47 (N Richmond St) | Diamond | 11.00 | 6.20 | 4 | 0.451 | 58500 | 88.0 | N | 12 | 6 | 10 | 60 | CFL |
| 58 | 98 | Urban | Dane | US 12,14 | Greenway Blvd | Diamond | 10.97 | 4.28 | 4 | 0.560 | 58280 | 87.8 | Y | 12 | 6 | 10 | 60 | CFL |
| 59 | 55 | Urban | Waukesha | I 43 | CT O (Moorland Rd) | ParClo | 10.90 | 12.98 | 4 | 0.586 | 60500 | 88.7 | N | 12 | 13 | 16 | 70 | CFL |
| 60 | 52 | Urban | Brown | ST 172 | CT GV (Monroe Rd) | Diamond | 10.71 | 5.20 | 6 | 0.616 | 54700 | 85.2 | Y | 12 | 6 | 10 | 60 | CFL |
| 61 | 142 | Urban | Brown | US 41 | CT F (Scheuring Rd) | Diamond | 10.62 | 13.28 | 4 | 0.542 | 52200 | 83.8 | N | 12 | 6 | 8 | 48 | CFL |
| 62 | 79 | Urban | Waukesha | I 43 | CT Y (S Racine Ave) | Diamond | 10.53 | 6.03 | 4 | 0.465 | 54500 | 85.0 | Y | 12 | 10 | 10 | 60 | CFL |
| 63 | 101 | Urban | Rock | I 39,90 | US 14 Bus (E Racine St) | Cloverleaf | 10.53 | 5.31 | 4 | 0.452 | 54500 | 85.0 | N | 12 | 6 | 10 | 60 | CFL |
| 64 | 102 | Urban | Outagamie | US 41 | CT E (N Ballard Rd) | Diamond | 10.50 | 5.66 | 4 | 0.353 | 54300 | 84.3 | N | 12 | 6 | 10 | 60 | CFL |
| 65 | 50 | Rural | Dane | I 39,90,94 | CT V | Diamond | 10.45 | 4.90 | 6 | 0.449 | 55200 | 85.9 | N | 12 | 12 | 12 | 32 | CIL |
| 66 | 38 | Urban | Fond Du Lac | US 41 | CT VW (S Hickory St) | Diamond | 10.41 | 9.41 | 4 | 0.340 | 36600 | 69.7 | N | 12 | 8 | 14 | 60 | CFL |
| 67 | 56 | Rural | Washington | US 41 | ST 140 (Pioneer Rd) | Diamond | 10.38 | 8.38 | 4 | 0.453 | 42900 | 77.5 | N | 12 | 9 | 12 | 60 | CIL |
| 68 | 103 | Urban | Winnebago | US 41 | W 9th Ave | Diamond | 10.33 | 10.24 | 6 | 0.371 | 65200 | 90.1 | N | 12 | 8 | 8 | 48 | CFL |
| 69 | 63 | Urban | Outagamie | US 41 | ST 15,CT OO (W Northland Ave) | ParClo | 10.29 | 10.72 | 6 | 0.485 | 55600 | 86.4 | Y | 12 | 10 | 10 | 26 | CFL |
| 70 | 69 | Urban | Brown | ST 172 | Pilgrim Way/Van Der Perren Way | Diamond | 10.25 | 5.16 | 6 | 0.388 | 64100 | 89.9 | Y | 12 | 10 | 10 | 60 | CFL |
| 71 | 59 | Urban | Brown | I 43 | E Shore Dr (N Webster Ave) | ParClo | 10.23 | 7.13 | 4 | 0.477 | 40100 | 74.4 | N | 12 | 6 | 10 | 64 | CFL |
| 72 | 78 | Urban | Waukesha | I 94 | CT P | Diamond | 10.16 | 5.97 | 4 | 0.536 | 51500 | 83.1 | N | 12 | 8 | 11 | 60 | CFL |
| 73 | 109 | Urban | Outagamie | ST 441 | CT CE (E College Ave) | Diamond | 10.07 | 7.18 | 4 | 0.473 | 51700 | 83.3 | Y | 12 | 6 | 10 | 64 | CFL |
| 74 | 93 | Urban | Brown | US 41,141 | CT B (Sunset Beach Rd) | Diamond | 9.98 | 12.99 | 4 | 0.477 | 47200 | 79.6 | Y | 12 | 6 | 10 | 48 | CFL |
| 75 | 77 | Urban | Marathon | US 51, ST 29 | CT NN (N Mt Rd) | Diamond | 9.98 | 11.32 | 4 | 0.371 | 50000 | 81.9 | N | 12 | 6 | 12 | 60 | CFL |
| 76 | 106 | Urban | Rock | I 39,90 | US 14 (Humes Rd) | ParClo | 9.92 | 10.40 | 4 | 0.442 | 49500 | 81.7 | Y | 12 | 6 | 10 | 60 | CFL |
| 77 | 66 | Rural | St Croix | I 94,US 12 | US 35 | Trumpet | 9.78 | 8.22 | 6 | 0.413 | 71200 | 92.3 | N | 12 | 13 | 15 | 60 | CIL |
| 78 | 240 | Rural | Dane | I 39,90 | CT N | Diamond | 9.73 | 6.43 | 4 | 0.529 | 56520 | 87.3 | N | 12 | 6 | 10 | 60 | CIL |
| 79 | 110 | Urban | Rock | I 39,90 | ST 26 (Milton Ave) | ParClo | 9.69 | 7.05 | 4 | 0.454 | 47700 | 80.8 | N | 12 | 6 | 10 | 60 | CFL |
| 80 | 191 | Rural | Dane | I 39,90,94 | ST 19 | Diamond | 9.67 | 7.48 | 6 | 0.561 | 70040 | 91.8 | N | 12 | 10 | 10 | 38 | CIL |
| 81 | 58 | Rural | Washington | US 41,45 | US 45 | Three Leg Directional | 9.65 | 6.32 | 6 | 0.918 | 42900 | 77.5 | Y | 12 | 10 | 10 | 24 | CIL |
| 82 | 111 | Urban | Outagamie | US 41 | CT N (Freedom Rd) | Diamond | 9.64 | 7.55 | 4 | 0.344 | 47300 | 80.0 | N | 12 | 6 | 10 | 60 | CFL |
| 83 | 112 | Urban | Outagamie | US 41 | ST 441 | Trumpet | 9.64 | 7.97 | 4 | 0.477 | 47300 | 80.0 | N | 12 | 6 | 10 | 60 | CFL |
| 84 | 71 | Urban | Dane | ST 30 | Fair Oaks Ave | Diamond | 9.63 | 5.11 | 4 | 0.313 | 38920 | 73.0 | N | 12 | 5 | 12 | 20 | CFL |
| 85 | 175 | Urban | Brown | US 41,141 | CT M (Lineville Rd) | Diamond | 9.63 | 6.42 | 4 | 0.498 | 47200 | 79.6 | N | 12 | 6 | 8 | 60 | CFL |
| 86 | 64 | Rural | Fond Du Lac | US 41 | CT B | Diamond | 9.56 | 8.57 | 4 | 0.583 | 37600 | 71.6 | N | 12 | 6 | 10 | 60 | CIL |
| 87 | 241 | Rural | Rock | I 39,90 | ST 11 (E Avalon Rd) | Diamond | 9.56 | 2.39 | 4 | 0.502 | 54900 | 85.4 | N | 12 | 6 | 10 | 60 | CIL |
| 88 | 61 | Rural | St Croix | I 94,US 12 | US 12 | Diamond | 9.24 | 5.63 | 6 | 0.355 | 53600 | 84.0 | Y | 12 | 14 | 14 | 50 | CIL |
| 89 | 74 | Urban | Waukesha | I 94 | ST 67 (Summit Ave) | ParClo | 9.22 | 7.95 | 4 | 0.503 | 42200 | 76.5 | Y | 12 | 11 | 11 | 60 | CFL |
| 90 | 53 | Urban | Dane | US 151 | Windsor St | Diamond | 9.22 | 3.06 | 4 | 0.589 | 30100 | 56.6 | Y | 12 | 10 | 10 | 60 | CFL |
| 91 | 122 | Urban | Winnebago | US 10,ST 441 | ST 47 (Appleton Rd) | Diamond | 9.17 | 6.13 | 4 | 0.490 | 44500 | 78.4 | N | 12 | 6 | 10 | 64 | CFL |
| 92 | 60 | Urban | Winnebago | US 41 | US 10,ST 441 | ParClo | 9.13 | 13.79 | 4 | 0.580 | 24900 | 48.6 | N | 13 | 4 | 8 | 24 | CIL |
| 93 | 124 | Urban | Winnebago | US 10,ST 441 | Midway Rd | Diamond | 9.10 | 12.34 | 4 | 0.519 | 43900 | 78.2 | Y | 12 | 6 | 10 | 64 | CFL |
| 94 | 120 | Urban | Outagamie | US 41 | CT J (Hyland Ave) | Diamond | 9.05 | 8.93 | 4 | 0.359 | 42700 | 77.0 | N | 12 | 6 | 10 | 60 | CFL |
| 95 | 54 | Urban | Dane | US 151 | W Main St | Diamond | 9.02 | 5.99 | 6 | 0.401 | 36960 | 70.9 | Y | 12 | 12 | 12 | 12 | CFL |
| 96 | 57 | Urban | Dane | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | 8.95 | 2.69 | 6 | 0.595 | 38950 | 73.2 | N | 12 | 10 | 10 | 60 | CFL |
| 97 | 242 | Rural | Dane | I 39,90 | US 51 | Trumpet | 8.87 | 5.90 | 4 | 0.543 | 48700 | 81.2 | N | 12 | 6 | 10 | 60 |  |
| 98 | 90 | Urban | Marathon | US 51 | ST 52 | Diamond | 8.86 | 1.83 | 4 | 0.546 | 34800 | 68.1 | N | 12 | 6 | 10 | 25 | CFL |
| 99 | 92 | Urban | Brown | US 41 | US 141 (Velp Ave) | Diamond | 8.80 | 9.67 | 6 | 0.372 | 51300 | 82.9 | N | 12 | 6 | 10 | 60 | CFL |
| 100 | 125 | Urban | Rock | I 39,90 | I 43,ST 81 | Cloverleaf | 8.80 | 11.86 | 4 | 0.422 | 40800 | 75.1 | N | 12 | 6 | 10 | 60 | CFL |

a. AADT Pctl refer to the percentile of AADT of a particular segment compared with all segments of its type in Wisconsin.

b. AASHTO Standard refer to which lighting type is warranted based on AASHTO criteria. The lighting types include CFL (Continuous Freeway Lighting), CIL (Complete Interchange Lighting) and PIL (Partial Interchange Lighting). Blank indicates lighting is not warranted.

c. The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 17.00 |  | 7.51 |  |
|  | 11.01 | 16.99 | 4.97 | 7.50 |
|  | 7.42 | 11.00 | 3.04 | 4.96 |
|  | 4.96 | 7.41 | 1.56 | 3.03 |
|  |  | 4.95 |  | 1.55 |

Table C-3 Top 100 Unlighted Ramp Segments Concerning Crash Risk (crashes/mile/year)

| Risk Rank | Light Ben Rank | Area Type | County Name | On Road | Crossing Road | Direction | Interchange Type | Ramp Type | Predicted Crash Dens (crash/mile/year) | Actual Crash Dens (crash/mile/year) | Number of Lanes | Length (mi) | AADT | AADT Pctla | Lane Width (ft) | Left Shld Width (ft) | Right Shld Width (ft) | AASHTO Standardb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Urban | Milwaukee | I 94,US 41 | ST 100 (W Ryan Rd) | SB Exit | Diamond | Diamond | 6.83 | 15.92 | 1 | 0.251 | 24200 | 100.0 | 17 | 2 | 6 | CIL |
| 2 | 2 | Urban | Dane | I 39,90,94 | US 151 (E Washington Ave) | SB Entry | Cloverleaf | Free Flow Loop | 5.40 | 1.42 | 1 | 0.281 | 16000 | 99.6 | 20 | 5 | 6 | CIL |
| 3 | 3 | Urban | Brown | US 41 | ST-29 (Shawano Ave) | SB Entry | Diamond | Diamond | 4.59 | 22.70 | 1 | 0.273 | 15400 | 99.5 | 12 | 8 | 8 | CIL |
| 4 | 84 | Urban | Marathon | US 51, ST 29 | ST 29 | NB Entry | Three Leg Directional | Direct Connection | 4.54 | 3.12 | 2 | 0.706 | 21800 | 99.9 | 12 | 6 | 10 | CIL |
| 5 | 4 | Urban | Winnebago | US 41 | US 10,ST 441 | SB Entry | ParClo | Free Flow Loop | 4.52 | 0.47 | 1 | 0.845 | 15100 | 99.3 | 15 | 8 | 8 | CIL |
| 6 | 12 | Urban | Outagamie | US 41 | ST 125 (W College Ave) | SB Entry | Diamond | Diamond | 4.46 | 6.96 | 1 | 0.201 | 13500 | 99.1 | 15 | 2 | 10 | CIL |
| 7 | 5 | Urban | Marathon | ST 29 | US 51 Bus | WB Entry | ParClo | ParClo Loop | 4.31 | 6.92 | 1 | 0.202 | 11000 | 98.3 | 15 | 2 | 6 | CIL |
| 8 | 10 | Urban | Dane | I 39,90,94 | US 151 (E Washington Ave) | NB Exit | Cloverleaf | Outer Connection | 4.20 | 4.08 | 1 | 0.392 | 16300 | 99.6 | 15 | 6 | 8 | CIL |
| 9 | 7 | Urban | Outagamie | ST 441 | CT OO (E Northland Ave) | SB Entry | Diamond | Diamond | 4.11 | 0.00 | 1 | 0.174 | 11900 | 98.5 | 22 | 4 | 8 | CIL |
| 10 | 6 | Urban | Outagamie | ST 441 | CT CE (E College Ave) | NB Entry | Diamond | Diamond | 3.91 | 0.00 | 1 | 0.225 | 10300 | 97.9 | 26 | 4 | 6 | CIL |
| 11 | 143 | Rural | Washington | US 41,45 | US 45 | SB Entry | Three Leg Directional | Direct Connection | 3.86 | 1.28 | 2 | 0.936 | 17700 | 99.8 | 12 | 6 | 10 | CIL |
| 12 | 31 | Rural | Winnebago | US 41 | St-26 (Co Rd N) | NB Entry | Diamond | Diamond | 3.85 | 5.89 | 1 | 0.136 | 9900 | 97.1 | 24 | 2 | 2 | CIL |
| 13 | 8 | Urban | Outagamie | US 41 | ST 96 (W Wisconsin Ave) | SB Entry | Diamond | Diamond | 3.67 | 16.63 | 1 | 0.144 | 7600 | 93.2 | 24 | 2 | 2 | CIL |
| 14 | 14 | Urban | Brown | US 41 | ST 32,54 (W Mason St) | SB Entry | Diamond | Diamond | 3.62 | 15.67 | 1 | 0.191 | 8800 | 95.3 | 12 | 2 | 6 | CIL |
| 15 | 38 | Urban | Outagamie | ST 441 | CT OO (E Northland Ave) | NB Exit | Diamond | Diamond | 3.61 | 0.00 | 1 | 0.267 | 12000 | 98.7 | 15 | 4 | 8 | CIL |
| 16 | 185 | Urban | Waukesha | I 94 | ST 16 | EB Entry | Three Leg Directional | Direct Connection | 3.57 | 1.99 | 2 | 0.804 | 16000 | 99.6 | 12 | 6 | 10 | CIL |
| 17 | 11 | Urban | Brown | I 43 | CT V (E Mason St) | SB Entry | Diamond | Diamond | 3.52 | 3.44 | 1 | 0.174 | 8300 | 94.8 | 12 | 4 | 4 | CIL |
| 18 | 9 | Urban | Brown | I 43 | ST 172 | SB Exit | Three Leg Directional | Direct Connection | 3.50 | 0.47 | 1 | 0.858 | 18000 | 99.8 | 12 | 10 | 12 | CIL |
| 19 | 77 | Urban | Outagamie | US 41 | ST 125 (W College Ave) | NB Exit | Diamond | Diamond | 3.50 | 4.43 | 1 | 0.226 | 12900 | 98.9 | 17 | 6 | 6 | CIL |
| 20 | 200 | Urban | Brown | I 43 | ST 172 | NB Entry | Three Leg Directional | Direct Connection | 3.46 | 3.72 | 2 | 0.859 | 15400 | 99.5 | 12 | 6 | 10 | CIL |
| 21 | 114 | Urban | Brown | US 41 | ST-29 (Shawano Ave) | NB Exit | Diamond | Diamond | 3.43 | 13.43 | 1 | 0.223 | 14100 | 99.2 | 15 | 8 | 5 | CIL |
| 22 | 21 | Urban | Outagamie | US 41 | ST 96 (W Wisconsin Ave) | NB Entry | Diamond | Diamond | 3.33 | 11.39 | 1 | 0.176 | 7900 | 94.0 | 24 | 2 | 6 | CIL |
| 23 | 43 | Urban | Waukesha | I 94 | CT F (Redford Blvd) | WB Entry | Diamond | Diamond | 3.33 | 5.30 | 1 | 0.226 | 9300 | 96.2 | 20 | 2 | 10 | CIL |
| 24 | 47 | Urban | Brown | US 41 | ST 172 ( Airport Dr) | NB Exit | ParClo | Outer Connection | 3.32 | 0.50 | 1 | 0.398 | 13500 | 99.1 | 12 | 8 | 8 | CIL |
| 25 | 22 | Urban | Outagamie | US 41 | ST 441 | EB Entry | Trumpet | Direct Connection | 3.30 | 1.43 | 1 | 0.280 | 9000 | 95.6 | 15 | 4 | 8 | CIL |
| 26 | 56 | Urban | Brown | I 43 | St-57, 54 (Sturgeon Bay Rd) | WB Exit | ParClo | Outer Connection | 3.30 | 1.88 | 1 | 0.746 | 10700 | 98.0 | 12 | 4 | 8 | CIL |
| 27 | 240 | Urban | Brown | ST 172 | Pilgrim Way/Van Der Perren Way | EB Entry | Diamond | Diamond | 3.30 | 0.91 | 2 | 0.221 | 13200 | 99.0 | 12 | 8 | 6 | CIL |
| 28 | 15 | Urban | Dane | US 12,14 | Mineral Point Rd | NB Entry | Diamond | Diamond | 3.29 | 2.30 | 1 | 0.174 | 10200 | 97.7 | 15 | 7 | 9 | CIL |
| 29 | 13 | Urban | Brown | US 41,141 | I 43 | SB Entry | Trumpet | Direct Connection | 3.27 | 0.38 | 1 | 0.522 | 10000 | 97.2 | 12 | 8 | 8 | CIL |
| 30 | 23 | Urban | Outagamie | US 41 | ST 15,CT OO (W Northland Ave) | SB Entry | ParClo | Free Flow Loop | 3.26 | 5.04 | 1 | 0.318 | 7700 | 93.4 | 17 | 2 | 6 | CIL |
| 31 | 35 | Urban | Winnebago | US 41 | ST 114 (Winneconne Ave) | NB Entry | Diamond | Diamond | 3.26 | 4.10 | 1 | 0.244 | 9600 | 96.7 | 14 | 4 | 10 | CIL |
| 32 | 52 | Urban | Brown | US 41 | ST 172 ( Airport Dr) | SB Exit | ParClo | Outer Connection | 3.22 | 1.07 | 1 | 0.375 | 13000 | 98.9 | 15 | 8 | 8 | CIL |
| 33 | 27 | Urban | Outagamie | ST 441 | CT KK (E Calumet St) | NB Entry | Diamond | Diamond | 3.19 | 2.97 | 1 | 0.270 | 9900 | 97.1 | 20 | 6 | 10 | CIL |
| 34 | 282 | Urban | Brown | US 41 | ST 172 ( Airport Dr) | SB Entry | ParClo | Free Flow Loop | 3.16 | 0.61 | 2 | 0.660 | 12900 | 98.9 | 12 | 10 | 5 | CIL |
| 35 | 34 | Urban | Marathon | US 51, ST 29 | ST 29 | SB Exit | Three Leg Directional | Direct Connection | 3.15 | 1.55 | 2 | 1.036 | 19300 | 99.9 | 12 | 6 | 12 | CIL |
| 36 | 20 | Urban | Outagamie | US 41 | CT E (N Ballard Rd) | WB Entry | Diamond | Diamond | 3.15 | 4.38 | 1 | 0.183 | 7800 | 93.6 | 17 | 4 | 6 | CIL |
| 37 | 16 | Urban | LaCrosse | I 90,US 53 | US 53,ST 35 | EB Entry | ParClo | Outer Connection | 3.12 | 4.01 | 1 | 0.449 | 9400 | 96.2 | 15 | 8 | 8 | CIL |
| 38 | 33 | Urban | Brown | ST 172 | CT GV (Monroe Rd) | WB Entry | Diamond | Diamond | 3.09 | 0.62 | 1 | 0.323 | 9500 | 96.5 | 14 | 6 | 10 | CIL |
| 39 | 18 | Urban | Dane | US 12,14 | Old Sauk Rd | SB Entry | Diamond | Diamond | 3.03 | 0.00 | 1 | 0.280 | 7900 | 94.0 | 15 | 6 | 6 | CIL |
| 40 | 37 | Urban | Brown | I 43 | St-57, 54 (Sturgeon Bay Rd) | WB Entry | ParClo | Direct Connection | 3.01 | 0.00 | 1 | 0.452 | 8000 | 94.1 | 12 | 4 | 8 | CIL |
| 41 | 29 | Urban | Outagamie | ST 441 | CT KK (E Calumet St) | SB Entry | Diamond | Diamond | 2.99 | 2.11 | 1 | 0.284 | 7300 | 92.1 | 24 | 4 | 6 | CIL |
| 42 | 24 | Urban | Outagamie | US 41 | ST 47 (N Richmond St) | EB Entry | Diamond | Diamond | 2.98 | 2.74 | 1 | 0.292 | 7500 | 92.7 | 12 | 5 | 6 | CIL |
| 43 | 322 | Urban | Brown | US 41 | ST 172 ( Airport Dr) | WB Entry | ParClo | Direct Connection | 2.97 | 0.60 | 2 | 0.334 | 11900 | 98.5 | 12 | 10 | 5 | CIL |
| 44 | 61 | Rural | Washington | US 41,45 | US 45 | NB Exit | Three Leg Directional | Direct Connection | 2.95 | 1.85 | 2 | 0.756 | 17700 | 99.8 | 12 | 6 | 10 | CIL |
| 45 | 69 | Urban | Brown | ST 172 | Pilgrim Way/Van Der Perren Way | WB Exit | Diamond | Diamond | 2.93 | 7.30 | 2 | 0.192 | 14400 | 99.2 | 12 | 4 | 8 | CIL |
| 46 | 286 | Urban | Calumet | ST 441 | US 10 (Oneida St) | WB Entry | Diamond | Diamond | 2.90 | 8.18 | 2 | 0.196 | 10100 | 97.4 | 12 | 5 | 6 | CIL |
| 47 | 149 | Urban | Outagamie | US 41 | ST 441 | EB Exit | Trumpet | Direct Connection | 2.90 | 0.78 | 1 | 0.257 | 10100 | 97.4 | 15 | 4 | 8 | CIL |
| 48 | 26 | Urban | Dane | I 39,90,94 | I 94 | NB Entry | All Directional | Direct Connection | 2.88 | 2.10 | 1 | 0.190 | 8500 | 95.0 | 12 | 8 | 8 | CIL |
| 49 | 102 | Urban | Calumet | ST 441 | US 10 (Oneida St) | EB Exit | Diamond | Diamond | 2.87 | 12.18 | 1 | 0.197 | 9500 | 96.5 | 15 | 5 | 8 | CIL |
| 50 | 28 | Urban | Winnebago | US 41 | US 10, STH 441 | NB Entry | ParClo | Direct Connection | 2.85 | 3.38 | 1 | 0.296 | 8400 | 94.9 | 15 | 8 | 8 | CIL |
| 51 | 42 | Urban | Dane | US 151 (E Washington Ave) | American Pkwy | SB Entry | ParClo | Outer Connection | 2.85 | 0.00 | 1 | 0.167 | 6600 | 90.2 | 19 | 3 | 6 | CIL |
| 52 | 294 | Urban | Brown | US 41 | ST-29 (Shawano Ave) | NB Entry | Diamond | Diamond | 2.82 | 2.66 | 2 | 0.375 | 8300 | 94.8 | 12 | 5 | 2 | CIL |
| 53 | 30 | Urban | Dane | I 39,90 | US 12,18 (Beltline Hwy) | SB Entry | Cloverleaf | Direct Connection | 2.81 | 0.29 | 1 | 0.698 | 9500 | 96.5 | 12 | 10 | 10 | CIL |
| 54 | 19 | Urban | Waukesha | I 94 | ST 83 | EB Entry | Diamond | Diamond | 2.80 | 4.04 | 1 | 0.248 | 7800 | 93.6 | 16 | 9 | 6 | CIL |
| 55 | 32 | Urban | Brown | I 43 | St-57, 54 (Sturgeon Bay Rd) | EB Entry | ParClo | Free Flow Loop | 2.80 | 0.00 | 1 | 0.268 | 8200 | 94.7 | 12 | 8 | 8 | CIL |
| 56 | 107 | Urban | Outagamie | ST 441 | CT KK (E Calumet St) | SB Exit | Diamond | Diamond | 2.80 | 0.00 | 1 | 0.204 | 9700 | 96.8 | 28 | 6 | 8 | CIL |
| 57 | 39 | Urban | Walworth | US 12 | ST 50 | NB Entry | ParClo | Outer Connection | 2.79 | 1.70 | 1 | 0.235 | 7700 | 93.4 | 16 | 6 | 8 | CIL |
| 58 | 25 | Urban | Outagamie | US 41 | ST 125 (W College Ave) | NB Entry | Diamond | Diamond | 2.75 | 6.51 | 1 | 0.215 | 7400 | 92.6 | 12 | 8 | 6 | CIL |
| 59 | 125 | Rural | Racine | I 94,US 41 | ST 20 (Washington Ave) | NB Entry | Diamond | Diamond | 2.74 | 1.97 | 1 | 0.407 | 10000 | 97.2 | 20 | 6 | 10 | CIL |
| 60 | 44 | Urban | Marathon | ST 29 | CT X(Camp Philips Rd) | WB Entry | Diamond | Diamond | 2.73 | 0.85 | 1 | 0.237 | 8600 | 95.1 | 15 | 8 | 10 | CIL |
| 61 | 17 | Urban | Brown | US 41 | US 141 (Velp Ave) | NB Entry | Diamond | Diamond | 2.72 | 5.55 | 1 | 0.252 | 6200 | 89.1 | 19 | 8 | 2 | CIL |
| 62 | 53 | Urban | Marathon | US 51 | Maple Ridge Rd | NB Entry | Diamond | Diamond | 2.72 | 1.86 | 1 | 0.108 | 8100 | 94.4 | 15 | 6 | 10 | CIL |
| 63 | 384 | Urban | Brown | ST 172 | ST 57 (Riverside Dr) | WB Entry | ParClo | Diamond | 2.72 | 1.49 | 2 | 0.268 | 12100 | 98.7 | 12 | 8 | 10 | CIL |
| 64 | 354 | Urban | Waukesha | I 94 | ST 164 (Pewaukee Rd) | WB Entry | Diamond | Diamond | 2.72 | 9.45 | 2 | 0.212 | 10400 | 98.0 | 20 | 6 | 8 | CIL |
| 65 | 49 | Urban | Dane | US 12,14 | Greenway Blvd | SB Entry | Diamond | Diamond | 2.71 | 0.66 | 1 | 0.301 | 7200 | 91.7 | 12 | 5 | 8 | CIL |
| 66 | 355 | Urban | Brown | I 43 | St-57, 54 (Sturgeon Bay Rd) | EB Exit | ParClo | Outer Connection | 2.68 | 1.06 | 1 | 0.566 | 10300 | 97.9 | 12 | 8 | 4 | CIL |
| 67 | 54 | Urban | Outagamie | US 41 | ST 47 (N Richmond St) | WB Entry | Diamond | Diamond | 2.68 | 3.78 | 1 | 0.212 | 6000 | 88.2 | 12 | 2 | 6 | CIL |
| 68 | 73 | Urban | Marathon | ST 29 | US 51 Bus | EB Exit | ParClo | ParClo Loop | 2.66 | 1.77 | 1 | 0.338 | 9100 | 95.9 | 15 | 6 | 10 | CIL |
| 69 | 344 | Urban | Outagamie | US 41 | ST 441 | WB Entry | Trumpet | Outer Connection | 2.65 | 0.63 | 2 | 0.632 | 10200 | 97.7 | 12 | 4 | 10 | CIL |
| 70 | 48 | Urban | Washington | US 45 | ST 144 | SB Entry | Diamond | Diamond | 2.65 | 3.74 | 1 | 0.267 | 8300 | 94.8 | 15 | 8 | 10 | CIL |
| 71 | 250 | Urban | Outagamie | US 41 | ST 96 (W Wisconsin Ave) | NB Exit | Diamond | Diamond | 2.65 | 10.56 | 1 | 0.133 | 8100 | 94.4 | 12 | 4 | 6 | CIL |
| 72 | 55 | Urban | Dane | US 151 | American Pkwy | SB Entry | ParClo | ParClo Loop | 2.64 | 1.28 | 1 | 0.469 | 7800 | 93.6 | 21 | 6 | 10 | CIL |
| 73 | 214 | Urban | Winnebago | US 41 | US 10,ST 441 | SB Exit | ParClo | Free Flow Loop | 2.64 | 0.47 | 1 | 0.848 | 9100 | 95.9 | 15 | 4 | 8 | CIL |
| 74 | 40 | Urban | Waukesha | ST 16 | ST 190 (Capitol Dr) | NB Entry | Diamond | Diamond | 2.64 | 2.66 | 1 | 0.301 | 7600 | 93.2 | 15 | 8 | 8 | CIL |
| 75 | 36 | Urban | Eau Claire | US 53 | ST 312 (N Crossing) | SB Entry | Diamond | Diamond | 2.64 | 3.18 | 1 | 0.252 | 6800 | 90.5 | 14 | 7 | 6 | CIL |
| 76 | 41 | Urban | Outagamie | US 41 | ST 15,CT OO (W Northland Ave) | NB Entry | ParClo | Direct Connection | 2.64 | 4.53 | 1 | 0.309 | 6600 | 90.2 | 17 | 6 | 6 | CIL |
| 77 | 142 | Urban | Brown | I 43 | CT V (E Mason St) | NB Exit | Diamond | Diamond | 2.62 | 0.00 | 1 | 0.286 | 8000 | 94.1 | 15 | 4 | 8 | CIL |
| 78 | 65 | Urban | Waukesha | I 94 | CT F (Redford Blvd) | WB Exit | Diamond | Diamond | 2.61 | 7.10 | 2 | 0.197 | 12000 | 98.7 | 18 | 2 | 10 | CIL |
| 79 | 96 | Rural | Columbia | I 90,94 | I 39 | NB Entry | Other | Outer Connection | 2.61 | 1.57 | 1 | 0.383 | 9200 | 96.0 | 12 | 8 | 8 | CIL |
| 80 | 360 | Urban | Marathon | US 51, ST 29 | CT N(Rib Mt Dr) | SB Entry | Diamond | Diamond | 2.61 | 1.73 | 2 | 0.231 | 9200 | 96.0 | 12 | 4 | 8 | CIL |
| 81 | 103 | Urban | Waukesha | I 94 | ST 16 | WB Exit | Three Leg Directional | Direct Connection | 2.59 | 1.19 | 2 | 0.675 | 15000 | 99.3 | 12 | 6 | 10 | CIL |
| 82 | 51 | Urban | Chippewa | US 53 | ST 29 | SB Entry | Cloverleaf | Free Flow Loop | 2.59 | 0.58 | 1 | 0.347 | 7000 | 91.0 | 14 | 6 | 8 | CIL |
| 83 | 45 | Urban | Brown | US 41,141 | CT M (Lineville Rd) | SB Entry | Diamond | Diamond | 2.58 | 0.00 | 1 | 0.259 | 7400 | 92.6 | 15 | 8 | 8 | CIL |
| 84 | 46 | Urban | Dane | I 39,90,94 | I 94 | SB Entry | All Directional | Direct Connection | 2.58 | 1.95 | 1 | 0.205 | 7400 | 92.6 | 12 | 8 | 8 | CIL |
| 85 | 126 | Urban | Dane | US 12,14 | Mineral Point Rd | SB Exit | Diamond | Diamond | 2.58 | 1.33 | 1 | 0.150 | 9800 | 96.9 | 12 | 8 | 8 | CIL |
| 86 | 368 | Urban | Marathon | US 51 | W Bridge St | SB Entry | Diamond | Diamond | 2.58 | 0.00 | 2 | 0.195 | 7400 | 92.6 | 15 | 5 | 2 | CIL |
| 87 | 76 | Urban | Brown | ST 172 | CT GV (Monroe Rd) | EB Exit | Diamond | Diamond | 2.56 | 0.00 | 1 | 0.316 | 9700 | 96.8 | 20 | 8 | 10 | CIL |
| 88 | 404 | Urban | Winnebago | US 41 | CT II (Winchester Rd) | NB Entry | Diamond | Diamond | 2.55 | 0.00 | 2 | 0.206 | 9600 | 96.7 | 13 | 6 | 8 | CIL |
| 89 | 427 | Urban | Brown | I 43 | ST 172 | EB Entry | Three Leg Directional | Direct Connection | 2.51 | 1.19 | 2 | 0.670 | 10200 | 97.7 | 12 | 6 | 10 | CIL |
| 90 | 454 | Urban | Waukesha | I 94 | CT F (Redford Blvd) | EB Entry | Diamond | Diamond | 2.51 | 1.45 | 2 | 0.138 | 10900 | 98.2 | 21 | 8 | 10 | CIL |
| 91 | 375 | Urban | Dane | US 12,14 | US 14 (University Ave) | SB Entry | ParClo | ParClo Loop | 2.48 | 3.65 | 2 | 0.274 | 9100 | 95.9 | 12 | 2 | 11 | CIL |
| 92 | 83 | Urban | Brown | I 43 | Manitowoc Rd | SB Entry | ParClo | ParClo Loop | 2.46 | 3.13 | 1 | 0.255 | 6700 | 90.3 | 12 | 4 | 10 | CIL |
| 93 | 229 | Urban | Outagamie | US 41 | ST 15,CT OO (W Northland Ave) | NB Exit | ParClo | Direct Connection | 2.45 | 1.78 | 1 | 0.450 | 9100 | 95.9 | 15 | 6 | 8 | CIL |
| 94 | 183 | Urban | Outagamie | ST 441 | CT KK (E Calumet St) | NB Exit | Diamond | Diamond | 2.44 | 2.52 | 1 | 0.317 | 7300 | 92.1 | 15 | 4 | 8 | CIL |
| 95 | 184 | Urban | Outagamie | US 41 | ST 96 (W Wisconsin Ave) | SB Exit | Diamond | Diamond | 2.44 | 12.39 | 1 | 0.178 | 7300 | 92.1 | 15 | 4 | 8 | CIL |
| 96 | 139 | Urban | Chippewa | US 53 | ST 29 | NB Exit | Cloverleaf | Outer Connection | 2.41 | 1.52 | 1 | 0.394 | 7600 | 93.2 | 14 | 5 | 9 | CIL |
| 97 | 71 | Urban | Brown | I 43 | CT V (E Mason St) | NB Entry | Diamond | Diamond | 2.40 | 0.86 | 1 | 0.232 | 6000 | 88.2 | 12 | 4 | 8 | CIL |
| 98 | 490 | Urban | Eau Claire | US 53 | Golf Rd | NB Entry | Diamond | Diamond | 2.40 | 3.80 | 2 | 0.369 | 10300 | 97.9 | 12 | 8 | 10 | CIL |
| 99 | 459 | Urban | Dane | I 39,90,94 | I 94 | NB Entry | All Directional | Direct Connection | 2.40 | 0.57 | 2 | 0.352 | 9600 | 96.7 | 12 | 6 | 10 | CIL |
| 100 | 317 | Urban | Eau Claire | US 53 | Golf Rd | SB Exit | Diamond | Diamond | 2.39 | 7.37 | 2 | 0.244 | 10900 | 98.2 | 12 | 3 | 2 | CIL |

a. AADT Pctl refer to the percentile of AADT of a particular segment compared with all segments of its type in Wisconsin.

b. AASHTO Standard refer to which lighting type is warranted based on AASHTO criteria. The lighting types include CFL (Continuous Freeway Lighting), CIL (Complete Interchange Lighting) and PIL (Partial Interchange Lighting). Blank indicates lighting is not warranted.

c. The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 2.51 |  | 1.08 |  |
|  | 1.58 | 2.50 | 0.62 | 1.07 |
|  | 1.00 | 1.57 | 0.03 | 0.61 |
|  | 0.55 | 0.99 | 0.08 | 0.30 |
|  |  | 0.54 |  | 0.07 |

# Appendix D Top 100 Unlighted Segments Concerning Lighting Benefit

Table D-1 Top 100 Unlighted Freeway Segments Concerning Lighting Benefit (crash/mile/year)

| Lighting Benefit Rank | Crash Risk Rank | County Name | County ID | Segment ID | Route Number | From Interchange/Cross Rd | To Interchange/Cross Rd | Reduction in Crash Density by Lighting (crash/mile/year) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Dane | 13 | 13014 | US 12,14,18,151 | Fish Hatchery Rd | US 14 (Park St) | 2.19 |
| 2 | 3 | Waukesha | 67 | 67014 | I 94 | S Brookfield Rd | CTH O | 2.16 |
| 3 | 8 | Dane | 13 | 13017 | US 12,18 | John Nolen Dr | S Towne Dr | 2.03 |
| 4 | 4 | Waukesha | 67 | 67013 | I 94 | CTH Y (S Barker Rd) | S Brookfield Rd | 2.02 |
| 5 | 12 | Dane | 13 | 13018 | US 12,18 | S Towne Dr | Monona Dr | 2.02 |
| 6 | 5 | Waukesha | 67 | 67012 | I 94,ST 164 | CTH F (Redford Blvd) | CTH Y (S Barker Rd) | 1.97 |
| 7 | 7 | Waukesha | 67 | 67011 | I 94,ST 16 | STH 164 (Pewaukee Rd) | CTH F (Redford Blvd) | 1.96 |
| 8 | 2 | Dane | 13 | 13011 | US 12,14,18,151 | US 18,151 (Verona Rd) | Seminole Hwy | 1.96 |
| 9 | 13 | Dane | 13 | 13019 | US 12,18 | Monona Dr | US 51 (Stoughton Rd) | 1.95 |
| 10 | 21 | Winnebago | 70 | 70002 | US 41 | US 10 | CTH B | 1.89 |
| 11 | 6 | Dane | 13 | 13015 | US 12,18 | US 14 (Park St) | Rimrock Rd | 1.83 |
| 12 | 64 | Milwaukee | 40 | 40030 | I 94,US 41 | STHH 100 (W Ryan Rd) | W Oakwood Rd | 1.81 |
| 13 | 44 | Milwaukee | 40 | 40029 | I 94,US 41 | W Puetz Rd | STHH 100 (W Ryan Rd) | 1.80 |
| 14 | 17 | Racine | 51 | 51002 | I 94,US 41 | 7 Mile Rd | 5 1/2 Mile Rd | 1.73 |
| 15 | 18 | Racine | 51 | 51003 | I 94,US 41 | 5 1/2 Mile Rd | CTH K | 1.72 |
| 16 | 10 | Waukesha | 67 | 67010 | I 94 | STH 16 | STH 164 (Pewaukee Rd) | 1.70 |
| 17 | 22 | Dane | 13 | 13080 | I 39,90 | I 94 | Cottage Grove Rd | 1.68 |
| 18 | 43 | Dane | 13 | 13079 | I 39,90,94 | US 151 (E Washington Ave) | I 94 | 1.67 |
| 19 | 25 | Kenosha | 30 | 30007 | I 94,US 41 | CTH Q | Illinois State Line | 1.67 |
| 20 | 9 | Dane | 13 | 13009 | US 12,14 | Gammon Rd | Whitney Way | 1.63 |
| 21 | 24 | Dane | 13 | 13082 | I 39,90 | E Buckeye Rd | US 12,18 (Beltline Hwy) | 1.63 |
| 22 | 32 | Kenosha | 30 | 30003 | I 94,US 41 | STH 142 (Burlington Rd) | STH 158 | 1.62 |
| 23 | 33 | Kenosha | 30 | 30004 | I 94,US 41 | STH 158 | STH 50 (75th St) | 1.62 |
| 24 | 62 | Waukesha | 67 | 67006 | I 94 | STHH 83 | CTH E | 1.61 |
| 25 | 79 | Waukesha | 67 | 67025 | I 43 | S Calhoun Rd | CTH O | 1.58 |
| 26 | 46 | Kenosha | 30 | 30002 | I 94,US 41 | CTH E | STHH 142 (Burlington Rd) | 1.58 |
| 27 | 63 | Winnebago | 70 | 70001 | US 41 | CTH BB (W Prospect Ave) | US 10 | 1.57 |
| 28 | 26 | Dane | 13 | 13081 | I 39,90 | Cottage Grove Rd | E Buckeye Rd | 1.56 |
| 29 | 113 | Kenosha | 30 | 30005 | I 94,US 41 | STHH 50 (75th St) | CTH C (Wilmot Rd) | 1.56 |
| 30 | 136 | Dane | 13 | 13078 | I 39,90,94 | Hoepker Rd | US 151 (E Washington Ave) | 1.56 |
| 31 | 98 | Sauk | 56 | 56002 | I 90,94 | STH 13 | Trout Rd | 1.55 |
| 32 | 52 | Ozaukee | 45 | 45018 | I 43,ST 32 | STHH 57 | Donges Bay Rd | 1.54 |
| 33 | 60 | Milwaukee | 40 | 40031 | I 94,US 41 | Winchester Rd | Main St | 1.54 |
| 34 | 58 | Washington | 66 | 66024 | US 41,45 | US 45 | STH 167 (Holy Hill Rd) | 1.54 |
| 35 | 53 | Racine | 51 | 51004 | I 94,US 41 | CTH K | 2 Mile Rd | 1.53 |
| 36 | 54 | Racine | 51 | 51005 | I 94,US 41 | 2 Mile Rd | STHH 20 (Washington Ave) | 1.53 |
| 37 | 120 | Kenosha | 30 | 30006 | I 94,US 41 | S Oneida St | CTH G (Main Ave) | 1.52 |
| 38 | 130 | Marathon | 37 | 37010 | US 51,ST 29 | STH 29 | CTH NN | 1.52 |
| 39 | 69 | Racine | 51 | 51007 | I 94,US 41 | Main St | STHH 114 (Winneconne Ave) | 1.51 |
| 40 | 39 | Dane | 13 | 13021 | US 12,18 | US 51 (Stoughton Rd) | I 39,90 | 1.50 |
| 41 | 144 | Sauk | 56 | 56008 | I 90,94 | CTH T | Van Hoosen Rd | 1.50 |
| 42 | 15 | Brown | 5 | 5027 | US 41 | CTH C (Wilmot) | CTH Q | 1.50 |
| 43 | 74 | Washington | 66 | 66025 | US 41,45,ST 167 | STH 167 (Holy Hill Rd) | CTH F (Freistadt Rd) | 1.49 |
| 44 | 87 | Waukesha | 67 | 67004 | I 94 | CTH P | CTH C | 1.48 |
| 45 | 27 | Outagamie | 44 | 44012 | US 41 | CTH OO | CTH A | 1.47 |
| 46 | 103 | Winnebago | 70 | 70005 | US 41 | STHH 114 (Winneconne Ave) | Breezewood Ln | 1.47 |
| 47 | 11 | Waukesha | 67 | 67008 | I 94 | CTH SS | CTH G | 1.47 |
| 48 | 90 | Waukesha | 67 | 67024 | I 43 | CTH Y | S Calhoun Rd | 1.46 |
| 49 | 59 | Winnebago | 70 | 70003 | US 41 | CTH B | Oakridge Rd | 1.45 |
| 50 | 71 | Waukesha | 67 | 67026 | I 43 | CTH O | S 124th St | 1.45 |
| 51 | 40 | Ozaukee | 45 | 45019 | I 43,ST 32 | Donges Bay Rd | W County Line Rd | 1.45 |
| 52 | 67 | Winnebago | 70 | 70004 | US 41 | Oakridge Rd | STHH 114 (Winneconne Ave) | 1.43 |
| 53 | 165 | Winnebago | 70 | 70016 | US 41 | W Ripple Ave | STH 26,CTH N | 1.43 |
| 54 | 153 | Columbia | 11 | 11014 | I 90,94 | Blount Rd | STH 33 | 1.43 |
| 55 | 75 | Washington | 66 | 66026 | US 41,45,ST 167 | CTH F (Freistadt Rd) | STH 167 (Lannon Rd) | 1.42 |
| 56 | 115 | Kenosha | 30 | 30001 | I 94,US 41 | CTH KR | CTH E | 1.41 |
| 57 | 82 | Outagamie | 44 | 44015 | US 41 | STHH 125 (W College Ave) | W Prospect Ave | 1.40 |
| 58 | 124 | Racine | 51 | 51006 | I 94,US 41 | STHH 20 (Washington Ave) | STHH 11 (Durand Ave) | 1.39 |
| 59 | 80 | Dane | 13 | 13004 | US 12,14 | US 14 (University Ave) | Greenway Blvd | 1.39 |
| 60 | 56 | Rock | 53 | 53007 | I 39,90 | CTH A | STHH 47 | 1.37 |
| 61 | 158 | Sauk | 56 | 56009 | I 90,94 | Van Hoosen Rd | Schepp Rd | 1.36 |
| 62 | 61 | Dane | 13 | 13090 | I 39,90 | CTH B | E Church Rd | 1.36 |
| 63 | 128 | Winnebago | 70 | 70007 | US 41 | CTH G | CTH GG | 1.35 |
| 64 | 110 | Dane | 13 | 13076 | I 39,90,94 | STH 19 | US 51 | 1.35 |
| 65 | 195 | Dane | 13 | 13077 | I 39,90,94 | US 51 | Hoepker Rd | 1.34 |
| 66 | 14 | Brown | 5 | 5023 | US 41,ST 32 | STH 29 | STH 32 | 1.34 |
| 67 | 169 | Columbia | 11 | 11013 | I 90,94 | Tritz Rd | Blount Rd | 1.33 |
| 68 | 170 | Sauk | 56 | 56006 | I 90,94 | CTH A | Gillem Rd | 1.33 |
| 69 | 171 | Sauk | 56 | 56007 | I 90,94 | Gillem Rd | CTH T | 1.33 |
| 70 | 172 | Sauk | 56 | 56010 | I 90,94 | N Hein Rd | East County Line | 1.33 |
| 71 | 244 | Columbia | 11 | 11020 | I 39,90,94 | Black Rd | Kent Rd | 1.33 |
| 72 | 177 | St. Croix | 55 | 55003 | I 94,US 12 | Carmichael Rd | STH 35 | 1.33 |
| 73 | 134 | Winnebago | 70 | 70006 | US 41 | Breezewood Ln | CTH G | 1.32 |
| 74 | 93 | Winnebago | 70 | 70034 | US 10,ST 441 | Midway Rd | STH 47 | 1.32 |
| 75 | 23 | Rock | 53 | 53011 | I 39,90 | US 14 Bus | STH 11 | 1.32 |
| 76 | 174 | Waukesha | 67 | 67003 | I 94 | STH 67 | CTH P | 1.31 |
| 77 | 213 | Chippewa | 9 | 9018 | US 53 | STH 29 | CTH OO | 1.31 |
| 78 | 65 | Dane | 13 | 13086 | I 39,90 | CTH N | Church St | 1.30 |
| 79 | 19 | Winnebago | 70 | 70008 | US 41 | CTH GG | STH 76 | 1.30 |
| 80 | 168 | Washington | 66 | 66014 | US 41 | Western Ave | STH 145 | 1.30 |
| 81 | 41 | Brown | 5 | 5028 | US 41 | CTH G (Main Ave) | CTH F (Scheuring Rd) | 1.29 |
| 82 | 240 | Columbia | 11 | 11025 | I 39,90,94 | STH 60 | CTH K | 1.29 |
| 83 | 16 | Dane | 13 | 13006 | US 12,14 | Old Sauk Rd | Mineral Point Rd | 1.29 |
| 84 | 137 | Brown | 5 | 5035 | ST 172 | US 41 | Pilgrim Way | 1.28 |
| 85 | 241 | St. Croix | 55 | 55011 | I 94 | 140th St | CTH J | 1.28 |
| 86 | 68 | Dane | 13 | 13087 | I 39,90 | Church St | Drotning Rd | 1.28 |
| 87 | 78 | Brown | 5 | 5021 | US 41,141 | N Lakeview Dr | I 43 | 1.28 |
| 88 | 243 | St. Croix | 55 | 55012 | I 94 | CTH J | CTH T | 1.27 |
| 89 | 162 | Waukesha | 67 | 67007 | I 94 | CTH G (Elmhurst Rd) | CTH SS | 1.27 |
| 90 | 76 | Waukesha | 67 | 67009 | I 94 | CTH G | CTH T | 1.27 |
| 91 | 77 | Rock | 53 | 53002 | I 39,90 | STH 59 | E Knutson Rd | 1.27 |
| 92 | 117 | Dane | 13 | 13092 | I 39,90 | CTH A | US 51 | 1.27 |
| 93 | 173 | Dane | 13 | 13061 | US 151 | American Pkwy | CTH C (Reiner Rd) | 1.27 |
| 94 | 210 | Winnebago | 70 | 70014 | US 41 | W 9th Ave | STH 44 | 1.27 |
| 95 | 20 | Brown | 5 | 5026 | US 41 | STHH 172 | S Oneida St | 1.26 |
| 96 | 34 | Dane | 13 | 13083 | I 39,90 | US 12,18 (Beltline Hwy) | Siggelkow Rd | 1.25 |
| 97 | 189 | Winnebago | 70 | 70013 | US 41 | STH 21 | W 9th Ave | 1.25 |
| 98 | 179 | Washington | 66 | 66013 | US 41 | Sherman Rd | Western Ave | 1.25 |
| 99 | 149 | Outagamie | 44 | 44014 | US 41 | STH 96 (W Wisconsin Ave) | STH 125 (W College Ave) | 1.25 |
| 100 | 55 | Outagamie | 44 | 44017 | ST 441 | CTH OO (E Northland Ave) | CTH CE | 1.25 |

Note: The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 4.10 |  | 1.40 |  |
|  | 2.92 | 4.09 | 1.03 | 1.39 |
|  | 2.10 | 2.91 | 0.78 | 1.02 |
|  | 1.45 | 2.09 | 0.57 | 0.77 |
|  |  | 1.44 |  | 0.56 |

Table D-2 Top 100 Unlighted Interchange Segments Concerning Lighting Benefit (crash/mile/year)

| Lighting Benefit Rank | Crash Risk Rank | County Name | County ID | Interchange ID | On Road | Crossing Road | Interchange Type | Reduction in Crash Density by Lighting (crash/mile/year) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | Waukesha | 67 | 6709 | I 94 | ST 164 (Pewaukee Rd) | Diamond | 16.17 |
| 2 | 1 | Dane | 13 | 1315 | US 12,18 (Beltline Hwy) | John Nolen Dr | Trumpet | 13.23 |
| 3 | 2 | Waukesha | 67 | 6710 | I 94 | CT F (Redford Blvd) | Diamond | 12.61 |
| 4 | 7 | Winnebago | 70 | 7004 | US 41 | ST 114 (Winneconne Ave) | Diamond | 11.21 |
| 5 | 6 | Dane | 13 | 1317 | US 12,18 (Beltline Hwy) | Monona Dr | Diamond | 9.84 |
| 6 | 9 | Outagamie | 44 | 4412 | US 41 | CT BB (W Prospect Ave) | Diamond | 9.45 |
| 7 | 8 | Winnebago | 70 | 7002 | US 41 | CT II (Winchester Rd) | Diamond | 9.28 |
| 8 | 10 | Dane | 13 | 1318 | US 12,18 (Beltline Hwy) | US 51 (Stoughton Rd) | Diamond | 9.16 |
| 9 | 3 | Dane | 13 | 1316 | US 12,18 (Beltline Hwy) | S Towne Dr | Diamond | 8.82 |
| 10 | 5 | Dane | 13 | 1314 | US 12,18 (Beltline Hwy) | Rimrock Rd | Diamond | 7.91 |
| 11 | 15 | Waukesha | 67 | 6708 | I 94 | ST 16 | Three Leg Directional | 7.51 |
| 12 | 11 | Kenosha | 30 | 3006 | I 94,US 41 | ST 165 | ParClo | 7.08 |
| 13 | 23 | Washington | 66 | 6615 | US 41,45 | ST 167 (Lannon Rd) | Diamond | 6.87 |
| 14 | 16 | Dane | 13 | 1349 | I 39,90,94 | I 94 | All Directional | 6.70 |
| 15 | 18 | Milwaukee | 40 | 4036 | I 94,US 41 (North South Fwy) | ST 100 (W Ryan Rd) | Diamond | 6.58 |
| 16 | 13 | Waukesha | 67 | 6706 | I 94 | CT G | Diamond | 6.14 |
| 17 | 12 | Racine | 51 | 5102 | I 94,US 41 | 7 Mile Rd | Diamond | 6.10 |
| 18 | 26 | Waukesha | 67 | 6705 | I 94 | CT SS (Prospect Ave) | Diamond | 6.06 |
| 19 | 29 | Waukesha | 67 | 6707 | I 94 | CT T (N Grandview Blvd) | Diamond | 6.06 |
| 20 | 19 | Winnebago | 70 | 7003 | US 41 | Oakridge Rd (Main St) | ParClo | 6.03 |
| 21 | 33 | Dane | 13 | 1347 | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | 6.02 |
| 22 | 20 | Winnebago | 66 | 6614 | US 41,45 | ST 167 (Holly Hill Rd) | Diamond | 5.99 |
| 23 | 24 | Kenosha | 30 | 3004 | I 94,US 41 | ST 50 (75th St) | Diamond | 5.92 |
| 24 | 30 | Brown | 5 | 528 | ST 172 | S Webster Ave | Diamond | 5.86 |
| 25 | 25 | Winnebago | 70 | 7001 | US 41 | US 10,ST 441 | ParClo | 5.79 |
| 26 | 31 | Ozaukee | 45 | 4508 | I 43,ST 32,57 | ST 57 (Mequon Rd) | Diamond | 5.67 |
| 27 | 14 | Racine | 51 | 5103 | I 94,US 41 | CT G | Diamond | 5.60 |
| 28 | 17 | Dane | 13 | 1348 | I 39,90,94 | High Crossing Blvd | Half-Diamond | 5.54 |
| 29 | 32 | Waukesha | 67 | 6704 | I 94 | ST 83 | Diamond | 5.53 |
| 30 | 22 | Kenosha | 30 | 3003 | I 94,US 41 | ST 158 (52nd St) | Diamond | 5.49 |
| 31 | 36 | Waukesha | 67 | 6703 | I 94 | CT C (Genesee St) | ParClo | 5.37 |
| 32 | 37 | Winnebago | 70 | 7016 | US 10,ST 441 | Racine Rd | Trumpet | 5.34 |
| 33 | 50 | Dane | 13 | 1337 | US 151 (E Washington Ave) | American Pkwy | ParClo | 5.31 |
| 34 | 54 | Dane | 13 | 1338 | US 151 (E Washington Ave) | Reiner Rd | Diamond | 5.25 |
| 35 | 41 | Outagamie | 44 | 4411 | US 41 | ST 125 (W College Ave) | Diamond | 5.08 |
| 36 | 51 | Fond Du Lac | 20 | 2004 | US 41 | S Military Rd | Diamond | 4.97 |
| 37 | 21 | Kenosha | 30 | 3002 | I 94,US 41 | ST 142 (Burlington Rd) | Diamond | 4.95 |
| 38 | 66 | Fond Du Lac | 20 | 2005 | US 41 | CT VW (S Hickory St) | Diamond | 4.94 |
| 39 | 45 | Outagamie | 44 | 4410 | US 41 | ST 96 (W Wisconsin Ave) | Diamond | 4.91 |
| 40 | 49 | Dane | 13 | 1306 | US 12,14 (Beltline Hwy) | Mineral Point Rd | Diamond | 4.82 |
| 41 | 27 | Kenosha | 30 | 3001 | I 94,US 41 | CT E (12th St) | Diamond | 4.69 |
| 42 | 28 | Racine | 51 | 5104 | I 94,US 41 | CT K | Diamond | 4.69 |
| 43 | 53 | Ozaukee | 45 | 4506 | I 43,ST 32,57 | ST 60 (Washington St) | Diamond | 4.68 |
| 44 | 39 | Dane | 13 | 1346 | I 39,90,94 | US 51 (Stoughton Rd) | ParClo | 4.68 |
| 45 | 34 | Racine | 51 | 5106 | I 94,US 41 | ST 11 (Durand Ave) | ParClo | 4.49 |
| 46 | 35 | Racine | 51 | 5107 | I 94,US 41 | CT KR (1st Ave) | Diamond | 4.49 |
| 47 | 55 | Fond Du Lac | 20 | 2006 | US 41 | US 151 | Diamond | 4.37 |
| 48 | 38 | Kenosha | 30 | 3005 | I 94,US 41 | CT C (Wilmot Rd) | Diamond | 4.34 |
| 49 | 44 | Racine | 51 | 5105 | I 94,US 41 | ST 20 (Washington Ave) | Diamond | 4.31 |
| 50 | 65 | Dane | 13 | 1344 | I 39,90,94 | CT V | Diamond | 4.07 |
| 51 | 52 | Ozaukee | 45 | 4507 | I 43,ST 32,57 | CT C (Pioneer Rd) | Diamond | 4.04 |
| 52 | 60 | Brown | 5 | 530 | ST 172 | CT GV (Monroe Rd) | Diamond | 4.03 |
| 53 | 90 | Dane | 13 | 1340 | US 151 | Windsor St | Diamond | 4.01 |
| 54 | 95 | Dane | 13 | 1339 | US 151 | W Main St | Diamond | 4.01 |
| 55 | 59 | Waukesha | 67 | 6718 | I 43 (Rock Fwy) | CT O (Moorland Rd) | ParClo | 3.98 |
| 56 | 67 | Washington | 66 | 6606 | US 41 | ST 140 (Pioneer Rd) | Diamond | 3.98 |
| 57 | 96 | Dane | 13 | 1347 | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | 3.88 |
| 58 | 81 | Washington | 66 | 6613 | US 41,45 | US 45 | Three Leg Directional | 3.81 |
| 59 | 71 | Brown | 5 | 506 | I 43 | E Shore Dr (N Webster Ave) | ParClo | 3.68 |
| 60 | 92 | Winnebago | 70 | 7001 | US 41 | US 10,ST 441 | ParClo | 3.67 |
| 61 | 88 | St Croix | 55 | 5505 | I 94,US 12 | US 12 | Diamond | 3.62 |
| 62 | 135 | Dane | 13 | 1341 | US 151 | N Bristol St | Diamond | 3.54 |
| 63 | 69 | Outagamie | 44 | 4409 | US 41 | ST 15,CT OO (W Northland Ave) | ParClo | 3.31 |
| 64 | 86 | Fond Du Lac | 20 | 2007 | US 41 | CT B | Diamond | 3.25 |
| 65 | 107 | Washington | 66 | 6602 | US 41 | ST 33 | Diamond | 3.18 |
| 66 | 77 | St Croix | 55 | 5504 | I 94,US 12 | US 35 | Trumpet | 3.18 |
| 67 | 109 | Outagamie | 44 | 4413 | ST 441 | CT OO (E Northland Ave) | Diamond | 3.13 |
| 68 | 166 | Columbia | 11 | 1108 | I 90,94 | I 39 | Other | 3.10 |
| 69 | 70 | Brown | 5 | 527 | ST 172 | Pilgrim Way/Van Der Perren Way | Diamond | 3.04 |
| 70 | 146 | LaCrosse | 32 | 3202 | I 90 | Neshonoc Rd | Diamond | 2.96 |
| 71 | 84 | Dane | 13 | 1321 | ST 30 | Fair Oaks Ave | Diamond | 2.93 |
| 72 | 110 | Marathon | 37 | 3709 | US 51, ST 29 | CT N(Rib Mt Dr) | Diamond | 2.93 |
| 73 | 151 | Dane | 13 | 1303 | US 12,14 (Beltline Hwy) | US 14 (University Ave) | ParClo | 2.85 |
| 74 | 89 | Waukesha | 67 | 6701 | I 94 | ST 67 (Summit Ave) | ParClo | 2.84 |
| 75 | 195 | Eau Claire | 18 | 1804 | I 94 | US 53 | Cloverleaf | 2.74 |
| 76 | 118 | Fond Du Lac | 20 | 2003 | US 41 | ST 23 (W Johnson St) | Diamond | 2.67 |
| 77 | 75 | Marathon | 37 | 3708 | US 51, ST 29 | CT NN (N Mt Rd) | Diamond | 2.66 |
| 78 | 72 | Waukesha | 67 | 6702 | I 94 | CT P | Diamond | 2.63 |
| 79 | 62 | Waukesba | 67 | 6717 | I 43 (Rock Fwy) | CT Y (S Racine Ave) | Diamond | 2.61 |
| 80 | 191 | Waupaca | 68 | 6802 | US 10 | CT K | Diamond | 2.60 |
| 81 | 222 | Columbia | 11 | 1106 | I 39 | Cascade Mt Rd | ParClo | 2.55 |
| 82 | 40 | Brown | 5 | 520 | US 41 | CT VK (Hazelwood Ln/Lombardi Ave) | Diamond | 2.54 |
| 83 | 201 | Portage | 49 | 4914 | US 10 | CT J | ParClo | 2.52 |
| 84 | 115 | Brown | 5 | 505 | I 43 | Atkinson Dr | ParClo | 2.51 |
| 85 | 42 | Brown | 5 | 519 | US 41 | ST 32,54 (W Mason St) | Diamond | 2.51 |
| 86 | 43 | Winnebago | 70 | 7005 | US 41 | Breezewood Ln (W Bell St) | Diamond | 2.49 |
| 87 | 183 | Brown | 5 | 504 | ST 54,57 (Sturgeon Bay Rd) | University Ave | ParClo | 2.49 |
| 88 | 48 | Brown | 5 | 521 | US 41 | ST 172 ( Airport Dr) | ParClo | 2.46 |
| 89 | 184 | Dane | 13 | 1326 | US 14 | McCoy Rd | Diamond | 2.44 |
| 90 | 98 | Marathon | 37 | 3705 | US 51 | ST 52 | Diamond | 2.44 |
| 91 | 101 | Ozaukee | 45 | 4505 | I 43,ST 32,57 | ST 32 (Port Washington Rd) | Diamond | 2.41 |
| 92 | 99 | Brown | 5 | 517 | US 41 | US 141 (Velp Ave) | Diamond | 2.41 |
| 93 | 74 | Brown | 5 | 514 | US 41,141 | CT B (Sunset Beach Rd) | Diamond | 2.39 |
| 94 | 119 | Waukesha | 67 | 6716 | I 43 (Rock Fwy) | ST 164 (Big Bend Dr) | Diamond | 2.38 |
| 95 | 108 | Winnebago | 70 | 7010 | US 41 | ST 91,ST 44 | Diamond | 2.38 |
| 96 | 56 | Brown | 5 | 518 | US 41 | ST-29 (Shawano Ave) | Diamond | 2.37 |
| 97 | 57 | Outagamie | 44 | 4408 | US 41 | ST 47 (N Richmond St) | Diamond | 2.37 |
| 98 | 58 | Dane | 13 | 1304 | US 12,14 (Beltline Hwy) | Greenway Blvd | Diamond | 2.37 |
| 99 | 140 | Washington | 66 | 6611 | US 45 | ST 60 | Diamond | 2.36 |
| 100 | 120 | Marathon | 37 | 3706 | US 51 | ST 29 | Three Leg Directional | 2.35 |

Note: The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 17.00 |  | 7.51 |  |
|  | 11.01 | 16.99 | 4.97 | 7.50 |
|  | 7.42 | 11.00 | 3.04 | 4.96 |
|  | 4.96 | 7.41 | 1.56 | 3.03 |
|  |  | 4.95 |  | 1.55 |

Table D-3 Top 100 Unlighted Ramp Segments Concerning Lighting Benefit (crash/mile/year)

| Lighting Benefit Rank | Crash Risk Rank | County Name | County ID | Interch ID | On Road | Crossing Road | Interchange Type | Ramp Type | Direction | Reduction in Crash Density by Lighting (crash/mile/year) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Milwaukee | 40 | 4036 | I 94,US 41 (North South Fwy) | ST 100 (W Ryan Rd) | Diamond | Diamond | SB Exit | 2.36 |
| 2 | 2 | Dane | 13 | 1347 | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | Free Flow Loop | SB Entry | 2.21 |
| 3 | 3 | Brown | 5 | 518 | US 41 | ST 29 (Shawano Ave) | Diamond | Diamond | SB Entry | 1.95 |
| 4 | 5 | Winnebago | 70 | 7001 | US 41 | US 10,ST 441 | ParClo | Free Flow Loop | SB Entry | 1.93 |
| 5 | 7 | Marathon | 37 | 3722 | ST 29 | US 51 Bus | ParClo | ParClo Loop | WB Entry | 1.65 |
| 6 | 10 | Outagamie | 44 | 4414 | ST 441 | CT CE (E College Ave) | Diamond | Diamond | NB Entry | 1.60 |
| 7 | 9 | Outagamie | 44 | 4413 | ST 441 | CT OO (E Northland Ave) | Diamond | Diamond | SB Entry | 1.59 |
| 8 | 13 | Outagamie | 44 | 4410 | US 41 | ST 96 (W Wisconsin Ave) | Diamond | Diamond | SB Entry | 1.58 |
| 9 | 18 | Brown | 5 | 510 | I 43 | ST 172 | Three Leg Directional | Direct Connection | SB Exit | 1.55 |
| 10 | 8 | Dane | 13 | 1347 | I 39,90,94 | US 151 (E Washington Ave) | Cloverleaf | Outer Connection | NB Exit | 1.53 |
| 11 | 17 | Brown | 5 | 508 | I 43 | CT V (E Mason St) | Diamond | Diamond | SB Entry | 1.53 |
| 12 | 6 | Outagamie | 44 | 4411 | US 41 | ST 125 (W College Ave) | Diamond | Diamond | SB Entry | 1.50 |
| 13 | 29 | Brown | 5 | 516 | US 41,141 | I 43 | Trumpet | Direct Connection | SB Entry | 1.43 |
| 14 | 14 | Brown | 5 | 519 | US 41 | ST 32,54 (W Mason St) | Diamond | Diamond | SB Entry | 1.41 |
| 15 | 28 | Dane | 13 | 1306 | US 12,14 (Beltline Hwy) | Mineral Point Rd | Diamond | Diamond | NB Entry | 1.37 |
| 16 | 37 | LaCrosse | 32 | 3205 | I 90,US 53 | US 53,ST 35 | ParClo | Outer Connection | EB Entry | 1.37 |
| 17 | 61 | Brown | 5 | 517 | US 41 | US 141 (Velp Ave) | Diamond | Diamond | NB Entry | 1.36 |
| 18 | 39 | Dane | 13 | 1305 | US 12,14 (Beltline Hwy) | Old Sauk Rd | Diamond | Diamond | SB Entry | 1.33 |
| 19 | 54 | Waukesha | 67 | 6704 | I 94 | ST 83 | Diamond | Diamond | EB Entry | 1.33 |
| 20 | 36 | Outagamie | 44 | 4407 | US 41 | CT E (N Ballard Rd) | Diamond | Diamond | WB Entry | 1.31 |
| 21 | 22 | Outagamie | 44 | 4410 | US 41 | ST 96 (W Wisconsin Ave) | Diamond | Diamond | NB Entry | 1.31 |
| 22 | 25 | Outagamie | 44 | 4406 | US 41 | ST 441 | Trumpet | Direct Connection | EB Entry | 1.30 |
| 23 | 30 | Outagamie | 44 | 4409 | US 41 | ST 15,CT OO (W Northland Ave) | ParClo | Free Flow Loop | SB Entry | 1.28 |
| 24 | 42 | Outagamie | 44 | 4408 | US 41 | ST 47 (N Richmond St) | Diamond | Diamond | EB Entry | 1.28 |
| 25 | 58 | Outagamie | 44 | 4411 | US 41 | ST 125 (W College Ave) | Diamond | Diamond | NB Entry | 1.28 |
| 26 | 48 | Dane | 13 | 1349 | I 39,90,94 | I 94 | All Directional | Direct Connection | NB Entry | 1.27 |
| 27 | 33 | Outagamie | 44 | 4401 | ST 441 | CT KK (E Calumet St) | Diamond | Diamond | NB Entry | 1.26 |
| 28 | 50 | Winnebago | 70 | 7001 | US 41 | US 10,ST 441 | ParClo | Direct Connection | NB Entry | 1.26 |
| 29 | 41 | Outagamie | 44 | 4401 | ST 441 | CT KK (E Calumet St) | Diamond | Diamond | SB Entry | 1.25 |
| 30 | 53 | Dane | 13 | 1350 | I 39,90 | US 12,18 (Beltline Hwy) | Cloverleaf | Direct Connection | SB Entry | 1.25 |
| 31 | 12 | Winnebago | 70 | 7011 | US 41 | St-26 (Co Rd N) | Diamond | Diamond | NB Entry | 1.25 |
| 32 | 55 | Brown | 5 | 507 | I 43 | St-57, 54 (Sturgeon Bay Rd) | ParClo | Free Flow Loop | EB Entry | 1.24 |
| 33 | 38 | Brown | 5 | 530 | ST 172 | CT GV (Monroe Rd) | Diamond | Diamond | WB Entry | 1.23 |
| 34 | 35 | Marathon | 37 | 3710 | US 51, ST 29 | ST 29 | Three Leg Directional | Direct Connection | SB Exit | 1.22 |
| 35 | 31 | Winnebago | 70 | 7004 | US 41 | ST 114 (Winneconne Ave) | Diamond | Diamond | NB Entry | 1.21 |
| 36 | 75 | Eau Claire | 18 | 1810 | US 53 | ST 312 (N Crossing) | Diamond | Diamond | SB Entry | 1.20 |
| 37 | 40 | Brown | 5 | 507 | I 43 | St-57, 54 (Sturgeon Bay Rd) | ParClo | Direct Connection | WB Entry | 1.20 |
| 38 | 15 | Outagamie | 44 | 4413 | ST 441 | CT OO (E Northland Ave) | Diamond | Diamond | NB Exit | 1.20 |
| 39 | 57 | Walworth | 64 | 6413 | US 12 | ST 50 | ParClo | Outer Connection | NB Entry | 1.18 |
| 40 | 74 | Waukesha | 67 | 6729 | ST 16 | ST 190 (Capitol Dr) | Diamond | Diamond | NB Entry | 1.17 |
| 41 | 76 | Outagamie | 44 | 4409 | US 41 | ST 15,CT OO (W Northland Ave) | ParClo | Direct Connection | NB Entry | 1.17 |
| 42 | 51 | Dane | 13 | 1337 | US 151 (E Washington Ave) | American Pkwy | ParClo | Outer Connection | SB Entry | 1.16 |
| 43 | 23 | Waukesha | 67 | 6710 | I 94 | CT F (Redford Blvd) | Diamond | Diamond | WB Entry | 1.16 |
| 44 | 60 | Marathon | 37 | 3723 | ST 29 | CT X(Camp Philips Rd) | Diamond | Diamond | WB Entry | 1.15 |
| 45 | 83 | Brown | 5 | 515 | US 41,141 | CT M (Lineville Rd) | Diamond | Diamond | SB Entry | 1.15 |
| 46 | 84 | Dane | 13 | 1349 | I 39,90,94 | I 94 | All Directional | Direct Connection | SB Entry | 1.15 |
| 47 | 24 | Brown | 5 | 521 | US 41 | ST 172 ( Airport Dr) | ParClo | Outer Connection | NB Exit | 1.14 |
| 48 | 70 | Washington | 66 | 6608 | US 45 | ST 144 | Diamond | Diamond | SB Entry | 1.13 |
| 49 | 65 | Dane | 13 | 1304 | US 12,14 (Beltline Hwy) | Greenway Blvd | Diamond | Diamond | SB Entry | 1.12 |
| 50 | 105 | LaCrosse | 32 | 3208 | US 53 | CT S (Sand Lake Rd) | Diamond | Diamond | SB Entry | 1.11 |
| 51 | 82 | Chippewa | 9 | 907 | US 53 | ST 29 | Cloverleaf | Free Flow Loop | SB Entry | 1.10 |
| 52 | 32 | Brown | 5 | 521 | US 41 | ST 172 ( Airport Dr) | ParClo | Outer Connection | SB Exit | 1.10 |
| 53 | 62 | Marathon | 37 | 3712 | US 51 | Maple Ridge Rd | Diamond | Diamond | NB Entry | 1.09 |
| 54 | 67 | Outagamie | 44 | 4408 | US 41 | ST 47 (N Richmond St) | Diamond | Diamond | WB Entry | 1.08 |
| 55 | 72 | Dane | 13 | 1337 | US 151 (E Washington Ave) | American Pkwy | ParClo | ParCloLoop | SB Entry | 1.07 |
| 56 | 26 | Brown | 5 | 507 | I 43 | St-57, 54 (Sturgeon Bay Rd) | ParClo | Outer Connection | WB Exit | 1.05 |
| 57 | 125 | Rock | 53 | 5307 | I 39,90 | I 43,ST 81 | Cloverleaf | Free Flow Loop | SB Entry | 1.04 |
| 58 | 114 | Portage | 49 | 4908 | I 39,US 51 | CT B(Plover Rd) | Diamond | Diamond | NB Entry | 1.04 |
| 59 | 173 | Eau Claire | 18 | 1804 | I 94 | US 53 | Cloverleaf | Free Flow Loop | EB Entry | 1.02 |
| 60 | 102 | Outagamie | 44 | 4412 | US 41 | CT BB (W Prospect Ave) | Diamond | Diamond | NB Entry | 1.02 |
| 61 | 44 | Washington | 66 | 6613 | US 41,45 | US 45 | Three Leg Directional | Direct Connection | NB Exit | 1.02 |
| 62 | 122 | Brown | 5 | 520 | US 41 | CT VK (Hazelwood Ln/Lombardi Ave) | Diamond | Diamond | SB Entry | 1.01 |
| 63 | 116 | Sheboygan | 59 | 5903 | I 43 | ST 28 (Washington Ave) | Diamond | Diamond | NB Entry | 1.00 |
| 64 | 113 | Dane | 13 | 1306 | US 12,14 (Beltline Hwy) | Mineral Point Rd | Diamond | Diamond | SB Entry | 1.00 |
| 65 | 78 | Waukesha | 67 | 6710 | I 94 | CT F (Redford Blvd) | Diamond | Diamond | WB Exit | 0.99 |
| 66 | 129 | Dane | 13 | 1321 | ST 30 | Fair Oaks Ave | Diamond | Diamond | EB Entry | 0.99 |
| 67 | 130 | Waukesha | 67 | 6701 | I 94 | ST 67 (Summit Ave) | ParClo | ParClo Loop | EB Entry | 0.99 |
| 68 | 151 | Brown | 5 | 511 | I 43 | US 141 ( Elm View Rd) | Diamond | Diamond | WB Entry | 0.98 |
| 69 | 45 | Brown | 5 | 527 | ST 172 | Pilgrim Way/Van Der Perren Way | Diamond | Diamond | WB Exit | 0.98 |
| 70 | 137 | Brown | 5 | 520 | US 41 | CT VK (Hazelwood Ln/Lombardi Ave) | Diamond | Diamond | NB Entry | 0.98 |
| 71 | 97 | Brown | 5 | 508 | I 43 | CT V (E Mason St) | Diamond | Diamond | NB Entry | 0.97 |
| 72 | 138 | Washington | 66 | 6615 | US 41,45 | ST 167 (Lannon Rd) | Diamond | Diamond | NB Entry | 0.97 |
| 73 | 68 | Marathon | 37 | 3722 | ST 29 | US 51 Bus | ParClo | ParClo Loop | EB Exit | 0.97 |
| 74 | 142 | Brown | 5 | 514 | US 41,141 | CT B (Sunset Beach Rd) | Diamond | Diamond | SB Entry | 0.97 |
| 75 | 175 | Manitowoc | 36 | 3606 | I 43 | US 151,ST 42 (Calumet Ave) | Diamond | Diamond | SB Entry | 0.96 |
| 76 | 87 | Brown | 5 | 530 | ST 172 | CT GV (Monroe Rd) | Diamond | Diamond | EB Exit | 0.95 |
| 77 | 19 | Outagamie | 44 | 4411 | US 41 | ST 125 (W College Ave) | Diamond | Diamond | NB Exit | 0.95 |
| 78 | 127 | Sheboygan | 59 | 5908 | ST 23 | CT Y (Highland Dr) | ParClo | Diamond | EB Entry | 0.95 |
| 79 | 164 | LaCrosse | 32 | 3209 | US 53 | CT OT | ParClo | Outer Connection | SB Entry | 0.95 |
| 80 | 120 | Brown | 5 | 519 | US 41 | ST 32,54 (W Mason St) | Diamond | Diamond | NB Entry | 0.94 |
| 81 | 166 | Marathon | 37 | 3703 | US 51 | CT U (Merill Ave) | Half-Diamond | Diamond | SB Entry | 0.94 |
| 82 | 148 | Rock | 53 | 5302 | I 39,90 | ST 26 (Milton Ave) | ParClo | ParClo Loop | SB Entry | 0.94 |
| 83 | 92 | Brown | 5 | 509 | I 43 | Manitowoc Rd | ParClo | ParClo Loop | SB Entry | 0.94 |
| 84 | 4 | Marathon | 37 | 3710 | US 51, ST 29 | ST 29 | Three Leg Directional | Direct Connection | NB Entry | 0.94 |
| 85 | 134 | Dane | 13 | 1340 | US 151 | Windsor St | Diamond | Diamond | SB Entry | 0.94 |
| 86 | 109 | Outagamie | 44 | 4412 | US 41 | CT BB (W Prospect Ave) | Diamond | Diamond | SB Entry | 0.93 |
| 87 | 118 | Brown | 5 | 517 | US 41 | US 141 (Velp Ave) | Diamond | Diamond | SB Entry | 0.93 |
| 88 | 160 | Portage | 49 | 4907 | I 39,US 51 | CT HH(McDill Ave) | ParClo | Outer Connection | NB Entry | 0.92 |
| 89 | 121 | Eau Claire | 18 | 1810 | US 53 | ST 312 (N Crossing) | Diamond | Diamond | EB Entry | 0.92 |
| 90 | 192 | Eau Claire | 18 | 1803 | I 94 | ST 93 | Diamond | Diamond | WB Entry | 0.92 |
| 91 | 266 | Dane | 13 | 1338 | US 151 (E Washington Ave) | Reiner Rd | Diamond | Diamond | SB Entry | 0.91 |
| 92 | 168 | Waukesha | 67 | 6729 | ST 16 | ST 190 (Capitol Dr) | Diamond | Diamond | SB Entry | 0.91 |
| 93 | 126 | Marathon | 37 | 3707 | US 51 | Sherman St | Half-Diamond | Diamond | SB Entry | 0.90 |
| 94 | 150 | Brown | 5 | 527 | ST 172 | Pilgrim Way/Van Der Perren Way | Diamond | Diamond | WB Entry | 0.90 |
| 95 | 117 | Outagamie | 44 | 4407 | US 41 | CT E (N Ballard Rd) | Diamond | Diamond | EB Entry | 0.90 |
| 96 | 79 | Columbia | 11 | 1108 | I 90,94 | I 39 | Other | Outer Connection | NB Entry | 0.90 |
| 97 | 132 | Rock | 53 | 5303 | I 39,90 | US 14 (Humes Rd) | ParClo | Outer Connection | SB Entry | 0.89 |
| 98 | 139 | Outagamie | 44 | 4405 | US 41 | CT N (Freedom Rd) | Diamond | Diamond | WB Entry | 0.88 |
| 99 | 229 | Brown | 5 | 504 | ST 54,57 (Sturgeon Bay Rd) | University Ave | ParClo | Outer Connection | WB Entry | 0.88 |
| 100 | 230 | Brown | 5 | 506 | I 43 | E Shore Dr (N Webster Ave) | ParClo | Outer Connection | EB Entry | 0.88 |

Note: The criteria for defining colors in the risk and benefit ranks are as follows:

|  | Crash Risk |  | Lighting Benefit |  |
| --- | --- | --- | --- | --- |
|  | (nighttime crash/mile/year) |  | (nighttime crash/mile/year) |  |
|  | Lower Bound | Upper Bound | Lower Bound | Upper Bound |
|  | 2.51 |  | 1.08 |  |
|  | 1.58 | 2.50 | 0.62 | 1.07 |
|  | 1.00 | 1.57 | 0.03 | 0.61 |
|  | 0.55 | 0.99 | 0.08 | 0.30 |
|  |  | 0.54 |  | 0.07 |