TR-55 (Just out of range)

| Cover | Surface Cond | Soil Type | CN | % Area | CN*% Area |
| --- | --- | --- | --- | --- | --- |
| Row Crops | Straight Row | C | 87 | 62 | 5394 |
| Row Crops | Straight Row | D | 90 | 8 | 900 |
| Woods | Dense | C | 70 | 5 | 350 |
| Farmsteads | Normal | C | 82 | 8 | 656 |
| Suburb | N/A | C | 82.5 | 15 | 1237.5 |
| Impervious Surf | N/A | N/A | 100 | 2 | 200 |
|  |  |  |  | Sum= | 8737.5 |
|  |  |  |  | CN=Sum/100 | 87 |

P2=2-year, 24-hour=2.5in

Tc, time of concentration = Tt1+ Tt2 + Tt3 = 0.17 + 0.65 + 6.35 = 7.17

Sheet Flow, Tt1 = 0.007(nL)0.8/(P20.5*s0.4) = 0.007(0.05*300)0.8/(2.50.5*0.0230.4) = 0.17 hrs

n, Manning’s = 0.05 (Fallow Row Crops)

L, length = 300 ft

s, slope = 0.023

Shallow Concentrated Flow, Tt2 = L/(3600*V) = 4940/(3600*2.1) = 0.653 hrs

L, Length = 4940 ft

s = 0.017

V, average velocity = 2.1 ft/s, from https://www.wcc.nrcs.usda.gov/ftpref/wntsc/H&H/other/TR55documentation.pdf (Figure 3-1)

Open Channel Flow, Tt3 = L/(3600*V) = 6.35 hrs

V = 1.49r2/3s0.5/n = 1.49*0.4782/3*0.00180.5/0.05 = 0.77 ft/s

r, Hydraulic Radius = a/p = 1.91/4 = 0.4775

s, average slope = 0.0018

n, Manning’s = 0.05 (Unmaintained channel, clean bottom, brush on sides)

Fp= % storage adjustment = 0.90 (Table 4-2)

https://www.wcc.nrcs.usda.gov/ftpref/wntsc/H&H/other/TR55documentation.pdf

Am, Area= 3.6mi2 from Watershed Analysis

Q, Runoff Depth = 3.49 in from https://www.wcc.nrcs.usda.gov/ftpref/wntsc/H&H/other/TR55documentation.pdf (Table 2-1)

P, 50 year, 24 hour rainfall= 4.8 inches from https://www.wcc.nrcs.usda.gov/ftpref/wntsc/H&H/other/TR55documentation.pdf (Figure B-7)

Tt, Time of Concentration = 0.17+0.65+6.35hr = 7.17 hrs

Ia, Initial Abstraction=0.299, for CN 87 from https://www.wcc.nrcs.usda.gov/ftpref/wntsc/H&H/other/TR55documentation.pdf (Table 4-1)

Ia/P = 0.299/4.8 = 0.0623

Rainfall Distribution Type II

Qu=95csm/in (Exhibit 4-II)

Qp=quAmQFp

Qp=95csm/in*3.6mi2*3.49in*0.9

Qp= 1074 cfs

Check 100-yr Storm

P, 100 year, 24 hour rainfall= 5.2 in (Figure B-8)

Ia/P = 0.299/5.8 = 0.0516

Flood-Frequency Characteristics of Wisconsin Streams, 1992

[A, contributing drainage area in square miles; S, main-channel slope in feet per mile; I25, 25-year, 24-hour precipitation intensity, in inches minus 4.2; ST, storage, in percent of basin area plus 1.0; SP, soil permeability of the least-permeable soil horizon in inches per hour; SN, mean annual snowfall for 1961 through 1990 in inches; FOR, forest cover in percent of basin area plus 1; Qn, peak flood discharge in cubic feet per second, with an n-year recurrence interval; SE, standard error of estimate of regression, in log units; ESE, equivalent standard error of estimate, in percent. Flood frequency areas are shown in figure 3.]

https://pubs.usgs.gov/wri/wri034250/

Area 3

Q50=127.0A0.819*SP-0.761*ST-0.215*I250.136

Q50=127.0*3.60.819*0.1-0.761*1.9-0.215*0.260.136 = 1517 cfs

A=3.6 sq. miles (See Watershed Analysis)

SP=0.1in/hr (0.05-0.2 in/hr median 0.1)

ST=1.9%, 0.9% Storage = Storage Area/A = 0.03 sq. mi/3.59 sq. mi

I25=4.46-4.2=0.26 (from Figure 1 https://pubs.usgs.gov/wri/wri034250/)

Standard error of the logarithm of discharge: SE=0.1446

Equivalent Standard Error in percent of predicted discharge ESE=34

Area 4

Q50=16.6*A.857*ST-0.293*S0.357*SP-0.238*SN0.281

Q50=16.6*3.60.857*1.9-0.293*10.90.357*0.1-0.238*400.281=472 cfs

Standard error of the logarithm of discharge: SE=0.0964

Equivalent Standard Error in percent of predicted discharge ESE=22

The watershed falls on the line between two Areas, so the average of the two was taken in accordance to the guidance.

Average Discharge Q100 = (1517+472)/2

Q100= 995 cfs

Check 100-yr Storm

Area 3

Q100=149.0A0.818*SP-0.775*ST-0.227*I250.136

Q100=149.0*3.60.818*0.1-0.775*1.9-0.227*0.260.136

Q100=1821 cfs

Standard error of the logarithm of discharge: SE=0.1466

Equivalent Standard Error in percent of predicted discharge ESE=34

Area 4

Q100=19.4*A0.857*ST-0.296*S0.371*SP-0.229*SN0.269

Q100=19.4*3.60.857*1.9-0.296*10.90.371*0.1-0.229*400.269

Q100=533 cfs

Standard error of the logarithm of discharge: SE=0.0954

Equivalent Standard Error in percent of predicted discharge ESE=22

The watershed falls on the line between two Areas, so the average of the two was taken in accordance to the guidance.

Average Discharge Q100 = (1821+533)/2

Q100= 1177 cfs

USGS Gaged Stream Adjustment Method

Note: design drainage area is 28% of the size of the gauged stream drainage stream, which falls outside of the minimum 50%.

r = Qa/Qr=2761/4267=0.647

Qa=2761cfs

Qr=127.0*12.730.819*0.1-0.761*1.9-0.215*0.260.136=4267 cfs

r= Adjustment ratio

r’= difference in drainage area

A=difference in area of ungauged and gauged site = 12.73mi2-3.6mi2=8.4 mi2

Ag=drainage area of gauged site= 12.73mi2

r’=r-[A/(0.5*Ag)*(r1.0)]= 0.647-[8.4mi2/(0.5*12.73 mi2)*( 0.647-1.0)]=-4.38

Qw=Qrug*r’=64.4cfs*-4.38=282cfs

Techniques for Estimating Magnitude and Frequency of Floods for Wisconsin, 1981

Area 3

https://pubs.usgs.gov/of/1980/1214/report.pdf

Q50=143.2*A0.845S0.281FOR-0.137*I24,21.68*SP-0.613

A, Area= 3.6 mi2

S, Slope= (847.5’-819.8’)/2.69mi = 10.3 ft/mi

FOR, Forest Cover= 5+1= 6

I24,2, 2-yr, 24-hour Intensity= (2.55 in/hr/3)4 = 0.522

SP, Soil Permeability= 0.5 in/hr

Q50=143.2*3.60.845*10.30.281*6-0.137*0.5221.68*0.5-0.613 = 327 cfs

Area 4

Q50=7.63*A0.837S0.276ST-0.249*SN0.540*SP-0.300

Q50=7.63*3.60.83710.30.2761.9-0.249*390.540*0.5-0.300= 322 cfs

A, Area= 3.6 mi2

S, Slope= (847.5’-819.8’)/2.69mi = 10.3 ft/mi

ST=Storage=0.9+1=1.9

SN=Mean Annual Snowfall=39

SP, Soil Permeability= 0.5 in/hr

USGS Transfer Method, 1981

Qud=Qg*(Aud/Ag)n=2500*(3.6/12)0.76=1001 cfs

Qud=design discharge

Qg=discharge at gauge site=2500 cfs

Aud=design drainage area=3.6mi2

Ag=gauge station drainage area=12.0mi2

n=regional drainage exponent (area 3)=0.76

Ranges of basin characteristics used in regression analysis:

| Area 3 (57 stations) |  |  |  |
| --- | --- | --- | --- |
| Drainage area (mi2) | 1.00 | 22 | 2,240 |
| Soil permeability (in/hr) | .12 | 1.81 | 8.46 |
| Storage (percent) | .00 | 15.5 | 39.7 |
| 25-year, 24-hour precipitation (in.) | 4.24 | 4.38 | 5.29 |

Stream Gauge Data

Sawyer Creek

Area= 12.73 mi2

| Recurrence (Yr) | Peak Flow (cfs) |
| --- | --- |
| 2 | 507.8 |
| 5 | 1007 |
| 10 | 1448 |
| 25 | 2140 |
| 50 | 2761 |
| 100 | 3477 |
| 200 | 4299 |

| Station number | Station name | Contributing area(mi2) | Slope(ft/mi) | Storage(percent) | Forest(percent) | 25-yr, 24-hrprecipitation(in.) | Snowfall(in.) | Soilpermeability(in./hr) | Q100 byregression(ft3/s) | Flood-frequency area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 04081900 | Sawyer Creek at Oshkosh, Wis. | 12.0 | 11.0 | .33 | 1.96 | 4.46 | 45.0 | .12 | 4,607 | 3 |

| Stationnumber | Station name | WRC skew | Discharge for indicated recurrence interval |  |  |  |  |  |  |  | Type | Period of record |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 |  | 5 | 10 | 25 | 50 | 100 | SE100 |  |  |
| 04081900 | Sawyer Creek at Oshkosh, Wis. | -.234 |  | 554 |  |  |  |  |  |  |  |  |  | 1,030 | 1,400 | 1,920 | 2,330 | 2,770 | .09267 | C | 1961–2000 |