The Adverse Conditions Communications/Coordination plan is a guide and reference for WisDOT to use in response to potential weather conditions that may severely affect the interstate system.  The plan provides communication guidelines to ensure that WisDOT is able to monitor conditions before and during a major weather event and possibly prevent interstate failure.  This section supplements the Adverse Conditions Communications/Coordination plan and gives STOC Control Room Operators detailed instructions on their duties.

## Area of Influence

The area of influence for the communications plan includes the counties within Wisconsin that have interstate roadways and connect to major destinations.  A map of these areas is displayed in the Adverse Conditions Communications/Coordination plan. There is a copy of this map at every work station. All affected counties are highlighted in yellow on the map.

## Adverse Weather Communications/Coordination Procedures

There are two trigger-points that require the STOC to make notifications.  Those trigger-point thresholds are:

- Request for Assistance

- Potential Interstate Closure

Listed below are instructions of who the STOC is to contact in the event of a trigger-point threshold.  The complete communication flow diagrams are located in the Adverse Conditions Communication/Coordination plan located in the control room.

### Request for Assistance:

If a County Highway Department is in need of resources from a neighboring county, the County Highway Department will call the STOC to request assistance (road treatment material is needed, additional plows are needed, etc).  The control room will first call the Control Room Manager, and then contact:

- The appropriate WisDOT RIMC. The RIMC is to contact their Regional Duty Officer.

- The on-call STOC Supervisor, and inform them of the Resource Request.  The STOC Supervisor is to contact the STOC Manager.

- The appropriate Regional Duty Officer.  If the request was fulfilled, contact the RDO and notify them of the request and its status, between the hours of 0700-2200.  If the request was unable to be supported, contact the RDO immediately, regardless of the hour. (Dual RDO notification is  per DTSD management request to ensure notification)

- Call the WEM Duty Officer: 800-943-0003 x2

### Potential Interstate Closure

In the event that a roadway may need to be closed (poor visibility, driver hours have exceeded safe levels, impassable, etc) the County Highway Department/Law Enforcement agency will call the STOC.

- The control room will first call the Control Room Manager, and then contact:

- The appropriate WisDOT RIMC. The RIMC is to contact their Regional Duty Officer and inform them of the worsening conditions and the possibility of closing the roadway.

- Call the on-call STOC Supervisor and inform them of the Potential Interstate Closure.  The STOC Supervisor is to contact the STOC Manager.

- Call that county’s State Patrol Communication Center for informational purposes.

- Call the WEM Duty Officer: 800-943-0003 x2

## Contact Information

All contact information is located in the Adverse Conditions Communications Coordination plan, located on the ETO Resource Repository webpage. There is also a hard copy located in the control room.

Adverse Conditions Communications Flow Diagram