Placing “Adverse Conditions” messages on DMS or PCMS:

When conditions warrant, the STOC can implement messages on DMSs to warn motorists about adverse driving conditions.  Operators should closely monitor roadway conditions and consult with law enforcement if they believe adverse conditions signing might be appropriate.

Operators should also consider the following information before implementing adverse conditions messages.

- Has the STOC received a request for signing from a law enforcement agency or county highway department?

- Is there a Winter Storm Warning in effect for the County or area?

  - If so, use the warning’s expiration time as a guide for how to long to place the message on the DMS

- Is there a significant increase in the number of traffic incidents in that county/area?

If the operator feels that conditions are appropriate for adverse conditions signing, TIS plan 6 can be used for Milwaukee County and TIS plan 7 can be used for Waukesha County. These plans display the following message:

Phase one:  	      Phase two:

| ICY ROADS REDUCE SPEED | TRAVEL TIMES |
| --- | --- |

*Use of a DMS to notify traffic of an incident will take priority over adverse conditions messages.

For other adverse conditions or for signs not included in the above plans, the following messages are examples that may be used:

| FIRE |  |  |  |
| --- | --- | --- | --- |
| Phase 1 Message: | SMOKE OVER ROADWAY REDUCED VISIBILITY REDUCE SPEED | Phase 2 Message: |  |
| FLOODING |  |  |  |
| Phase 1 Message: | FLOODING (Cross Street/Location) | Phase 2 Message: | ROAD CLOSED USE (Alternate Route) |
| Phase 1 Message: | FLOODING (#) MILES AHEAD MERGE LEFT | Phase 2 Message: |  |
| Phase 1 Message: | FLOODING AT (Cross Street/Location) | Phase 2 Message: | (Audience) USE (Alternate Route) |
| Phase 1 Message: | (LOCATION) WATCH FOR FLOODING DEBRIS ON ROADWAY | Phase 2 Message: |  |
| Phase 1 Message: | WATER ON ROADWAY AT (LOCATION) | Phase 2 Message: |  |
| FOG |  |  |  |
| Phase 1 Message: | DENSE FOG | Phase 2 Message: |  |
| Phase 1 Message: | DENSE FOG ON BRIDGE | Phase 2 Message: | REDUCE SPEED |
| Phase 1 Message: | DENSE FOG LOW VISABILITY | Phase 2 Message: | REDUCE SPEED |
| Phase 1 Message: | PATCHES OF FOG AHEAD | Phase 2 Message: | REDUCE SPEED USE HEADLIGHTS |
| Phase 1 Message: | DENSE FOG REDUCE SPEED | Phase 2 Message: |  |
| Phase 1 Message: | DENSE FOG NEXT (#) MILES | Phase 2 Message: | REDUCE SPEED |
| ICE |  |  |  |
| Phase 1 Message: | ICY ROADS REDUCE SPEED | Phase 2 Message: | DO NOT USE CRUISE CONTROL |
| Phase 1 Message: | ICY ROADS NEXT (#) MILES | Phase 2 Message: |  |
| Phase 1 Message: | SLIPPERY ROADS REDUCE SPEED | Phase 2 Message: |  |
| Phase 1 Message: | ICY CONDITIONS POSSIBLE REDUCE SPEED | Phase 2 Message: |  |

| SNOW |  |  |  |
| --- | --- | --- | --- |
| Phase 1 Message: | BLOWING SNOW (Location) | Phase 2 Message: |  |
| Phase 1 Message: | BLOWING SNOW (Location) | Phase 2 Message: | REDUCED VISIBILITY REDUCE SPEED |
| Phase 1 Message: | BLOWING SNOW REDUCE SPEED | Phase 2 Message: |  |
| Phase 1 Message: | SNOW COVERED ROADS REDUCE SPEED | Phase 2 Message: |  |
| Phase 1 Message: | BLOWING SNOW PATCHES OF ICE NEXT 30 MILES | Phase 2 Message: | REDUCE SPEED |
| Phase 1 Message: | HIGH WINDS BLOWING SNOW REDUCE SPEED | Phase 2 Message: |  |
| WIND |  |  |  |
| Phase 1 Message: | HIGH WINDS ON BRIDGE | Phase 2 Message: |  |
| Phase 1 Message: | HIGH WINDS ON BRIDGE | Phase 2 Message: | TRUCKS AND RVS TRAVEL NOT ADVISED ROLLOVER RISK |
| Phase 1 Message: | STRONG WIND GUSTS (Number)+ MPH | Phase 2 Message: | TRUCKS AND RVS TRAVEL NOT ADVISED |
| Phase 1 Message: | STRONG WIND GUSTS (Number)+ MPH | Phase 2 Message: | TRUCKS AND RVS MUST USE EXIT (#) FOLLOW ALT ROUTE |
| OTHER |  |  |  |
| Phase 1 Message: | MAJOR WINTER STORM TRAVEL RESTRICTED BTWN (LOCATIONS) | Phase 2 Message: | EXIT AT (LOCATION) |
| Phase 1 Message: | BRIDGE CLOSED (#) MILES AHEAD | Phase 2 Message: | ALL TRAFFIC MUST EXIT AT (location) |
| Phase 1 Message: | WATCH FOR PLOW REDUCE SPEED | Phase 2 Message: |  |

***Please send an email to the DOT DL STOC Internal Notification email list when any adverse condition signing is implemented***

Placing “Adverse Conditions” floodgate messages on 511 (Snow, Fog, Flooding, etc):

Similarly, an operator may put either a Statewide or County level floodgate message on 511 if conditions warrant

***Because of the frequency of Lake Effect Snow Warnings for the Iron and Ashland zones, floodgate messages for these areas will be placed at the County level of all counties identified in the warning, and that show either Snow Covered, Ice Covered, or Impassable road conditions on 511***

Floodgate template:

“As of (time/date) Law Enforcement is reporting adverse road conditions in (much of the state, northern/southern part of the state, etc).  Motorists are encouraged slow down and avoid travel when possible.”

The audio and text floodgate messages should be taken down when the Winter Storm Warning expires or State Patrol reports that conditions have improved.