Overview

Over the past several years high wind conditions have affected driving conditions on the Leo Frigo Bridge, spanning the Fox River in Green Bay.  From information learned during high wind events in October 2010 and September 2011, WisDOT has adopted the following mitigation procedures.

When a Wind Advisory is issued for Green Bay by the National Weather Service that includes a recorded wind gust of 45 mph or greater:

- The reporting agency will contact State Patrol and the STOC with the weather information.

- State Patrol will position a cruiser at each end of bridge or request assistance, as needed.

- WisDOT (RCM or STOC, upon request) will issue a media release* advising of possible high winds & caution to motorists using the bridge; and suggest use of alternate routes, especially higher profile vehicles. *see template attached

- STOC will activate message boards on IH 43 SB/EB @ Military Avenue (VMS 3509) and IH 43 NB/WB @ S. of Webster Avenue (VMS 3510) with the following messages:

ADVISORY

SPEED

45 MPH

WIND ADVISORY

EXPECT HIGH WINDS

ON BRIDGE

When a Wind Warning is issued for Green Bay by the National Weather Service that includes a recorded wind gust of 58 mph or greater:

- The reporting agency will contact State Patrol and the STOC with the need for closure of the bridge.

- State Patrol will enact closure with their forces or others, as needed.

- WisDOT (RCM or STOC STOC, upon request) will issue a media release* advising closure of the bridge due to hazardous winds and that traffic is being directed to a designated alternate route at bridge. *see template attached

- STOC will activate message boards on IH 43 SB/EB @ Military Avenue (VMS 3509) and IH 43 NB/WB @ S. of Webster Avenue (VMS 3510) with the following messages:

ALL TRAFFIC

MUST

EXIT RIGHT

BRIDGE

CLOSED

AHEAD

- STOC will activate message boards on USH 41 SB at CTH B (PCMS 03-3006) and on USH 41 NB north of CTH F (VMS 3029) with the following messages:

I-43

BRIDGE

CLOSED

USE

ALT

ROUTE

- STOC will activate message boards on IH 43 NB at N. of County MM (VMS 3512) with the following messages:

USE

ALTERNATE

ROUTE

I-43

BRIDGE CLOSED

9 MILES AHEAD

- STOC will activate message boards on STH 54/57 WB @ County EA (VMS 3511) with the following messages:

USE

ALTERNATE

ROUTE

I-43 NORTHBOUND

BRIDGE CLOSED

3 MILES AHEAD

- Bridge to be re-opened to traffic after winds subside to 45 mph or less for one hour.

- WisDOT (RCM or STOC) would be asked to provide final media release.

STOC:  800-375-7302

Duty Officer:  920-492-7711

State Patrol Dispatch in Fond du Lac:  920-929-3703

Cullen Peltier (Brown County EOC):  920-391-7430

# National Weather Service Definitions:

Wind Advisory

Sustained winds 30+ mph (26 kts) for 1 hour or any gust of at least 45 mph (39 kts) for any duration is expected or occurring.

High Wind Warning

Sustained winds 40+ mph (35 kts) for 1 hour or any gusts of 58 mph (50 kts) or greater for any duration is expected or occurring.