Teleste VMX

Teleste VMX is the software used to view and record the CCTV cameras that monitor the freeway system in Wisconsin. There is limited CCTV coverage in each of the DOT regions with extensive CCTV coverage in the SE Region Milwaukee area. CCTV camera images provide WisDOT and other first responders a valuable tool for not only monitoring traffic conditions, but also for monitoring activities at major incident scenes.

Using Teleste VMX

- Click the               icon to open VMX. The following login window will appear:

- User name and Password will be provided by the Control Room Supervisor.

2. Enter User name and Password and click Login.  The VMX user interface will open:

Viewing Window

Video Source Tree

3. The Video Source Tree (on the right-hand side of the screen) lists all available video sources (CCTV cameras) which can be viewed using VMX.  A camera listing in green font signifies that the camera is ready for use.  Red font signifies that the camera is unavailable, and can be caused by hardware or communication failure.  Camera listings may also turn red briefly during a reboot of the VMX underlying database.  A camera listing in black font signifies that the camera is locked for use by another VMS user.

- When a user begins controlling a camera, that camera is “locked” to that user, and cannot be used by any other users until it is unlocked.  A camera unlocks automatically after a short period of non-use.  Cameras can also be unlocked by using the menu in the upper left: With the camera highlighted in the Video Source Tree, ControlUnlock.

4. To view a camera feed, click on the camera listing in the Video Source Tree, hold down the left mouse button, and drag to an open Viewing Window.  The camera name/location is reflected at the top of the Viewing Window, as well as overlaid across the video image.  Most cameras also have a compass direction overlaid across the top of the Viewing Window.

- Double click in the Viewing Window to enter full screen mode.  Double click again to return to standard view.

5. To control a camera,

- Click on the camera image in the Viewing Window, and click             on the toolbar, or right-click on the camera image in the Viewing Window and select Control from the menu.  The Control Window will open:

Zoom

Focus

Iris

Presets

Virtual Joystick

The camera is then controlled using the mouse to click and/or drag the camera controls as desired.  Each camera also has a list of preset views, usually of each cardinal compass direction.  To use a preset view, click on it, then click the Set button.

.

- The STOC has primary control of the cameras. Agencies other than the STOC (i.e. Wisconsin State Patrol) have the ability to control WisDOT cameras.  As a courtesy, unless there is an immediate STOC need for a camera, do not unlock or attempt to control a camera if it is being controlled by another agency.

- IMPORTANT: WisDOT camera feeds are viewed by the public as well as several different government and media agencies.  When no longer controlling a camera, ALWAYS move it back to a zoomed-out view of the freeway.  This can be done quickly by using camera presets.

6.  To select different Teleste VMX user interface views:

- Select one of the View buttons from the Toolbar:

Map View

Dual Screen

Tree View

Quad View

- Quad View:

- Tree View (Default View):

The Teleste VMX User Interface is highly customizable.  All sections on the screen can be moved and resized by clicking and dragging:

Security Cameras

Some cameras monitored at the STOC are security cameras and they have ability to monitor key infrastructure locations, such as around the Hoan Bridge and around the Marquette Interchange. These images should only be shared with law enforcement, the Coast Guard, or the Port Authority. Listed below are the security cameras:

- I-794 @ Hoan Bridge & Summerfest

- I-794 @ South Hoan Bridge Lower

- I-794 @ South Hoan Bridge Upper

- I-794 @ North Hoan Bridge Lower

- I-794 @ North Hoan Bridge Upper

- I-794 @ Car Ferry Lower

- I-794 @ Car Ferry Upper

- I-794 @ Layton Ave

- I-794 @ James Lovell Lower

- I-94 @ 13th St Lower

Archived Video Overview

The purpose of the Archived Video is to capture incidents and events that can be used for preventative safety measures, future training possibilities, and the incorporation into STOC-related presentations.

Currently, all of the cameras on the digital video system are being archived for 72 hrs.  The video files are divided into 15-minute intervals throughout the day.  Each evening at midnight the oldest day’s files are deleted.

# Incidents/Events to Save*

- All incidents classified as a severity type 3 or greater should be saved.  All video files of the incident for the duration of the incident should be saved.

- Any incident/event that can be categorized as unusual (i.e. large car fire, animal on freeway, protesters on/above freeway causing delays, etc).

- Any incident/event that can be used for future training (i.e. poor response techniques from law enforcement or towing agencies, delays/queuing due to poor/no planning for special events, etc).

- Any incident/event that may be of interest to law enforcement/Homeland Security (i.e. vandalism, terrorist behavior, criminal activities, etc)

- Any incident that has been formally requested through open records.

*All video files of the incident/event, from the beginning of the incident’s detection to the end of its cleanup, should be saved.  A Control Room supervisor will delete any unnecessary video files at his/her discretion.

Outside Requests for Archived Video

Note: None of the archived video should be distributed without the STOC DOT Supervisor’s approval.  A full overview of the video release procedure can be found under the Open Record Requests portion, Section 4, of this manual.

When a person/agency requests any of the archived video, the requesting person/agency’s contact information (if available) should be obtained and saved.

- Once a request has been made to receive any archived video, an operator should contact the Archived Video Administrator. They will contact the responding Law Enforcement agency that handled the incident to see if the incident is under an active investigation.  The Law Enforcement agency must be made aware that a request has been made.  The Law Enforcement Agency may not want the video shared.

- Once permission is granted, a CD/DVD needs to be created. The Archived Video Administrator will create and distribute the video. The Archived Video Administrator is the only person who is authorized to create or distribute Archived Video, with the approval of the DOT STOC Supervisor.

Note: A full overview of the Open Records Policy is provided in Section 4 of this manual.

Teleste Video Archiving Procedures

All WisDOT freeway camera images are archived for a period of 72 hours.  The archiving is done on the Teleste NVRs (Network Video Recorders).  Teleste VMX can save video to a permanent location, download it to a workstation, convert it to .mpeg format, or burn it to a disk.

When saving video, control room operators should save to a permanent location (NVR); only under special circumstances should an operator download video to their workstation or burn the video to a to disk.

IMPORTANT: When saving video, send an email to the Archive Video Administrator with all available information (camera, date/time, incident description).  When taking a request for archived video, save the requested video to a permanent location and forward the request to the Archive Video Administrator (Eric).

On the VMX Client interface, a camera that is being archived will contain a NVR control bar at the bottom of the display:

If this control bar is present, video from this camera may be reviewed and/or saved.

To access archived video:

- Either:

  - Click the          button on the NVR control bar.  This will switch the display window from the live camera feed to archived video on the NVR.  The control bar will switch to display NVR tools:

b.   Skip to: To save video to a permanent location:

OR

c.   Expand the desired camera listing in the Source tree view on the right side of the user interface OR select the Sink tab at the bottom of the tree view to display a list of all NVRs.  Right click on the NVR listing and select Search Recordings:

To search recordings:

The top window displays the camera being queried.  To change the camera or query multiple cameras, click the Add… button.

When Recording Search has been selected, the following window will open:

After you click on the Add button the following window will open:

Check any cameras to be included in the query and click OK.

- Select a Start and End time and date for the query.  If Start of query period is left unchecked, the query will include all archived video for the camera(s) selected starting with the oldest archive and ending with the End of query period.  If the End of query period is left unchecked, the query will begin at the Start of the query period selected, and end with the most recent archive.  If both are left unchecked, the query will include all archived video for the camera(s) selected.

- The default query searches for the 72 hour archive video stream and any permanent recordings that have been saved from the selected camera(s).  To search only permanent recordings, check Persistent recordings only.

- Click Query.

To work with queried video:

- When query results are retrieved, the storage browser interface appears:

Timeline

Archived Video

Viewing Windows

Camera name

The timeline reflects the period queried.  The snapshot represents the archived video from the queried time period.  To view/save video from this time period, drag the snapshot to one of the viewing windows on the right (hint: make the viewing windows easier to work with by closing some of them and dragging to expand).

To save video to a permanent location:

- After the archived video is dragged to a viewing window, the window’s control bar switches to display NVR tools:

- The video can be played back, paused, forwarded, or rewound using the appropriate button at the top of the control bar.  A specific time (within one hour) can be quickly located by using the slider (note: the slider reflects the time of day of the archived video being viewed, and it will change as it is moved).  Clicking the        button next to the slider bar jumps the archived video to the previous hour (or the next hour, if possible).

- When the desired starting point of the video is found, click           to open the save menu.  Select Set Mark A.  When the desired end point of the video is found, from the same menu, select Set Mark B.  Then, from the same menu, select Tag Video or Save as Recording.  The following window will open:

- The Beginning and Ending time and date section is filled in automatically.  Describe the video using the naming convention seen above: Descripton_Location_Date (i.e. Spinout_I94@Hwy100_20091208). Check Permanent Recording, and enter 120 as recording expiration time in days. Click OK, and the selected video will be moved to a permanent location on the NVR.

- From the STOC Home Page, click the link, or navigate to http://miltocwp1.dot.state.wi.us/STOCHomePage/VideoHome.aspx. The following page will open:

- Archive opens the form for recording an archived video.

- Request opens the form for requesting an archived video

- Archived Video List opens a gridview list of all archived videos

- Request List opens a gridview list of all  Rests

- Retention Polic y opens a copy of the Open Records (Video) Retention Policy.

NOTE: This system is intended to work by FIRST making a record of a video archive, THEN requesting that archive.

ARCHIVE VIDEO

- Fill in all available information.  Fields with asterisks (*) are required to submit this form.

- Default value for Video Expires In (Days) is 150.  In the Video Retention Policy, the period of time the STOC retains archived video is set at 120 days.  Extending this expiration to 150 days provides a 30 day cushion after the retention period, in case action is required before the video expires.  Video archived in Teleste is automatically deleted when the expiration date passes, while video moved to a permanent location requires manual deletion.  When archiving a video in Teleste, unless instructed otherwise, always set the expiration time for 150 days.

- If an Event Manager incident exists for the archived video, select the Incident # from the dropdown list to associate the video with the incident.

- When all available information is complete, click Submit.  A record will be added to the database, and an email is sent to the Video Archive Administrator.

NOTE: Submitting this form records a log of the archived video; it does NOT archive the video.  Video archiving should be done in Teleste.

NOTE: Editing archive details after this form is submitted must be done by the Archive Video Administrator, so check all fields for accuracy before submitting.

VIDEO REQUEST

- Fill in all available information.  Fields with asterisks (*) are required to submit this form.

- Agencies that request video frequently can be selected from the dropdown list.  If an agency is selected from the dropdown, the Agency Type is selected automatically.  If the desired agency is not found in the dropdown, click the Enter Text button to change the dropdown list to a free text field.  The Agency Type must then be selected manually from the Agency Type dropdown.

- Desired pickup date and time are not mandatory, but if one is filled out, the other must be filled out as well, to submit the form.  If Date is filled out, Time must be filled out, and vice versa.

- If this is a request for an existing archived video, select the video title from the Video ID/Title dropdown list, and click the >> button to add it to the request.  Repeat process if multiple videos are desired.  When a video is selected in this list, the manual video information below is disabled.  To re-enable the manual video information, select the blank record in the Video ID/Title dropdown, and clear all selected videos from the request list by highlighting the video in the list, and clicking the << button.  The preferred request method is to select an existing video archive from the Video ID/Title dropdown.

- If it is not possible to select an existing video archive, select a camera from the Camera dropdown, and fill out the start and end time/date fields below.  Once a camera is selected in the Camera dropdown, the start and end time/date fields below must be filled out to submit the form.  In addition, when a camera is selected, the Video ID/Title dropdown is disabled.

VIDEO LIST

- This is a gridview list of all archived video records.  The list can grow to 20 records, after which page select buttons will appear at the bottom.

- To display video details, click Select at the left of the list.  The detail view will open:

- The video detail view displays all the data for the selected archive video record.  If the Archive Video Administrator moves a video from the Teleste server to permanent storage, he/she will add that information to the record.

- To avoid confusion, editing of archive video records is password-protected, and is intended to be completed by the Archive Video Administrator.  If an edit is desired, click Request Edit.  The following box will open:

- Fill out Name, and enter the desired edit in the Details text box.  Clicking Submit will send an email to the Archive Video Administrator, who will then review and complete the record edit.

REQUEST LIST

- This is a gridview list of all video request records.  The list can grow to 30 records, after which page select buttons will appear at the bottom.  To display request details, click Select at the left of the list.  The detail view will open:

- The request detail view displays all the data for the selected request record.  As events in the request lifecycle occur, such as Release Approval, Disk Made, etc., the Archive Video Administrator will add that information to the record.

- To avoid confusion, editing of request records is password-protected, and is intended to be completed by the Archive Video Administrator.  If an edit is desired, click Request Edit.  The following box will open:

- Fill out Name, and enter the desired edit in the Details text box.  Clicking Submit will send an email to the Archive Video Administrator, who will then review and complete the record edit.

- The Print Details button prints out a paper copy of the request details, with a signature line at the bottom.  This form will be printed for each disk made, and the person picking up the disk will be asked to sign it.  Once the Archive Video Administrator enters the actual pickup information on the website, the printout will be discarded.