Overview

Traffic Incident Alerts (TIAs) are notifications that can be sent out by the STOC or any State Patrol post to notify media, DOT staff, neighboring states, etc of a traffic related incident or issue. TIA alerts entered are automatically emailed to defined distribution lists based on location and type of incident and are sent in plain text which makes them easily viewable on mobile devices, email clients, etc.

A TIA should be sent from the STOC when one of the following conditions is met:

- When the STOC receives a request from Wisconsin State Patrol to send a TIA

- When 50% or more lanes are blocked on a mainline roadway (including a 3, 4 or 5 incident) or there is a full closure of a system ramp.

- When excessive queuing surpasses defined thresholds (Currently 3 miles)

- STOC receives notification of an OSOW restriction

Instructions

Open a web browser and navigate to http://transportal.cee.wisc.edu/applications/TIA/ . In addition a link to the TIA site can be found on the STOC homepage.

Note: To access the TIA system, a WisTransPortal login will be needed. This will be provided by your supervisor or trainer during your first week.

Based on the action desired, select one of the following:

- Incident Alerts – sent by DSP, or the STOC if incident meets criteria outlined above.

- Media Releases – Control Room will send media releases upon request, however we do not write them.

- EOC Activation – typically sent by STOC when notified of EOC activation or deactivation.

- WI E-Sponder – link button to WI E-Sponder web page.

- TIA Admin – used to modify email lists and user accounts, and to send out generic email messages.

Incident Alerts

When Incident Alerts is selected, the following form opens:

Active Incidents

*Notice the Active Incidents list, from which an incident may be Updated, Cleared, or Viewed.  In the Incident ID field, the number in parentheses indicates the number of actions performed on this incident email.  For example, if an incident has had an Initial email and one update, the number in parentheses will be (2).

Creating a new incident

- Click Create New Incident.  The Incident Alert-Initial Notification form will open:

- Enter in all available incident information. A few sections to note are:

Nearest City:

This field is filtered by County, but checking Show All Cities will remove the filter and display cities statewide.

Nearest Landmark

This is intersection information based on County and Highway selected. To restrict the list to mileposts only, check Just Mile Posts.  If the location does not appear in the Nearest Landmark list, or if the nearest landmark/intersection is confusing for the intended recipients, a description must be entered in the Location Description text box.

Handling Agency

A handling agency must be entered before a TIA can be cleared.

Time Occurred

This field must contain the actual time that the incident occurred, not the time that the STOC was notified. This information can be obtained from the handling agency or video if available.

Alternate Route

If an alternate route is included, operator must verify on a map that alternate route is feasible, and Alternate Route Verified must be checked before email can be sent.

Note: The distribution lists on the right are automatically selected based on the county chosen, but can be modified by the operator.  To send a TIA to an individual address, check Individual Email, click Add Email Address, and type the address in the space provided.

- Check Continue to Confirmation Page.  The Confirmation form will open:

- Verify that the incident information and distribution lists are correct and there are no spelling or grammar errors and click Send Initial Notification.

Searching for a cleared incident

- Click Search Cleared Incidents.  The following form will open:

- Enter any information available, and click Retrieve Cleared Incidents.  The incidents retrieved are listed in the white section at the bottom of the form.  Cleared incidents may be viewed, but not modified.

Updating an active incident

- On the list of active incidents, click Update.  The Update Notification form will open:

- Enter any information you wish to update into the form

- If you need to add any additional distribution lists (besides those originally selected), check the appropriate boxes. Note: The update will go to all lists that the initial email was sent to and any additional lists added during an update

- Click Continue To Confirmation Page. Verify all information is accurate and free from any spelling or grammar error

- Click Send

Clearing an active incident

- On the list of active incidents, click Clear.  The Clear Notification page will open:

- The Cleared Date and Time is auto filled. If changes need to be made based on incident clearance time, these values should be changed

  - Cleared email is sent to all groups that have received Initial Notification and/or Updates.

  - The Handling Agency must be entered before a TIA can be cleared.

- Click Send CLEAR Notification.  When a Cleared notification is sent, the incident is removed from the Active Incidents List, and may only be viewed by searching cleared incidents.

Viewing an active incident:

- From the Active Incidents list, click View.  The View TIA form will open:

- A list of all TIA emails sent pertaining to the selected incident appears in the white section at the bottom of the form.  Each email can be viewed in detail from this list by clicking View.

- An operator can also click to Update or Clear the incident from this screen.

Manually Sending TIAs When the TIA site is Unavailable

There is a folder on the “N:\” drive of the DOT computer that contains a document with TIA templates and distribution lists with email contacts that can be used in the event that the TIA site goes down. To manually send a TIA:

- Navigate to this list as follows:

N:\ControlRoom\TIABackupDistro Lists\TIADistro

- Operators should open this document, and select the appropriate template (Initial Notification, Update, Clear, etc) and copy and paste into the body of an Outlook email message.

- All information (County, City, Handling Agency, etc) will need to be manually entered.

- Next, select the appropriate email distribution lists and copy and paste them into the Bcc field of your email. If Bcc is not available, click Options on the top menu bar of your message and “Show Bcc.”

- Enter the From address as DOT STATE TRAFFIC OPS CONTROL Center. If the From field is not available, click Options on the top menu bar of your message and select “Show From.”

- When finished, review for accuracy and any spelling / grammar errors and click send.

Any additional updates will need to be sent this way. For instructions on reporting outages of the Transportal system, please refer to the Control Room Troubleshooting and Maintenance Guide.