Overview

To assist in notifying the public of traffic incidents or events, there are 17 permanently mounted signs throughout the SE region that can be actived when a message is placed on the Highway Advisory Radio (HAR). Each of these signs can be actived independantly and when activated with flash with a ‘wig-wag’ pattern advising travelers to tune to AM 1610.

HAR Flasher Locations

The seventeen HAR flasher locations are as follows:

- I-94 NB at County ML

- I-94 SB at County KR

- I-94 NB at Puetz Rd

- I-94 SB at KK River

- I-43 NB at Keefe Ave

- I-43 NB at County U

- I-43 SB at WIS 60

- I-94 EB at WIS 83

- I-94 EB at Elm Grove Rd

- I-94 WB at Mitchell Blvd

- US 45 NB at Swan Blvd

- US 45 SB at Swan Blvd

- US 45 NB at Main St

- US 45 SB at Lannon Rd

- I-43 SB at North Ave

- I-94 EB at WIS 67

- I-94 WB at WIS 83

Activating the HAR Flashers

The HAR flashers are managed through a TAPCO web interface. To activate the flashers, follow the steps below:

- Navigate to http://www.tapcologin.com/ (this can also be found on the STOC homepage by clicking Highway Advisory Radio and HAR Flasher Web Control

- Enter your username and password. Each user will be set up with a login during their first week.

- Click  at the top of the page.

- Place a checkbox next to the flasher location(s) you wish to activate.

- If you want to manually control when the flashers go on and off, click the “Flash On” button. If you would like them to automatically come down at a certain time, set a number of hours and/or minutes you would like the flashers on followed by the “Auto Flash” button.

- After clicking “Flash On” or “Auto Flash” the red dot next to the flasher location should turn to green. This will let you know the flasher is turned on.

Turning HAR Flashers Off

To turn HAR flashers off, follow the steps below:

- Follow Steps 1-4 in the previous section

- After selecting the flashers you would like turned off, click the “Flash Off” button.

Additional Devices Controlled via the HAR Flasher Site

In addition to the Highway Advisory Radio flashers located in SE Wisconsin, the TAPCO web interface also controls two other devices. A brief description follows:

- Marquette Interchange W-N Curve Flashers: These are a series of flashing chevron signs to warn drivers to slow down through the curve. These can be set to stay lit. The procedures to activate (keep lit) are the same as activating HAR flashers

- Merrimac Ferry Flashers: Five flashers are available in the TAPCO web interface to advise of Merrimac ferry closures. These are also activated the same as the other HAR flashers.