# Overview:

# The Highway Advisory Radio system or HAR is a traveler information system that consists of 14 low power AM transmitter sites in and around southeast Wisconsin. The HAR broadcasts in SE Wisconsin on AM 1610 and statewide (via portable HAR trailers) on AM 1620.

# Travel times are the default message on the HAR unless a user specified message is requested.

# HAR units are treated like a sign in the TransCore system and appear in the TIS application just like dynamic signs.

# HAR Locations:

# Permanent HAR transmitters are at the following locations:

| # I-94 at WIS 158 | # US 45 at Hampton Ave |
| --- | --- |
| # I-94 at Hawley Rd | # I-94 at Layton Ave. |
| # I-43/94 at National Ave | # I-43 at Beloit Rd. |
| # I-43 at Daphne Rd. | # I-43 at WIS 167 |
| # I-43 at Falls Rd. | # US 45 at County Line Rd. |
| # I-94 at County G | # I-43 at Moorland Rd (1/2 mi west) |
| # I-94 at WIS 67 |  |

Adding a Custom Message to HAR Travel Times:

To add a custom HAR message to the default travel time plan (i.e. information about a current incident), follow these steps:

- Select the HAR in TIS (Device ID 1001)

- Click the button to bring up a list of messages and scroll to message 25090

- Click the wrench icon to edit the message

- Click Add Text and type the message you want the HAR to play.

- When finished, click the play  button to listen to your message.

- If you are satisfied with the message, click OK and save by clicking the red disk icon in the message window.

Creating a Standalone HAR Message

Occasionally, you may need to create a standalone HAR message (without travel times) for scheduled construction, planned or unplanned significant events or to add to a TIS plan or schedule. To create a standalone HAR message, these steps should be followed.

- Open TIS and select

- Scroll to HAR Message ID 25932

- Click on the Message number followed by the edit wrench. The following screen will pop up:

- Highlight the current message listed as TIS Message (in red above). Once the TIS message is highlighted, the Edit Text box will become available. Click Edit Text.

- The following window will open. From here, update the message as needed and click the play button to make sure text is being read correctly. You may have to make some changes to the message for Text-To-Speech to read it correctly.

- When finished updating your message and reviewing for accuracy, click the OK button.

- The HAR message has now been updated and can be added to a plan or schedule.