Overview

Ramp meters are traffic signals placed at freeway on-ramps that control the frequency of vehicles entering the flow of traffic on the freeway. This is accomplished by creating a delay between cars entering the freeway which can help reduce accidents and travel times for commuters.

Ramp meters are scheduled to activate at certain times of the day, usually during peak travel times. Ramp meters require minimal operator intervention however they are an important tool in overall traffic incident management.

Purpose

This module outlines how to carry out the most common and basic ramp meter functions required by the STOC operators.

There are two types of ramp meters currently in operation in the state:

- Older Model:	170

- Newer Model: 2070

Functions:

- Upload/Compare

- Download

- Check station time

- Set plan/Mode

- Online/Offline

- Holidays

- Look up schedule

Upload/Compare

This function sends information from the ramp meter “in the field” to our computers in the control room. To perform this task:

- Double click the ramp meter on the ATMS map

- Select the “System” drop down menu

- Select “Upload and Compare”

- Click “Compare” or ‘Save” based on what IT or DOT requests

Download

This is something the operator normally wouldn’t do unless it was requested.  This function sends information from our system out to the ramp meter in the field. To do this:

- Double click the ramp meter on the ATMS map to open the details window

- Click “System” drop down menu

- Select “Download” (applies to both 170 and 2070 ramp meters)

Note: If a ramp meter appears purple and magenta (no green) on the map, the ramp meter needs a download.  You can perform a download and that should fix the problem.

Check Station Time

This function is performed to make sure the ramp meter is set to the correct time of day. To check station time:

- Double click the ramp meter on the ATMS map

- In the details window, select the “System” drop down menu and select “check Station Time”

- A window will open showing you the controller time and the system time. They should both be the same. If they are not the same, you will need to download station time to the ramp meter (steps below)

Note: the server automatically downloads station time at the top of every hour) Same steps apply for both types of meters.

Download Station Time

To download station time to a ramp meter:

- Double click the RM on the ATMS map

- Click the “System” drop down menu and select “Download Station Time”

Note: Nothing will happen after selecting “Download Station Time.” To verify the time was updated follow steps to check station time above.

Set Plan/Mode

This will change the rate of metering for the ramp meter or turn on/off a ramp meter that is currently not metering. To start these processes double click the meter on the map and follow the process below for the type of meter.

Activating a 170:

- From the “System” menu select “Set Plan / Mode”.

- A new window will open. In the drop down menu for “Plan” chose a rate from 1 to 6.

  - Six is the slowest and will hold cars at a red light on the ramp for the longest period of time.

- Set a terminate time, and click “OK”

Releasing a 170:

- Double click the ramp meter on the ATMS map

- From the System menu select  Set Plan/Mode

- Select “Command Ramp to Return to TOD Operations”

- Click OK

Activating a 2070:

- Double click the ramp meter on the ATMS map

- From the “System “menu, select “Set Action/Rate”.

- Change the drop down menu from “Fixed Rate, VPH per Ramp” to “Fixed Rate Plan”

- Change the rate from 1 to 6 with 6 being the slowest release and 1 being the fastest

- Set a termination time and click “Okay”.

Releasing a 2070:

- Follow steps 1 and two above

- Check “Release Ramp”

- Click OK

Online/Offline

This function deals with COMMUNICATION between our computers and the ramp meters.  Taking a ramp meter offline only means that we’re not communicating with it anymore.  IT DOES NOT MAKE IT STOP METERING. Control room operators would not normally turn a meter offline unless they were instructed to do so.

Viewing Ramp Meter Schedule

To View Weekly Schedule:

Double click the ramp meter on the map, click “Tools” drop down menu and select “Device Configuration”, then click schedules tab.

To View Daily Schedule:

Double click the ramp meter on the map and click the “General” tab.

Holidays

To check the holiday schedule for a ramp meter or to change the holiday schedule, right click on the meter on the map, click “Monitor” and click “General”.  The ramp meter’s schedule will open.  Then click “Tools”, click “Device Configuration” and when the next window opens click “Schedules”.  You can now view the holiday schedule, which shows the month and day that the meter does not meter.  The holiday schedule overrides the TOD schedule and has to be updated once every year.  The procedure is the same for both types of ramp meters.