Guidelines for Creating Messages

The signs are generally capable of sequencing up to six frames. However, for driver comprehension, messages shall be limited to one or two frames (see MUTCD 2003 Edition Section 6F.55). Blank or other filler frames between the two frames of text shall not be used. It is desirable for the driver to be able to read the entire message sequence twice as they pass by the sign. For an Interstate highway application, the total viewing time is about seven seconds. Each frame is usually displayed for 2.0 seconds or less. Using more than two frames makes it difficult for drivers to read the entire message sequence twice. Do not flash any part of a message.

As a general rule, the following format should be used when creating messages for incidents, roadwork, congestion or emergency situations:

- Problem: This refers to the reason for posting a message and provides information about a situation the driver will encounter.

  - Examples include: DELAYS AHEAD, LANE CLOSED or RAMP CLOSED

- Location: Describes the location or distance to the event. The location should be specified using cross streets or distances downstream of the DMS.

  - Examples include: LOCUST ST, HWY 83 or # MILES AHEAD

- Action: This is the action that motorists should take in response to the problem and location information.

  - Examples include: USE ALTERNATE ROUTE, EXIT NOW or MERGE RIGHT

Messages should be kept to one phase if possible and be presented in the order shown below:

[ Problem ]

[Location]

[Action]

Travel Time Messages

Travel times should be displayed as the default message on DMS boards (if they are available) and only when they are accurately displaying travel times.

If it is determined a travel time is not accurate, the travel time message should be removed from the DMS and IT notified.

In the event that a location displayed on a travel time is unreachable (full closure due to incident or construction) the travel time message should be taken down.

Signing for Future Roadwork

DMS signing should be used only for roadwork involving lane or ramp closures that will start in 10 days or less.

If the construction is scheduled to occur within 7 days, messages should contain day of the week instead of calendar dates. An example is included below:

RAMP CLOSED

THUR

9AM – 3 PM

If the construction is scheduled more than 7 days out but less than 10, calendar dates should be used. An example is included below:

RAMP TO CLOSE

FEB 22 - 28

10PM – 5AM

Exceptions to these guidelines should be approved by a DOT supervisor.

Adverse Weather Messages

Control room staff is encouraged to take a proactive approach in activating adverse weather conditions messages based on the following criteria:

- Any requests for signing received from law enforcement or highway departments

- Any adverse weather warnings in the county or area

- Any increase in incident or accidents due to weather

Adverse weather messages should also follow the PROBLEM, LOCATION, ACTION guidelines if possible. A full overview of adverse weather procedures can be found in Chapter 10

Additional Information

The information presented here are just excerpts from the Wisconsin Department of Transportation’s Traffic Guidelines Manual (TGM) Chapter 17. For more information about DMS message guidelines, please see TGM Chapter 17 which is included in the Appendix section of this manual.