Overview

Similar to DMS message boards, Portable Changeable Message Signs (PCMS) are capable of displaying multiple phases however, only one or two phases shall be used. Using more than two frames makes it difficult for drivers to read the entire message sequence twice.

In additions, PCMS signs are only capable of displaying 8 characters per line which can cause some additional challenges when creating messages.

Acceptable Use of PCMS Signs

Since PCMS signs are dynamic, the should only be used to display ‘real time’ or changing traffic conditions or traffic control information. They are used for work zone temporary traffic control, incident management, special events and unusual/hazardous road conditions due to weather. This could include expected delay times, warming of stopped traffic, ramp or lane closures, advisory speeds and alternate route advisories. They may also be used to provide advanced notice (up to 10 days) prior to projects or events expected to cause congestion or that will require drivers to use alternate routes.

Message Guidelines

As a general rule, the following format should be used when creating messages for incidents, roadwork, congestion or emergency situations:

- Problem: This refers to the reason for posting a message and provides information about a situation the driver will encounter.

- Action: This is the action that motorists should take in response to the problem and location information.

The first phase should describe the problem and the section phase should advise of appropriate action. An example can be found below:

CRASH

AHEAD

1 MILE

LEFT

LANE

CLOSED

The words DANGER, HAZARDOUS or CAUTION shall never be used on PCMS

Using Abbreviations on PCMS

Because PCMS signs are only capable of displaying 8 characters per line, oftentimes abbreviations will be used to communicate intended meaning. A list of acceptable abbreviations and abbreviations that shall not be used on PCMS signs can be found at the end of this section. These guidelines should be referred to any time abbreviations need to be used on a sign.

Additional Information

The information presented here are just excerpts from the Wisconsin Department of Transportation’s Traffic Guidelines Manual (TGM) Chapter 17. For more information about DMS message guidelines, please see TGM Chapter 17 which is included in the Appendix section of this manual.