Overview

Drivers entering the freeway system and traveling the wrong direction is and remains a challenging issue for the Department of Transportation, law enforcement agencies and the motoring public. A seemingly higher number of wrong-way driver occurrences have gained recognition creating a heightened demand for resolution.

The Wisconsin Department of Transportation (WisDOT), Division of Transportation Systems Development (DTSD) in coordination with Traffic and Parking Control Co., Inc. (TAPCO), the Statewide Traffic Operation Center (STOC) and Milwaukee County Sheriff’s Office (MCSO) have placed high visibility real-time warning detectors at selected off-ramps in Milwaukee County.  These devices are being placed as a pilot effort to deter drivers from making wrong-way movements onto freeways. Once a vehicle is detected traveling in the wrong direction an audible signal will be sent to the centers to alert MCSO dispatchers and STOC operators.

Detector Locations:

- I-43 NB @ Kilbourn

- US-41 NB Off @ W Martin Dr (State St)

- US-41 SB Off @ Alois St (State St)

- I-94 EB @ Miller Park (Mitchell Blvd)

- Miller Parkway NB @ Canal St

- Miller Pkwy SB @ Canal St

- I-794 EB Off @ N Lincoln Memorial Dr

- I-43 SB Off @ Green Bay Rd & Capitol

- US-45 NB @ Wisconsin Ave

- I-94 WB Off @ Hawley Rd

- Hoan Bridge

Control Room Operator Procedures

- Alert detected by STOC Operator

- WWD located on camera:

  - Stream video to MCSO dispatch center monitors immediately

  - Operator will follow up with MCSO dispatch center in one of two ways to confirm location:

    - Portable radio via MSCO F2 frequency

    - Telephone

- WWD is NOT located on camera:

  - Video will be reviewed to locate WWD and to determine if and where  the WWD exited the freeway

  - Operator will communicate findings to MCSO dispatch

- Operator will continue to monitor MCSO portable radio for further direction and coordination with MCSO dispatchers

- Create a SINS entry for documentation of the incident.  Refer to the SINS Chapter of this Manual for associated WWD Worksheets.

  - If the incident results in a crash associate Event Manager in SINS

  - Associate any video that may be related to the WWD

- Operator will send an email to the Wrong Way Driver email distribution list

- Operator will review video to try to obtain the location where the WWD entered the system

- Operator will archive all video located of the WWD and submit an open record request (if applicable)

- Control room will dismiss alert when predetermined criteria are met

Detector Alarm Reset

- Access the TAPCO Website:

  - Access the Website through the STOC Home page at WWD Web Access or access link below

  - http://www.tapcologin.com/Login.com/

- Log in to the Website

  - Username: stoc1

  - Password: tapco1

  - Click on the “Alert” tab at the top

  - Click on the location of the detector and clear or just click on clear all

In the case of an operator locating a wrong way driver in an area where there is no detection devices or warning signs, the operator will follow the above procedures for immediate notification to law enforcement.  In other locations of the state the operator will contact that county’s sheriff’s department, State Patrol and create a WWD SINS report.

Protocol for Using MCSO Radio

- Operators will switch the MCSO portable to the MCSO F2 channel

  - To switch the MCSO portable radio to F2, turn the middle knob to the dash between 1 and 3

  - The display will show MCSO F2

  - Listen to make sure you are not talking over anyone

  - Once the air is clear, depress and hold the button on the side of the portable and listen for a beep

  - With the button depressed broadcast, “STOC to Milwaukee County on F2, Wrong Way Driver” and release the button

  - Wait for MCSO dispatch to acknowledge

- Once MCSO dispatch acknowledges, operator will advise location of the wrong way driver

- Operators will use plain language when broadcasting over the radio

- Operators should monitor the portable for further radio traffic

Wrong Way Driver Worksheet

The worksheet for recording Wrong Way Driver (WWD) data can now be found in the SINS application.  On the switchboard form, there is a button titled Enter WWD:

This button opens the WWD form:

Notice the record navigation buttons at the bottom of the form:

Use the arrows to navigate between existing records, and use          to add a new record.  To save a record, use the arrows to navigate to a different record.  Navigating away from the current record saves it (as long as there has been data entered).

The main portion of the WWD Incident Form is broken up into five sections:

      - Date, Time, Location, and Direction: Fill in or select all the information that is available in either the Text Boxes, or Drop down boxes. If the WWD was reported but never confirmed a report still needs to be completed, put in the reported information.

      - How did the STOC learn of the WWD?  If Other is chosen, provide details in the Notes field.  If WWD System Alert is chosen, fill out the fields that correspond with a WWD Alert.

      - What was the result of the WWD? Was contact made? Was it stopped or was there a crash? Again fill in or select all the information that is available in either the Text Boxes, or Drop down boxes

      - Notes: Provide any other information that is available in the Free Text Notes section located on the second tab of the form.

      - Page 2 of the form also has the Video associating area; you must save work that has been done on the form and archive the video before completing this section. Find the archived video in the drop down menu and click Add to list to populate the Associated Videos box.

Video Tab Controls

- The current record must be saved before associating videos. Remember, to save the current record, as long as at least one of the form fields has been updated, use the arrows at the bottom of the form to navigate to a different record. After the initial save, videos can be associated.

- Choose the desired video from the Video List, and click the Down Arrow to add it to the Associated Videos list. To remove a video from the Associated Videos list, select the desired video and click the Up Arrow.

WWD Summary Report

- From SINS main switchboard, select Reports.  The reports form opens:

- Using this form, SINS can display and/or print any report listed, as well as email reports.  The last report listed is the WWD Summary.  When it’s selected, and Preview is clicked, the report opens:

- The WWD Summary Report is grouped by region and county, and organized by date, most recent to oldest.