Overview

There are 3 tunnel cameras at the US 53/WIS 93 interchange and 1 “tunnel closed” sign in Eau Claire.  In the event of camera or sign problems the following procedures will be followed.

- The Eau Claire State Patrol (DSP) Post restarts the cameras by restarting software or rebooting computer. STOC IT staff should also be contacted and can attempt to resolve the issue.

- If there are unresolved problems with the tunnel cameras or signs, they will be reported to the STOC by Eau Claire DSP.  When describing the problem, DSP staff will indicate whether cameras are black or whether they can be re-started but do not remain on-line. They may also report that the signs are not turning off or on as needed.

- The cameras or the multiplexer equipment in the hut may need to be physically reset if the problem is not yet resolved:

  - During normal business hours, create a SINS and contact the Eau Claire on-call electrician (715-855-7673).  Eau Claire electrician will reset the equipment by the end of the business day.

  - After 4:30 p.m. and on weekends, a sergeant at Eau Claire (DSP) may be able to re-start equipment. If not, contact the on-call electrician.

- If the problem is the cameras rather than the network, STOC will create a Work Order in Cartegraph for CCTV 18-0001, 0002, or 0003 and dispatch TAPCO to try to solve the problem.  STOC will email “DOT State Patrol Eau Claire Post” (dotstatepatroleauclairepost@dot.state.wi.us) to report status.

- STOC and the DSP Eau Claire Post will be notified when the cameras are operational again.

There have been times when a camera has spontaneously recovered.  Eau Clair DSP will contact the STOC to report this.