Amber Alert is a statewide child abduction alert system that uses immediate broadcasts through radio and television stations to assist law enforcement agencies in their quest to find a child.

Amber Alert Website

http://www.amberalertwisconsin.org/

WisDOT’s Role in Amber Alert

WisDOT will broadcast Amber Alert information on the 511 system, Highway Advisory Radio in SE Wisconsin, and will utilize the electronic permanently installed, overhead highway Dynamic Message Signs (DMS) in SE and SW Wisconsin, as well as permanently placed PCMSs to notify the traveling public of the Amber Alert.

Amber Alert Notification

The Dane County Public Safety (911) Communications Center will notify the control room in the event of an Amber Alert.  The control room operator will only respond to requests from the Dane County Public Safety Communications Center.

ALL requests for an Amber Alert will be confirmed by telephone with the Dane County Public Safety Communication Center.

The control room operator is responsible for documenting all Amber Alert activities on the log sheet included in Appendix C.

In the event of an Amber Alert the STOC will follow the procedures listed below.

- The STOC will confirm the Amber Alert with the Dane County Public Safety Communications Center – (608) 267-1507

**The STOC will activate the Amber Alert DMS, PCMS, HAR, and 511 messages only when contacted, via fax, phone call, or email by the Dane County Public Safety (911) Communications Center. If the STOC is notified via fax or email, the Amber Alert must be confirmed via telephone.

Duration of Alert Messages:

The Amber Alert DMS messages will be displayed, 511 floodgate will be activated, and the HAR message will be broadcasted for 5 hours from the time the Amber Alert is activated.   (NOT 5 hours from the time that WisDOT is notified.)  Messages will be removed from the system at the direction of the Dane County Public Safety Communications Center if the alert is cancelled sooner.

- Upload a 511 Statewide Floodgate, Audio and Text Message using the following pre-typed template located on every desktop. Simply insert information in the areas that are bold and italicized.  Once you have entered the required information, copy and paste the message to the 511 web page.  Be sure to verify the message via phone and web.

- The STOC will broadcast the same Amber Alert message on the HAR and activate the HAR Flashers, making sure to use the pre-typed template.  The HAR operating procedures and blank Amber Alert Broadcast Script are included as Appendix A and B respectively.  All Advance Warning Flashers are to be turned “on” after the HAR message is entered.

This is a Child Abduction Alert regarding the following child. CHILD’S NAME AND DESCRIPTION.  Child was last seen at LAST KNOWN LOCATION. Child was last seen wearing DESCRIPTION OF CHILD’S CLOTHING.  Law enforcement authorities believe the child maybe in the company of NAME AND DESCRIPTION.  They may be traveling in a VEHICLE DESCRIPTION AND LICESNE PLATE. Last seen heading LOCATION AND DIRECTION.  If you have any information on the whereabouts of CHILD’S NAME, please contact LAW ENFORCEMENT AGENCY at PHONE NUMBER.  Persons who may locate the abductor should take no action other than to immediately call law enforcement authorities and provide them with the location.

- The STOC will activate the applicable Amber Alert DMS messages via:

- Milwaukee:	TIS plan 1

- Madison:	TIS plan 2

- NE Region:	TIS plan 1513

- NC Region:	TIS plan 1514

- NW Region	TIS plan 1515

DMS/PCMS Amber Alert Messages:

(Milwaukee DMS Message)

| CHILD ABDUCTION ALERT | TUNE TO 1610 AM OR CALL 511 |
| --- | --- |

(Madison DMS Message)

| CHILD ABDUCTION ALERT | TUNE TO LOCAL RADIO OR CALL 511 |
| --- | --- |

**if the HAR is not operating in Milwaukee, messages on the Milwaukee DMS should be    changed to:

| CHILD ABDUCTION ALERT | TUNE TO LOCAL RADIO OR CALL 511 |
| --- | --- |

**If 511 is not working properly, the message on any DMS should read:

| CHILD ABDUCTION ALERT | TUNE TO LOCAL RADIO |
| --- | --- |

Starting the appropriate TIS Plan will activate the messages displayed above.

- The STOC will activate all permanently placed PCMS throughout the state.

Messages to be placed on all permanently located PCMS signs (listed below):

| CHILD ABDUCT ALERT | CALL 511 |
| --- | --- |

| PCMS Semi-Permanent Locations to be used during Amber Alert |  |  |
| --- | --- | --- |
| PCMS ID | Region | Location |
| 01-1013 | SW | *I-90 NB mp 143.8 |
| 01-1017 | SW | *I-39 NB mp 149.8 |
| 04-4001 | NC | *Marathon County I-39/US 51 NB mp 178.0 South of exit 179 in Mosinee |
| 04-4002 | NC | *Marathon County Hwy 29 EB mp 156.0 |
| 04-4003 | NC | *Marathon County Hwy 29 WB mp 177.0 East of CTH J |
| 04-4004 | NC | *Marathon County Hwy 51 SB mp 199.0 |
| 06-6006 | NW | *I-94 WB mp 48.0 |
| 06-6013 | NW | *I-94 WB mp 12.0 |
| 06-6014 | NW | *I-94 WB mp 74.0 (median) |
| 06-6015 | NW | *I-94 EB mp 24.0 (median) |
| 08-8005 | NW | *St. Croix County, I-94 EB mp 1.0 |

6.   Internal Amber Alert Notifications

In the event that WisDOT resources have been deployed for an Amber Alert, the control room operator must notify the on-call STOC Supervisor.

7.    When an Amber Alert is canceled the STOC will follow these procedures.

- The STOC will cancel the Amber Alert DMS, PCMS, HAR, and 511 messages only when contacted, via fax, phone call, or email by the Dane County Public Safety (911) Communications Center.

- The STOC will remove the DMS and PCMS Message and all related 511 messages.

- The STOC will remove the Amber Alert Message from the HAR and deactivate the HAR Flashers according to the “Removing a Message from the HAR” instructions in  Appendix A.

- The STOC will notify the on-call STOC Supervisor.

- The STOC will email the DOJ according to the “Example Email to DOJ” in Appendix D.

Contact Information:

- WisDOT can contact the Dane County Public Safety Communications Center with Amber Alert notification questions.

Dane County Public Safety Communications Center: (608) 267-1507

- WisDOT will contact the Department of Justice via email after the conclusion of an Amber Alert with a summary report.  The emails listed below are to be used as contacts; Appendix C should be used as a guide for the report.  An example email is included in Appendix D located at the end of this document.

Department of Justice Call Center: 866-652-6237

David S. Matthews, Division of Criminal Investigation

E-mail:  Matthewsds@doj.state.wi.us

Tel:  	(608) 266-1671

Susan Whitehorse, Manager Wisconsin Clearinghouse Missing and Exploited Children

E-Mail:  whitehorsesa@doj.state.wi.us

Tel: 	(608) 266-1671

Cell:  	(608) 225-1926

Amber Alert Questions (from outside agencies)

Questions regarding WisDOT’s Amber Alert Policy should be directed to:

Anne L. Reshadi, P.E.

System Operations & Electrical Engineering Section Chief

Wisconsin Department of Transportation

Bureau of Traffic Operations

433 W. St. Paul Avenue, Suite 300

Milwaukee, WI 53203

(414) 227-2149

anne.reshadi@dot.wi.gov

Appendix A

Updating Amber Alert Message on the synced HAR

To update the message in TIS on the synced HAR:

- From the main TIS screen, select the blue M        at the top of the page.

- Select message ID 26276 and select the wrench        icon at the top of the window.

- Click on the TIS message in the Audio field and select Edit Text

- Copy and paste the updated Amber Alert script in the text field and click OK.

- Select OK and save your changes by selecting the small disk   on the upper left.

Appendix B

AMBER ALERT BROADCAST SCRIPT FORM

________________________________________________________________________

The operator must complete this form based on the information received from Dane County Public Safety Communications Center.  The script must then be read word-for-word on the 511 floodgate and cut and pasted in the TIS HAR message.

_________________________________________________________________________________

This is a Child Abduction Alert regarding the following child: ___________________________

(Child’s name)

_________________________________________________________________________________

(Description – age/weight/height/scars)

Child was last seen at:_____________________________________________________________

Child was last wearing:_____________________________________________________________

Law enforcement authorities believe the child maybe in the company of:

_________________________________________________________________________________

(Suspect/description)

They may be traveling in a:__________________________________________________________

(Vehicle description, year, color, make, model, license plate)

Last seen heading:_________________________________________________________________

(Direction of travel)

If you have any information on the whereabouts of:_____________________________________

(child’s name)

Please contact:________________________________	At:__________________________________

(Law enforcement agency)		(telephone number)

Person(s) who may locate the abductor should take NO action other than to immediately call law enforcement authorities and provide them with the location.

Appendix C

AMBER ALERT OPERATOR LOG SHEET

|  | DATE/TIME |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Amber Alert Activated |  |  |  |  |  |  |  |  |
|  |  |  | Notified by Dane County Public Safety Communications |  |  |  |  |  |  |  |  |
|  |  |  | Confirmed Amber Alert with Dane County |  |  |  |  |  |  |  |  |
|  |  |  | Messages Displayed on 511 Audio and Text Message |  |  |  |  |  |  |  |  |
|  |  |  | Message Broadcasted on Synchronized HAR (TIS) |  |  |  |  |  |  |  |  |
|  |  |  | DMS/PCMS Plans and HAR Advance Warning Flashers Activated |  |  |  |  |  |  |  |  |
|  |  |  | Notified On-Call STOC Supervisor |  |  |  |  |  |  |  |  |
|  |  |  | AMBER ALERT CANCELLED / EXPIRED |  |  |  |  |  |  |  |  |
|  |  |  | Contacted by Dane County Public Safety Communications |  |  |  |  |  |  |  |  |
|  |  |  | Removed Messages from DMS/PCMS/511 Audio and Text Message |  |  |  |  |  |  |  |  |
|  |  |  | Removed Messages from Synchronized HAR |  |  |  |  |  |  |  |  |
|  |  |  | HAR Advance Warning Flashers Deactivated |  |  |  |  |  |  |  |  |
|  |  |  | Notified On-Call STOC Supervisor |  |  |  |  |  |  |  |  |
|  |  |  | Emailed “Summary of Actions” to Department of Justice |  |  |  |  |  |  |  |  |

# Appendix D

Example Email to DOJ

From: stoc@dot.wi.gov

To: Whitehorse, Susan; Matthews, David

CC: DOT DL STOC Internal Notification

Subject: TOC Actions – Amber Alert, June 23, 2005

Below is a list of actions for June 23rd's Amber Alert.

5:00 AM - 	Heard about Amber Alert on news

5:15 AM - 	Contacted Dane County and confirmed Amber Alert

5:30 AM - 	Recorded Amber Alert floodgate on 511

Turned on Amber Alert messages on Synced HAR

Turned on Amber Alert messages on DMS

Activated Advance Warning Flashers

6:00 AM - 	Left message w/ STOC Supervisor

7:30 AM - 	Contacted Dane County and confirmed cancellation of Amber Alert

7:30 AM - 	Removed messages from DMS/PCMS

Removed messages from Synced HAR

Removed 511 floodgate message

Turned off Advance Warning Flashers

Problems Encountered:

-Dane County's fax had to be resent because operator didn't receive initial fax.

-Sign 22 (I-894 EB @ 72nd St) could not be displayed due to sign malfunction.

-Advance Warning Flasher I-94 WB @ Mitchell Blvd did not turn on

If you have any questions, please contact the control room at 414-227-2142.

Operator’s name