Overview

High water alarms sense a high water condition and transmit voice phone calls and e-mail notifications to the following agencies:

| Agency/Organization | Phone # | E-mail Address |
| --- | --- | --- |
| Milwaukee Co Sheriff | 414-278-5065 414-278-5067 | shfdispatch@milwcnty.com |
| WisDOT STOC | 414-227-2142 | stoc@dot.wi.gov |
| Miller Park | 414-902-4457 | stadiumcontrol@brewers.com |
| West Milwaukee PD | 414-645-2151 | dennisnasci@westmilwaukee.org |

The alert messages you will hear are as follows:

| Alarm Location | Notification Level 1 | Notification Level 2 |
| --- | --- | --- |
| No.1-Woods Creek | “High water on Woods Creek at Miller Parkway” | “Potential flooding of Miller Parkway from Woods Creek” |
| No. 2-Detention Pond | “Potential flooding of shoulder on NB Miller Parkway” | “Potential flooding of driving lanes of NB Miller Parkway” |
| No. 3-Miller Parkway NB Lanes | “Water on shoulder of NB Miller Parkway” | “Water on driving lanes of NB Miller Parkway” |
| No. 4-Miller Parkway SB Lanes | “Water on shoulder of SB Miller Parkway” | “Water on driving lanes of SB Miller Parkway” |

When notified of the alarm:

- Immediately contact the Southeast Region RIMC

- Follow-up with a call to the Milwaukee County Sheriff’s Department (they should also have been notified with an automated alert from the flood alarm system)

- Monitor the camera on Miller Parkway at National/Bridge

- Be prepared to post messages on the following signs:

  - Sign 98:  Miller Parkway NB near National Ave

  - Sign 7:  US 41 SB at Cherry St

  - Sign 18:  I-94 WB at 22nd St

  - Sign 19:  I-94 EB at 89th St

- If water is observed on the roadway, but the roadway is still open, use the following messages (TIS Plan #  1212):

Signs 7, 18, 19

WATER ON ROADWAY

MILLER PKWY AT CANAL

REDUCE SPEED

Sign 98

WATER ON ROADWAY

AT CANAL ST

- If the roadway is closed, use the following messages (TIS Plan # 1213):

Signs 7, 18, 19

MILLER PARKWAY

CLOSED AT I-94

USE ALT ROUTE

Sign 98

MILLER PKWY CLOSED

AT NATIONAL

- Once something is closed by the Sheriff’s Department, follow your typical procedures regarding TIA, SINS, Event Manager, HAR, and 511 Floodgates.