REHABILITATION STRUCTURE SURVEY REPORT

Wisconsin Department of Transportation

DT1696        4/2017

Grade Separation       Stream Crossing      Culvert

Railroad      Retaining Wall      Noise Barrier

Sign Structure      Other:

For guidance see: http://wisconsindot.gov/Pages/doing-bus/eng-consultants/cnslt-rsrces/strct/survey.aspx

| Design Project ID 1107-00-30 | Construction Project ID 1107-00-60 | Highway (Project Name) Allenton - Fond Du Lac |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Final Plan Due Date 6/1/2024 (adv 6/1/2020) | Preliminary Plan Due Date 6/1/2019 | Town Village City Lomira |  |  |  |  |  |
| PS&E Date 8/1/2024 (adv 8/1/2020) | Letting Date 2/9/2025 (adv 2/9/2021) | County Dodge |  |  |  |  |  |
| Structure Number B-14-116 |  | Section 35 |  | Town 13N |  | Range 17E |  |
| Station 150+02.05 | Latitude: 433326.25 Longitude: 882547.85 | YES NO Structure Located on National Highway System |  |  |  |  |  |
|  |  | Traffic Forecast Data |  |  |  |  |  |
| For Survey and CADD Files Horizontal Coordinate System: WISCRS Dodge NAD83 (2011) Vertical Datum: NAVD 88 (2012) |  |  |  |  |  |  |  |
|  |  | Design Year | Average Daily Traffic (ADT) |  | Roadway Design Speed |  | Functional Class |
| Feature On Soo Road |  | Feature On Soo Road | 600 (2045) |  | 60 MPH |  | Minor Collector |
| Feature Under IH 41 |  | Feature Under IH 41 | 37,690 (2045) |  | 70 MPH |  | Principal Arterial |
| Region Contact: James Buschkopf, PE (Area Code) Telephone Number(s): 608-884-7133 Email: James.Buschkopf@dot.wi.gov |  | Consultant Contact: Vincent DiFrances, PE (Area Code) Telephone Number(s): 262-573-3864 Email: vinced@jt-engineering.com |  |  |  |  |  |

Work To Be Performed

Field Information Required
	Item Number (see Pages 2–4)

A.	Structural Repair	1–3, 22

B.	Overlay	1–3, 10–22, 26–28, 32, 34

Concrete Overlay	 Asphalt Overlay

Polymer Modified Asphalt Overlay	 Thin Bonded Polymer Overlay

Other:

C.	New Bearings	3, 8, 9, 22

D.	New Railings	15–17, 20–23

E.	Curb and Sidewalk Repair	2, 3, 16, 22, 23

F.	Abutment Repair	2, 3, 12, 16

G.	Pier Repair	2, 3, 12, 16

H.	New Deck	1–6, 9, 10, 13–28, 32–34

I.	Widening	1–28, 30, 32–35

J.	Joint Repair	2, 3, 8, 16, 19, 22

K.	Surface Repair	2, 3, 22

L.	Raising Bridge	3, 6, 9, 16, 20–24

M.	Slope Stabilization	1–3, 30

N.	Scour Repair	1, 2 or 3, 16, 19, 21, 27, 29, 31–35

O.	Painting	16, 22, 24

P.	Other:

| Field Information Required If no structure number exists provide the following: Small County Map on which the location of proposed structure is shown in red and any highway relocation in green. In addition, provide Location Map of scale not less than 1" = 2000' showing the structure location and number. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Most recent inspection report, brief history of bridge construction date, and description of repairs with dates. 2. Outline deficient areas on existing structure plan or drawing. 3. Photographs of details requiring repairs or modifications, such as: bearings, x-frames, joints, etc. Photograph all deficient areas. Clearly label all photographs. 4. Provide proposed typical section for roadway and structure showing dimensions and cross slopes. 5. Survey beam seat or girder elevations at both sides of bridge at all substructure units. 6. Provide cross-section elevations at 10 foot intervals extending across the structure and a minimum of 100 feet beyond each end. Sections should be normal to centerline and show elevations at centerline roadway and gutter line. Take elevations along joints and at floor drains. 7. Show and identify starting stationing on bridge. 8. Record measurement, temperature of the structure, and date taken for each of the following: (a) Joint opening measured normal to joint at centerline of roadway and both curb lines. (b) Clearance between girder ends at piers. (c) Distance from front face of abutment backwall to closest point of girder end measured parallel to girder. (d) Temperature of structure determined by averaging top and under deck (if accessible) readings. 9. Fixed and expansion bearings - condition and orientation. 10. Number and width of proposed pours including construction staging sequence. 11. Location of existing construction joints in the deck. 12. Estimated Quantities: Preparation, Decks, Type 1 Sq. Yd. 80 Preparation, Decks, Type 2 Sq. Yd. 40 Full Depth Deck Repair Sq. Yd. 8 Galvanic Anodes? No Concrete Surface Repair Superstructure Sq. Ft. N/A Galvanic Anodes? N/A Concrete Surface Repair Substructure Sq. Ft. N/A Galvanic Anodes? N/A Curb Repair LF. N/A Galvanic Anodes? N/A 13. Sufficiency number: 94.0 (obtain from HSI Bridge Inventory System) 14. Appraisal and Condition Rating |  |  |  |  |  |  |
|  |  | Deck Condition | Superstructure Condition | Substructure Condition | Load Capacity Appraisal | Structural EVAL Appraisal |
|  | Current | 7 - Good | 7 - Good | 7 - Good | 5 - Legal Load Stress Not Exceeded | 7 - Cond Better Than Min Criteria |
| 15. Load Ratings |  |  |  |  |  |  |
|  | Current Calculated Date: 6/17/2013 |  | Inventory |  | Operational |  |
|  |  |  | HS-23 |  | HS-37 |  |
|  | After Completed by Bridge Designer |  | HS-20 (Preliminary) |  | HS-34 (Preliminary) |  |

16. Utilities on/near Structure. (WisDOT policy is to avoid placing utilities on the structure.) Yes No 17. Is existing bridge railing deficient? Yes No If Yes – Replacement Rail Type: Type; Owner and Contact Information; Size; Opening at Abutment; Weight; Pressure. 18. Drains to be: Raised Closed Downspouted New 19. Traffic maintained on bridge during work? Yes No If Yes – Include sketches 20. Will guard rail be attached? Yes No If Yes – Which corners? All 21. Will work to be performed eliminate all deficiencies? Yes No If No – Explain: See discussion in "Additional Information" below. 22. Hazardous waste (asbestos) to be removed? Yes No If Yes – Explain: 23. Wing location(s) for surface drain anchors: 24. Painting? Yes No If Yes – Explain on Page 4 (all, part, railing, color system, containment, bid items) 25. Desired roadway width: (new deck / widening) Ft. Desired sidewalk clear width: Left: Ft. Right: Ft. 26. Maximum increase in grade line elevation 1/4 (nominal overlay thickness) In. 27. Benchmark description to be shown 28. Desired final cross slopes on bridge 0.02 (existing) Ft./Ft. 29. Underwater Inspection Report including: - Streambed Cross Section With Pier, Footing and Seal Elevations - Pier Elevation Drawings - Pier Layout - Hydrographic Survey 30. Slope stabilization, provide: Type: Quantity: CY. Slope: Ft./Ft. Fill: CY. 31. Preliminary layout of grout bags or proposed scour repair. C.I.P. Articulated Mats (for Scour) CY. Grout Bags (for Scour) CY. Heavy Riprap CY. Extra Heavy Riprap CY. 32. Report submitted with Preliminary Plan requires no CADD file submittal (See ESubmittal instructions). 33. Report submitted for development of Preliminary Plan to structure design engineer requires CADD file (if available) submittal and Report submittal to Soils Engineer if project involves foundation modifications. 34. Coordinate with structure design engineer before going into the field if existing structure has no available plans, if staged construction is planned, or if there are adjoining/adjacent structures that will remain in place. 35. If project involves substructure widening coordinate with structure and/or hydraulic design engineer to determine if information on the separation and/or stream crossing SSR will be required.

Additional Information

Elaborate on other concerns such as: DNR, Local, Utility Conflicts, Aesthetics, Railing Type and Staged Construction. Please be as detailed and specific as possible.

Supplemental Notes to "Field Information Required"

1.) For most recent inspection report, see B-14-0116_oth. The bridge carrying Soo Road over IH 41 was originally constructed in 1995. No repairs have been completed on the structure per information available on HSI. The deck is noted to have extensive abrasive wear.

2.) See B-14-0116_oth for a schematic showing deficiencies. The Region added joint repair to this structure after initial project scoping.

3.) See B-14-0016_pic for photographs of the structure, including deficient areas.

4.) Not applicable.

5.) Not applicable.

6.) Not applicable.

7.) Not applicable.

8.) A worksheet containing pertinent data and field measurements on the existing expansion joint is included in B-14-0116_oth.

9.) Not applicable.

10.) For polymer overlay, the pours are considered to be placement of polymer and concrete pours required to replace the expansion joints. Soo Road is to be completely closed to traffic during construction, which will allow polymer to be placed continuously in a single operation. Within the limits of joint repair, concrete masonry will likely be placed in two separate operations prior to overlay (one at each abutment).

11.) No construction joints were observed in the deck.

12.) Estimated quantities only. JT Engineering is not scoped to sound the deck, and the Region has not expressed that it will be sounded prior to letting. It will therefore be placed on the contractor to verify in the field. Prep 1 is estimated as 10% of deck area, Prep 2 = 5%, and Full Depth = 1%.

13.) No additional notes.

14.) No additional notes.

15.) Preliminary rating indicates an HS-20 inventory rating, a decrease of 3. The ratings are controlled by negative moment over the pier and are sensitive to the number of continuity bars in the composite section. JT Engineering believes this should be modelled with 9-#9 bars. A run with 10-#9 produces a rating of HS-23 which more closely matches existing. Rating to be verified in final design.

16.) There are no observed utilities attached to the structure and none were observed to be marked in close proximity to the bridge on the approaches.

17.) Sloped Face Parapet "B" is established on the bridge and will be re-established in the area of joint repair.. This railing is sufficient.

18.) There are no existing drains in the bridge deck. Existing drop inlets are located on the west approach and are off the bridge.

19.) Soo Road will be closed to traffic during construction and detoured. See B-14-0116_rod for details.

20.) Guard rail is to be replaced at all quadrants of the bridge (wings 1 thru 4). Steel thrie at the approach will be replaced and then transitioned to MGS. Existing anchor assemblies are the correct dimension for new thrie beam terminal connector.

21.) The project scope has been limited by the Region to include only polymer overlay, deck repairs (if needed), and joint repair. This addresses the heavily abraided deck surface and the strip seal joints in Condition State 3, but does not rectify the concrete defects in the abutment (2 LF delam and 7 LF vertical cracking). It also does not rectify 1 LF of cracking in the pier cap nor the spalled diaphragm noted in the inspection report.

22.) The asbestos inspection report is included in B-14-0116_oth. ACM was found in the gray caulk between parapet wall sections. The ACM is also noted to have been transferred to the white coating (sealer) on the caulk. Based on the photographs, disturbance of the caulk is not anticipated within the limits of joint repair and parapet removal.

23.) Not Applicable

24.) Not Applicable

25.) Not Applicable

26.) The increase in grade line elevation will be limited to the existing plus the nominal polymer overlay thickness of 1/4".

27.) Benchmark is shown on the preliminary bridge plans. See B-14-0116_pln.

28.) The desired final cross slope shall match the existing of 0.02 ft/ft per the original construction plans.

Additional Preliminary Design Notes:

a.) Pigmented Surface Sealer bid item is intended for the entire length of both parapets.

b.) Traffic forecasting has not been completed for this project per the SW Region. The volumes on Soo Road were taken from HSI.

c.) WDNR provided an initial project reivew on 7/24/2019. The intial review letter can be found in B-14-0116_oth.