Multi-channel Analysis of Surface Waves (MASW)

MASW surveying is a non-invasive/ non-destructive imaging technique that uses surface seismic waves to calculate the Shear-Wave (S-wave) velocities of subsurface soils, sediments, and bedrock layers. S-Wave velocities are directly related to material strength (shear modulus and stiffness).  The results of a MASW survey are shown as layered model of S-wave velocities as well as 2D cross-sectional images of changes in strength vs. depth.  MASW cross sections provide detailed information about the depth, structure and strength of subsurface materials from the near surface to depths of over 200 meters.

S-wave velocities vary for different subsurface materials based on changes in composition (minerology) and degree of compaction, and cementation.  In general, very hard materials (e.g. bedrock) produce much faster velocities than unconsolidated sediments.  Subsurface velocity variations make it possible for MASW surveys to map the interface (depth and topography) between different geologic layers, and to identify structural changes (such as fractures or loose zones) within those layers.

FIGURE 1:  MASW data collection equipment

Figure 1 illustrates the data collection equipment used for a standard MASW survey.  During a MASW survey surface waves are generated by impacting a seismic source such as a hammer on the ground surface.  The surface waves are then recorded by an array of sensors (geophones) and a digital seismograph.  Oftentimes the geophone array is affixed to a flexible towing cable called a “land streamer” to facilitate rapid deployment and movement of the array along pre-planned survey lines.   A utility vehicle is used to pull the seismic source and land streamer across a survey area and along the survey lines at regular increments (source stations).  Source station interval, which typically ranges from 2 – 20 feet for most geotechnical and engineering applications, is the horizontal distance between MASW measurements along a survey line.

During MASW data processing each seismic record is analyzed using an FFT transform to isolate each component frequency from the field record and to calculate an accurate phase velocity for each frequency. This information is used to compile a dispersion curve for each source station along a survey line.  The dispersion curves show the velocity of the surface waves as a function of frequency. Lower frequencies are larger waveforms and travel deeper in the earth than higher frequencies.  The dispersion curve represents a unique profile in strength for each source point.  Figure 2 shows a seismic field record with the extracted dispersion curve.

FIGURE 2:  MASW field data and extracted dispersion result

After the dispersion curves are extracted an iterative inversion method is used to create one-dimensional S-wave versus depth profiles at each source station along a survey line.  The 1D velocity profiles are individual sampling points, analogous to a soil boring, that are very closely spaced.  At the completion of the MASW analysis the individual velocity vs. depth models along a survey line are combined and contoured to create a continuous two-dimensional S-wave velocity cross-section (Figure 3).

MASW cross sections display the changes in material shear strength beneath a survey line.  Figure 3 shows a typical MASW cross section in which the measured S-wave velocities are color-coded. The structure of the subsurface sediment and rock layers, as well as the identification of anomalous strength zones can be interpreted from MASW cross sections.  In the figure competent bedrock (red), weathered bedrock and fractures (yellow), sediment and soils (green), weathered or weak areas (blue), and potential void space (white) can be interpreted from the MASW data.

FIGURE 3: MASW 2D Cross Section

Unlike traditional seismic surveying, MASW benefits from the ease and speed of its data interpretation.  Seismic reflection and refraction data processing requires experienced interpreters that analyze seismic waveforms that are oftentimes similar in character to the background noise in a dataset.  This is a costly and time-consuming process that involves much human interaction.  In contrast, the MASW method utilizes the highest energy, and most easily interpretable events on a seismic record, the surface waves (“ground roll”).  MASW data processing uses automated software for the dispersion analysis and can provide results more quickly than traditional seismic methods.

MASW surveying has quickly been adopted as an important geotechnical tool.  Because MASW data interpretation and collection are less complex and more efficient than other geophysical methods, it is now cost effective for engineers to get detailed subsurface strength information for a variety of geotechnical and environmental applications.  With the ability to be rapidly performed as point samples (1D), along survey lines (2D), or across large areas (3D), MASW surveying is being utilized as a powerful site assessment tool.

Reference:  Figures provided by Park Seismic LLC and MASW.com